| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for OL Region |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Guangjie Li, Feng Zhou, Senjie Zhang, Alexie Davydov, Huaning Niu, Yang-seok Choi Intel Corporation | Guangjie.li@intel.com |
| Re: | IEEE 802.16m-09/0020, Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | This contribution provides the proposed text for OL Region. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for OL Region

Guangjie Li, Feng Zhou, Senjie Zhang, Alexie Davydov, Huaning Niu, Yang-seok Choi,

Intel Corporation

- Introduction

In AWD [2], the parameter of “OL region” is listed in the table 672-DL MIMO control parameters with TBD. In this contribution, the detailed concept of OL region and performance evaluation is provided. Text proposal is also included.

- Concept of OL region

The OL region is the predefined zone dedicated for OL MIMO transmission (for example the first several subframes may be allocated for OL region). In the OL region, only OL MIMO can be used. There are no special limitations on the supported LRU type in the OL region.

OL region resources allocation configuration is broadcasted by the ABS. The remaining resources may be used for the normal region (or called non-OL region) where any MIMO mode can be supported.

In OL region, the CQI can be estimated using precoded pilots only without using midamble.

Two kinds of OL regions are defined: rank-1 OL region and rank-2 OL region.

Rank-1 OL region can only support rank-1 precoder, while rank-2 OL region can support SU STBC/SM and OL MU MIMO.

In rank-1 region, 1 stream pilot is always transmitted except for the DRU allocated to control channel, which require 2 stream pilot for SFBC operation.

In rank-2 region, 2 stream pilots are always transmitted.  For OL SU MIMO, the STBC and SM can be flexibly adapted. For OL MU MIMO, full rank transmission is always used (no adaptation to SU MIMO rank-1 precoder).

The benefit to define the OL region is to maintain the static interference for OL MIMO, improve accuracy of the CQI estimation and covariance matrix of interference.

The following figure shows an example of OL region for rank-1 and rank-2 configurations.

Figure 1 Example of OL region allocation

- Performance evaluation

- Link level performance is evaluated and shown in following figure.  2x2 OL SU MIMO SM rank-2 in DRU is used for evaluation, and there are 2 interference BSs (power range are power are [4:10]  and [7:13] dB). The power of servering BS is 0 dB.

- In OL region (blue curve), we use neighboring 5 subframes to estimate the covariance matrix of interference, while in non-OL region (red curve), one subframe has to be used to estimate it because neighboring subframe could be any MIMO modes and the pilot stream is different.

- As the result, 0.5 dB gain is obtained in link level by utilizing OL region

Figure 2 LLS comparison of Ol region and non-OL region

SLS result is also provided in figure 3 to show necessity of OL region.  Without OL region, the interference is dynamic because the scheduling of different user and different precoder from CL MIMO and rank adaptation, so the interference is quite dynamic, which greatly impact the overall performance. In the evaluation, CL MIMO is assumed to be used in the interference cell for non-OL region. And for OL region, the interference cell uses the same MIMO mode.

According to the result, 8% performance loss is observed is OL region is not used.

Figure 3 SLS comparison

- Conclusion

OL region is important feature for the operation of OL MIMO in 16m. It should be supported in AWD.

- Reference

[1] 80216m-08/003r8, “System Description Document (SDD)”

[2] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.6.5 Donwlink signaling support of DL-MIMO modes

Table 1 – DL MIMO control parameters

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - Broadcast Information |  |  |  |  |
| - Nt | - Number of transmit antennas at the BS | - 0b00: 2 - 0b01: 4 - 0b10: 8 | - SFH (system information) | - Nt must be known before decoding the DL A-MAP IE |
| - OL Region [TBD] | - OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | - TBD | - Broadcast information | - Two kinds of OL region is defined:Rank-1 OL region and rank-2 OL region. - Rank-1 OL region supports rank-1 precoder, and rank-2 OL region supports OL SU STBC/SM and OL MU MIMO. - Both DRU and CRU are supported in OL region. - Rank-1 and rank-2 OL region is deinfed as first several continuous subframes |
| - SU_CT | - SU base codebook type |  | - Broadcast information | - SU base codebook subset indication |
| - MU_CT (TBD) | - MU base codebook type |  | - Broadcast Information | - MU base codebook subset indication |
| - BC_SI | - Rank-1 base codebook subset indication | - BitMAP - (Same size as rate-1 codebook for each number of transmit antenna) | - Broadcast information | - Rate-1 codebook element restriction/recommendation information - It shall be ignored if CCE = 0b0 |
| - MaxMt (TBD) | - Maximum number of streams | - 0b00: 2 - 0b01: 3 - 0b10: 4 - 0b11: reserved | - Broadcast information | - If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| - Unicast Information |  |  |  |  |
| - MEF | - MIMO encoder format | - 0b00: SFBC - 0b01: Vertical encoding - 0b10: Horizontal encoding - 0b11: n/a | - A-MAP IE (unicast) | - MIMO encoder format. - [MEF bitfield may not be explicitly indicated in DL A-MAP IE] |
| - Mt | - Number of streams in transmission | - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 - (Mt <= Nt ) | - A-MAP IE (unicast) | - Number of streams in the transmission. - When MEF=0b00: Mt =2 MEF=0b10, Mt <= 4 - [Bit-field length is variable, depending on the number of Tx at BS] |
| - RU allocation (TBD) | - RU [and stream] indicator for the burst of data | - TBD | - A-MAP IE (unicast) | - Refer to DL control group. |
| - SI (TBD) | - Index of pilot stream allocation | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 | - A-MAP IE (unicast) | - SI shall be indicated if MEF = 0b010 - [Bit-field length is variable, depending on the number of Tx at BS] - RU allocation and SI can be merged together depending on other DG’s decision |
| - Feedback Allocation IE |  |  |  |  |
| - MFM | - MIMO feedback mode | - Refer to Table 5 | - Feedback allocation IE (unicast) | - To decide the feedback content and related MS processing |
| - DLRU (TBD) | - Downlink RU, indicating - which RUs or which type of RU (DRU or miniband-based CRU) to work on for feedback | - TBD (Tree structure, bit map etc) | - Feedback allocation IE (unicast) | - To process CQI (PMI) estimation for the indicated RUs - Refer to other DG |
| - FT | - MIMO feedback type | - 0b00: codebook - 0b01: sounding | - Feedback allocation IE (unicast) |  |
| - CM | - Codebook feedback mode | - 0b00: standard - 0b01: adaptive - 0b10: differential | - Feedback allocation IE (unicast) | - Enabled when FT = 0b00 |
| - CCE | - Codebook Coordination Enable | - 0b0: Disable - 0b1: Enable |  | - CCE = 0b0: MS finds PMI within whole broadcasted codebook type entry - CCE = 0b1: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |

-------------------------------  Text End  ---------------------------------------------------