| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Allocation A-MAP Information Elements and FBCH contents for UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Hongmei Sun, Guangjie Li, Rath Vannithamby, Yuan Zhu, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou Intel Corporation Rajni Agarwal, Kevin Power Fujitsu Jianjun Li, Raj and Junnam Yun Posdata Co. Ltd Wookbong Lee, Jinyoung Chun LG Electronics Sangheon Kim, David Mazzarese Samsung Electronics Eldad Zeira, Ron Murias InterDigital Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Yutao Hsieh ITRI Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | {hongmei.sun, guangjie.li, rath.vannithamby, jong-kae.fwu}@intel.com {rajni.agarwal, kevin.power}@uk.fujitsu.com jianjun.Li [jianjun.li@posdata.co.kr] wbong@lge.com eldad.zeira@interdigital.com zhengyanxiu@itri.org.tw kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” (Chapter 15.3.6 DL control, Chapter 15.3. 9 UL-CTRL) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Feedback Allocation A-MAP Information Elements in the 802.16m AWD

Hongmei Sun, Guangjie Li, Rath Vannithamby, Yuan Zhu, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou

Intel Corporation

Rajni Agarwal, Kevin Power

Fujitsu

Jianjun Li, Raj and Junnam Yun

Posdata Co. Ltd

Wookbong Lee, Jinyoung Chun

LG Electronics

Sangheon Kim, David Mazzarese

Samsung Electronics

Eldad Zeira, Ron Murias

InterDigital

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Yutao Hsieh

ITRI

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

- Introduction

This contribution provides amendment text for fast feedback information content to be included in Section 15.9.3 and feedback allocation A-MAP IEs to be included in Section 15.3.6.4 in the 802.16m Amendment Working Document [1]. All MIMO (OL/CL, SU/MU) modes for 2, 4 and 8 transmit antennas are considered in the feedback allocation IE.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

Comment: Feedback Allocation A-MAP IE is not defined in the current AWD text. This contribution proposes the text of feedback allocation A-MAP IE be included in 802.16 amendment text. Suggest adopting the proposed text change #1 in C802.16m-09_0991.doc or its latest version.

Suggested Remedy: Adopt proposed text change #1 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text Change #1

# 15.3.6.4.2.2 DL basic assignment A-MAP IE

Table 1 describes Assignment A-MAP IE Types.

Table 1: A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | ReservedFeedback Allocation A-MAP IE |
| 0b0011 | Reserved |
| 0b0100 | Reserved |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

# 15.3.6.4.2.9 Feedback Allocation A-MAP IE

Table 2 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 2.

Table 2 Feedback Allocation A-MAP IE

Note: 1. Need to differentiate it is in OL region or not. In OL region, MaxMt is not required

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable | - Feedback channel index within the UL fast feedback control resource region - (Dependent on LFB,FPi defined in section 15.3.8.3.3.2) |
| Allocation Period (p) | [3] | A feedback is transmitted on the FBCH every 2pframes |
| Long-term Allocation Period (TBD) |  | TBD |
| Frame offset (TBD) |  | Depends on the decision of A-MAP allocation IE |
| Allocation Duration(d) | [3] |  |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2: - 0b0: 1 - 0b1: 2 - If Nt=4: - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO) - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8: (MU-MIMO) - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| MFM | [3] | - MIMO Feedback Mode as defined in Table 671 |
| Feedback Format | Variable [2-3] | Variable number of bits –depends on MIMO feedback mode |
| If(MFM=0,1,4,7){ |  | (TBD depending on IM group decision on FFR) |
| Number of Reports (n) | [1] | - 0: Same Feedback reported on each FFB opportunity - 1: Different Feedback reported in a time-interleaved fashion over the PFBC opportunities |
| If(Number of Reports>0) { |  | (TBD depending on IM group decision on FFR) |
| Long Allocation Period_multiple_reports (q) | [2] | Long Period Fast Feedback reports are transmitted on the FBCH channel every 2p+q+1frames |
| } |  |  |
| For (i==0 to n){ |  | - For specified Number of Reports (n) - (TBD depending on IM group decision on FFR) |
| FPS | [2] | - Frequency Partition and Reuse Factor as follows: - (Refer to section 20.1 for the frequency partition indexing definition.) - 0b00: Frequency Partition 0 (Reuse 1) - 0b01: Deboosted reuse 3 - 0b10: Boosted Reuse 3 - 0b11: Reserved. |
| } |  |  |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | Variable [0, 2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: adaptive with CCE disabled 0b10: differential with CCE disabled 0b11: standard with CCE enabled If MFM = 2,5 CM is empty (0 bit) |
| CS | [1] | Codebook subset |
| } |  |  |
| If(MFM==2,3,5,6) (TBD) { |  | Localized feedback mode |
| If (feedback_Format == 0) { |  | Specified CQI (used to report specified DLRU/subband CQI) |
| DLRU/subband index | Variable [3-4] | - Variable number of bits – depends on system bandwidth. - 0b000 - 0b110 (3bits): to index 6 subband when system bandwidth is 5MHz - 0b0000-0b1100(4bits): to index 12 subband when system bandwidth is 10MHz |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

MFM: MIMO feedback mode, defined in Table 671.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in Error: Reference source not found

Long-term Allocation Period: Long term period report punctures the normal report.

Number of Reports (n): Indicates whether the FFB channel allocated to the AMS carries 1 or 2 PFBCH Reports.

Allocation Period (p):

Long Allocation Period multiple reports (q) : Indicates the allocation period for Long-Period report and these puncture the Short-Period reports when MFM=0, 1, 4, 7.

FPS: Frequency Partition Selection & Reuse Factor Information. (Refer to section 20.1 for the frequency partition indexing definition)

MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CCE: Codebook Coordination Enable. When CCE is enabled, MS finds PMI within whole broadcasted codebook type entry; when this fields is set to 0b1, it means MS finds rate-1 PMI within broadcasted codebook entries indicated by BC_SI and codebook subset indication (CS in DL A-MAP IE).

DLRU: This field specifies the index of a certain RU which needs AMS to estimate and report CQI/PMI feedback. When a certain RU is specified, AMS only report one CQI (may plus RI)/PMI for it.

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

# 15.3.9.3 Uplink control information content

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified into the following categories:

1) Channel quality feedback

2) MIMO feedback

3) HARQ feedback (ACK/NACK)

4) Uplink synchronization signals

5) Bandwidth requests

6) E-MBS feedback

7) Frequency partition selection (for DRU only) (TBD)

# 15.3.9.3.1 Fast feedback control channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback, and MIMO feedback.

Table 3 PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI |  | 1) Wideband CQI 2) Subband CQI for Best -1 subband (TBD) 3) Average CQI over resource units indicated by BS (TBD) |
| STC Rate Indicator |  | Number of streams |
| BSI (Band Selection Info for best-1 subband) (TBD) |  |  |
| PMI (TBD) |  | - wideband PMI - subband PMI for best-1 |
| Event-driven Indicator (EDI) for CQICH de-allocation (TBD) | N/A | MS to indicate channel variation event |
| Event-driven for comfirmation of CQICH de-allocation. (TBD) |  |  |
| Event-driven Indicator (EDI) for Preferred MIMO operation (TBD) |  | Sequences: Option 1: 1) OL distributed 2) OL localized 3) CL distributed 4) CL localized Option 2: - OL - CL - SU - MU MaxMt =2 - MU MaxMt = 3 - MU MaxMt = 4 Option 3: - two steps method |
| Event-driven Indicator (EDI) for Multi-cell MIMO Initiation (TBD) |  | Sequence based |
| Event-driven Indicator (EDI) for Bandwidth Request Indicator | N/A | This is used to request UL bandwidth. 2 sequences (two services) |
| Event-driven Indicator (EDI) for Frequency partition selection (FPS) (TBD) | N/A | Inform BS the frequency partition of the reported CQI (DL diversity mode only) |
| Event-driven Indicator (EDI) for Buffer management (TBD) | N/A | Indicates occupancy status of HARQ soft buffer |

# 15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

Table 4 SFBCH Feedback Content

| SFBCH Feedback Content | Related MIMO feedback mode | Description/Note |
| --- | --- | --- |
| Subband CQI |  | Reporting of average and differential CQI of selected sub-bands. Reporting of transformed coefficients is TBD |
| Band selection information |  | Selected band position. Index to indicate sub-band selection If transformed coefficients method is used, band selection is not necessary. |
| Subband PMI |  | Precoding Matrix Indicator of one sub-band for CL MIMO |
| Wideband PMI (TBD) |  | Wideband PMI with subband CQI |
| Stream Indicator |  | It is needed for OL MU MIMO only and used to indicate which spatial stream to estimate CQI |
| STC Rate Indicator |  | Number of streams |
| Event-Driven (TBD) |  |  |

# 15.3.9.3.1.3 Channel quality indicator(CQI) definition

Jointly encoding of CQI and STC rate is TBD, which specifies the spectrum efficiency.

# 15.3.9.3.1.3.1 Wideband CQI

Wideband CQI is one average CQI over whole band corresponding to the MIMO mode. In case of FFR , the wideband CQI TBD.

# 15.3.9.3.1.3.2 Subband CQI

Subband CQI is represented by one base CQI (ave_CQI) over the selected M subbands (or whole bandwidth TBD) plus a differential CQI, which is used for best-M feedback.

For best-M feedback, the subband selection and Ave_CQI and Diff_CQI may be reported at different intervals, and share the same FBCH with TDM manner. Subband selection and Ave_CQI will puncture the Diff_CQI at every [4th TBD] report if TDM is enabled.

[All subband CQIs across the whole bandwidth may be organized in hierarchical tree (TBD)]

[Reporting of transformed coefficients is TBD, where MS linearly transforms all the subband CQIs (whole band) into a series of progressively insignificant coefficients, and quantize the coefficients to few bits for feedback. The transformed values are sent over one or several reports]

# 15.3.9.3.1.3.3 Specified CQI (TBD)

Specified CQI is the CQI specified for a certain RU indicated by DLRU parameters in feedback allocation A-MAP IE.

# 15.3.9.3.1.4 Feedback format

# 15.3.9.3.1.4.1 Feedback format for MFM 0,1,4,7

# 15.3.9.3.1.4.2 Feedback format for MFM 2

The detailed format is listed in Table 6. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 6 Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |

# 15.3.9.3.1.4.3 Feedback format for MFM 3 and 6

The detailed format is listed in Table 7. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 7 Feedback formats for MIMO feedback mode 3 and 6

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |

# 15.3.9.3.1.4.4 Feedback format for MFM 5

The detailed format is listed inTable 8. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] reporting period.

Table 8 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |
| 1 (M =1) |  |  |  |  |  |  |
| 2 (M =3) |  |  |  |  |  |  |
| 3 | TBD |  |  |  |  |  |
| … |  |  |  |  |  |  |