| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC PDU Format |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Haihong Zheng, Shashikant Maheshwari, Yousuf Saifullah, Adrian Boariu NSN Zexian Li Nokia | E-mail: haihong.zheng@nsn.com Zexian.li@nokia.com |
| Re: |  |  |
| Abstract | Amendment Text for MPDU Format |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC PDU Format

# Introduction

This contribution proposes the detailed MAC PDU format used over AAI.

# Text Proposal

================= Start of Proposed Text ========================

15.2.x MPDU Format

MAC PDUs shall be of the form illustrated in Figure x.1. Each MAC PDU contains a MAC header. The MAC PDU may contain one or more extended headers. The MAC PDU may contain payload.

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS may be multiplexed into a single MAC PDU.

Figure x MAC PDU Format

### 15.2.x.1 MAC header formats

The generic MAC header begins MAC PDUs containing either MAC management messages or user data. The compact header begins MAC PDUs transmitted in the allocation using persistent allocation or group allocation.

#### 15.2.x.1.1 Generic MAC Header (GMH)

The generic MAC header format is illustrated in Figure x.2. The fields of GMH are defined in Table x.1.

Figure x.2 Generic MAC header format

Table x.1 Generic MAC Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Flow ID | 4 | Flow Identifier: the flow identifier of the connection this MPDU is addressed |
| EH | 1 | Extended Header Presence Indicator 0 = no extended headers is present following the GMH, unless specified otherwise 1 = one or more extended headers is present following the GMH, unless specified otherwise |
| Length | 11 | Length of the payload |

#### 15.2.x.1.2 Compact Header (CH)

The compact header format is illustrated in Figure x.3. The fields of CH are defined in Table x.2.

Figure x.3 Compact Header format

Table x.2 Compact Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | Extended Header Presence Indicator 0 = no extended headers is present following the CH 1 = one or more extended headers is present following the CH |
| Length | 7 | Length of the payload |

### 15.2.x.2 Extended header (EH)

The inclusion of extended header(s) is indicated by EH bit in MAC Header. The general EH format is shown in Figure x.4 and is used unless specified otherwise. The fields of CH are defined in Table x.3.

Figure x.4 Extended Header Format

Table x.3 Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header indication 0 = one or more extended header follows the current extended header unless specified otherwise 1 = this extended header is the last extended header unless specified otherwise |
| Type | 5 | Type of extended header |
| Body Content | Variable | Type-dependent contents |
| TBD | 2 |  |

### 15.2.x.2 Fragmentation and Packing Extended Header Type 1 (FPEH1)

The format of fragmentation and packing extended header Type 1 is shown in Figure x.5. It is used when MAC PDU contains payload for single transport connection. FPEH1 exists after the last extended header (i.e. extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’, or after the GMH if ‘EH’ in GMH set to ‘0’. The fields of FPEH1 are defined in Table x.4. (Add the condition specifying the existence of this).

Figure x.5 Fragmentation and packing extended header type 1 format

Table x.4 Fragmentation and Packing Extended Header Type 1 Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| SN | 10 | Sequence Number for the payload |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.5) |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | Length of the correspondent SDU/SDU fragment. If N SDU/SDU fragments are included in the MPDU, N-1 “Length” + “End” fields are present in the FPEH1. |
| Rsvd | Variable | Reserved bits for byte alignment (clarification) |

Table x.5 Encoding of FC field

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

### 15.2.x.2 Fragmentation and Packing Extended Header Type 2 (FPEH2)

The format of fragmentation and packing extended header type 2 is shown in figure x.6. FPEH2 header is used when MAC PDU contains single management message or fragment of management message. The fields of FPEH2 are defined in Table x.6.

Figure x.6 Fragmentation and packing extended header type 2 format

Table x.6 Fragmentation and Packing Extended Header Type 2 Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header Indicator (always set to 1) |
| Type | 5 | Extended Header Type |
| SN | 8 | Sequence Number for the payload |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.5) |

### 15.2.x.3 Multiplexing Extended Header (MEH)

The format of multiplexing extended header is shown in figure x.7. It is used when SDUs/SDUs fragments from different connections are included in the same MPDU. Each MEH contains multiple Multiplexing Extended Header Blocks (MEHBs). The SDUs or SDU fragments belonging to the same connection are packed together and the information related to these SDUs or SDU fragments is included in one MEHB. The fields of MEH are defined in table x.7. The M bit in MEHB indicates if there is more MEHB followed. If the SDUs or SDU fragment(s) included belong to the same connection, only one MEHB is present.

Figure x.7 Multiplexing Extended Header Format

Table x.7 Multiplexing Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header Indicator (always set to 1) |
| Type | 5 | Extended Header Type |
| Rsvd | 2 | Reserved |
| MEHB | variable | Multiplexing extended header block (format shown in figure x.8 and x.9) |

The format of MEHB is shown in figure x.8, except the first MEHB. The format of the first MEHB is shown in figure x.9. The first MEHB doesn’t contain the Flow ID and the length for the first SDU/SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU/SDU fragment associated with the first MEHB. The fields of MEHB are defined in table x.8.

Figure x.8 Multiplexing Extended Header Block format

Figure x.9 First Multiplexing Extended Header Block format

Table x.8 Multiplexing Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| M | 1 | Indication of more MEHB 0 = no more MEHB follows the current MEHB 1 = one or more MEHB follows the current MEHB |
| Flow ID | 4 | Flow ID of the SDU/SDUs fragment identified in the MEHB |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.5) |
| SN | 10 | Sequence number for the payload identified in the MEHB |
| Length | 11 | Length of the correspondent SDU/SDU fragment |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Rsvd | Variable | Reserved bits for byte alignment |

### 15.2.x.4 Persistent Allocation Extended Header (PAEH)

The format of persistent allocation extended header is shown in figure x.10. It is used together with compact header to allow transmission of MPDUs for the connection (including both management and transport connections) that are not assigned to the persistent allocation or certain control information for the persistent allocation.

Figure x.6 Persistent allocation extended header format

Table x.9 Persistent Allocation Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | Last Extended Header Indicator |
| Type | 5 | Extended Header Type |
| PAEH Type | 2 | Type of PAEH (encoding shown in Table x.10) |
| Content | variable | PAEH Type specific content |

Table x.10 Encoding of PAEH Type field

| PAEH Type | Meaning | Example |
| --- | --- | --- |
| 0b00 | Data of other connection | The persistent allocation is used to carry data of another transport connection or management connection. The Content field in PAEH is shown in figure x.7. |
| 0b01 | De-allocation of the persistent allocation | Indicate the persistent allocation is de-allocated. |
| 0b10 – 0b11 | Reserved |  |

The format of content field in PAEH when PAEH Type is set to 0b00 is shown in figure x.7, and its fields are defined in Table x.11.

Figure x.7 Format of Content field in PAEH with PAEH Type 00

Table x.10 Content Fields in PAEH with PAEH Type 00

| Name | Length (bit) | Description |
| --- | --- | --- |
| Flow ID | 4 | Flow ID for the data carried in the payload |
| Rsvd | 4 | Reserved |

============================== End of Proposed Text ===============