| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Measurement Pilot Patterns for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Ting Liang Zirong Li Dengkui Zhu Kaiying Lv Yanfeng Guan Ying Liu ZTE Corporation | liang.ting12@zte.com.cn lizirong@zte.com.cn zhu.dengkui@zte.com.cn lv.kaiying@zte.com.cn guan.yanfeng@ zte.com.cn liu.ying@ zte.com.cn |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. 802.16m AWD（DL PHY 15.3.5） |  |
| Abstract | The contribution proposes the text of Measurement Pilot Patterns to be included in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO measurement pilot proposal

Ting Liang, Zirong Li, Dengkui Zhu,Kaiying Lv

ZTE Corporation

# 1.Introduction

In current AWD description, all pilots are precoded regardless of number of transmit antennas and allocation type. In order to accurately estimate PMI and CQI at the MS to facilitate efficient closed-loop downlink MIMO operation, the measurement pilot with no precoding is necessary for acquiring full MIMO channel at the MS for PMI selection.

This contribution proposes the MIMO measurement pilot structure for channel estimation of different number of TX antennas. And the performance of different measurement pilot patterns is evaluated.

# 2.The structure of MIMO measurement pilot

A physical structure of downlink MIMO measurement pilot and design requirements have been proposed in [2]. This contribution proposes alternative physical structures of the MIMO measurement pilot for 2Tx/4Tx. And also the contribution proposes a physical structure of the MIMO measurement pilot for 8Tx.

(a) Pattern A                                   (b) Pattern B

Figure 1 Proposed measurement pilot patterns in a RU for 2Tx

Figure1 shows two proposed measurement pilot patterns for 2Tx antennas. The gray squares stand for possible precoded pilot for data demodulation.

For pattern A, each antenna has 6 measurement pilots in a RU. Measurement pilot in different sectors can be placed in the way of TDM or CDM.

For pattern B as figure 2 shows, each antenna has 2 measurement pilots in a RU. Measurement pilot in neighbor sectors can be placed in the way of FDM in the same symbol thus the interference between them is avoid perfectly when power boosting is used.

(a) Sector 0                  (b) Sector 1                  (c) Sector 2

Figure 2  Proposed measurement pilot pattern B in a RU of different sectors for 2Tx

Figure 3 shows a proposed measurement pilot pattern for 4Tx antennas. Each antenna has 2 measurement pilots in a RU. Measurement pilot in different sectors can be placed in the way of TDM or CDM.

Figure 3 Proposed measurement pilot pattern in a RU for 4Tx

Figure 4 shows two proposed measurement pilots for 8Tx antennas. For pattern A, each antenna has 3 measurement pilots in a RU. For pattern B, each antenna has 2 measurement pilots in a RU. Measurement pilots in different sectors can be placed in the way of TDM or CDM.

(a) Pattern A                                (b) Pattern B

Figure 4  Proposed two measurement pilot patterns in a RU for 8Tx

# 3.The Performance of Proposal measurement pilot

Figure 5  2Tx measurement pilot performance in QPSK1/2

Figure 6  2Tx measurement pilot  performance in 16QAM1/2

Figure 7  2Tx measurement pilot performance in 64QAM1/2

Figure 8  4Tx measurement pilot performance in QPSK1/2

Figure 9  4Tx measurement pilot performance in 16QAM1/2

Figure 10  4Tx measurement pilot performance in 64QAM1/2

The performance of different measurement pilot structure for different number of Tx antennas was evaluated in noise limited scenario. MS estimates the channels of different antennas through MIMO measurement pilot and then calculates the index of PMI.

The simulation results show that the 2Tx pattern B measurement pilot achieves higher throughputs than others in the most of the scenarios while it has a neglectable performance degradation only when the SNR is very low above which most of the user’s target SNR.

Besides,different PMI report periods（T= 1 frames and T= 3 frames）are also evaluated and it shows that the report period of 1 frame is appropriate considering the system throughput.

Above all, the measurement pilot pattern of which each antenna has 2 measurement pilot achieves a balance between the pilot overhead and user performance. So we recommend to take it into the SDD text.

# 4．Appendix

The simulation parameters is bellow.

| Parameter | Values |
| --- | --- |
| Channel Bandwidth | 10MHz |
| Over-sampling Factor | 28/25 |
| FFT Size | 1024 |
| Cyclic Prefix (CP) ratio | 1/8 |
| Resource Unit | CRU (4x18x6) |
| Channel condition | mPED_B 3km/h |
| The number of antennas | 2Tx, 2Rx |
| Channel estimation for measurement | 2D-MMSE |
| Channel estimation for demodulation | 2D-MMSE |
| Receiver | MMSE receiver |
| Report period | 5ms/15ms |
| CL-SU-MIMO schemes | SM |
| Codebook | 16e 3bit/6bit |

# 5.References

[1] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[2] “Performance evaluation of MIMO measurement pilot”, IEEE C802.16m-09/408

- 6.Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ---------------------------------------------------

Insert the following text at section 15.3.5.4.2, page 47, line 54:

The measurement pilot patterns for 2Tx is shown in Figure xx1, with the sub-carrier index increasing from top to bottom. The interlaced pilot pattern are used by different ABSs . The interlaced pilot patterns are shown in

Figure xx1 (a) and (b) and (c) respectively. Each ABS chooses one of the pilot patterns among the three sets pilot  pilot(a), pilot (b) and pilot (C) .Pilot K will be used by a particular ABS and determined by the Cell_ID according to the following equation:

K = mod(Cell_ID,3)

The measurement pilot patterns for 4Tx is shown in Figure xx2, with the sub-carrier index increasing from top to bottom.

The measurement pilot patterns for 8Tx is shown in Figure xx3, with the sub-carrier index increasing from top to bottom.

(a)                     (b)                    (c)

Figure xx1  Proposed measurement pilot pattern in a RU for 2Tx

Figure xx2  Proposed measurement pilot pattern in a RU for 4Tx

Figure xx3 Proposed measurement pilot pattern in a RU for 8Tx

-------------------------------  Text End  ------------------------------------------------------------------------------------