| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Allocation A-MAP Information Elements and FBCH contents for UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Hongmei Sun, Guangjie Li, Rath Vannithamby, Yuan Zhu, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou Intel Corporation | {hongmei.sun, guangjie.li, rath.vannithamby, jong-kae.fwu}@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” (Chapter 15.3.6 DL control, Chapter 15.3. 9 UL-CTRL) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Feedback Allocation A-MAP Information Elements in the 802.16m AWD

Hongmei Sun, Guangjie Li, Rath Vannithamby, Yuan Zhu, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou

Intel Corporation

- Introduction

This contribution provides amendment text for fast feedback information content to be included in Section 15.9.3 and feedback allocation A-MAP IEs to be included in Section 15.3.6.4 in the 802.16m Amendment Working Document [1]. All MIMO (OL/CL, SU/MU) modes for 2, 4 and 8 transmit antennas are considered in the feedback allocation IE.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

Comment: Feedback Allocation A-MAP IE is not defined in the current AWD text. This contribution proposes the text of feedback allocation A-MAP IE be included in 802.16 amendment text. Suggest adopting the proposed text change #1 in C802.16m-09_0991.doc or its latest version.

Suggested Remedy: Adopt proposed text change #1 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text Change #1

# 15.3.6.4.2.2 DL basic assignment A-MAP IE

Table 1 describes Assignment A-MAP IE Types.

Table 1: A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | ReservedFeedback Allocation A-MAP IE |
| 0b0011 | Reserved |
| 0b0100 | Reserved |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

# 15.3.6.4.2.9 Feedback Allocation A-MAP IE

Table 2 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 2.

Table 2 Feedback Allocation A-MAP IE

Note: 1. Need to differentiate it is in OL region or not. In OL region, MaxMt is not required

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable | Physical channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in section 15.3.8.3.3.2) |
| Allocation Duration(d) | [3] | The allocation is valid for2dperiods starting from the frame defined by allocation relevance. If d=0b000, the FBCH is deallocated. |
| Allocation Period (p) | [3] | A feedback is transmitted on the FBCH every 2pframes |
| MaxMt | Variable [1-3] | Variable number of bits - depends on number of transmit antenna Nt - (MaxMt <= min(Nt,Nr)) - If Nt=2: - 0b0: 1 - 0b1: 2 - If Nt=4: - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8: - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 |
| MFM | [3] | - MIMO Feedback Mode as defined in Table 7 |
| if(MFM==2,3,5,6){ |  | Localized feedback mode |
| Feedback Format | Variable [2-3] | Variable number of bits –depends on MIMO feedback mode |
| If (MFM == 3,6) { |  | CL SU and MU MIMO |
| CM | Variable [0, 2] | Codebook mode If MFM = 3,6 (2bits) 0b00: Base codebook; 0b01: Transformed code book, 0b10: Differential code book 0b11: Reserved If MFM = 2,5 CM is empty (0 bit) |
| CCE(TBD) | [1] | - Codebook Coordination Enable - 0b0: Disable - 0b1: Enable |
| } |  |  |
| If (feedback_Format == specified CQI) { |  | Specified CQI (DLRU is used) |
| DLRU | Variable [3-4] | Variable number of bits – depends on system bandwidth. 0b000 - 0b110 (3bits): to represent 6 LRUs when system bandwidth is 5MHz 0b0000-0b1100(4bits): to represent 12 LRUs when system bandwidth is 10MHz |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

FCT: Feedback Channel Type

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

MFM: MIMO feedback mode, defined in XXX.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in table xxx in section xxx

MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CCE: Codebook Coordination Enable. When this fields is set to 0b0, MS finds PMI within whole broadcasted codebook type entry; when this fields is set to 0b1, it means MS finds rate-1 PMI within broadcasted codebook entries indicated by BC_ST, SU_CT and MU_CT.

DLRU: This field specifies the index of a certain RU which needs AMS to estimate and report CQI/PMI feedback. When a certain RU is specified, AMS only report one CQI (may plus RI)/PMI for it.

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

Comment: Many details about information content in fast feedback channel are not defined in current AWD. This contribution proposes the text of information content of PFBCH and SFBCH to be included in 802.16 amendment text. Suggest adopting the proposed text change #2 in C802.16m-09_0991.doc or its latest revision.

Suggested Remedy: Adopt proposed text change #2 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text change #2

# 15.3.9.3 Uplink control information content

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified into the following categories:

1) Channel quality feedback

2) MIMO feedback

3) HARQ feedback (ACK/NACK)

4) Uplink synchronization signals

5) Bandwidth requests

6) E-MBS feedback

7) Frequency partition selection (for DRU only).

# 15.3.9.3.1 Fast feedback control channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback, and MIMO feedback and other necessary CQI-related information. Information contents to be carried over PFBCH is described in Table 3, to support Frequency Selective Scheduling (FSS), Fractional Frequency Reuse (FFR) and MFM 0,1,4,7(15.3.7.2.6.4). Whether to report subband CQI, subband selection, PMI in PFBCH for MFM 2,3,5,6 (M=1) is TBD. Different contents are distinguished by sequence in PFBCH. The mapping of 6 bits payload to these contents is described in Table 5, while the mapping of 6 bits payload to 64 PFBCH coding sequences (defined in Table 692) is TBD.

PFBCH is semi statically allocated to AMS by feedback allocation A-MAP IE (in 15.3.6.4.15) to send periodic CQI and RI feedback with pre-configured reporting frequency. And only one PFBCH will be allocated for each active AMS. Non-regular information such as EDI (event-driven indicator), BW requests indicator, FPS (Frequency partition selection) and buffer management will be sent on-demand in same PFBCH and supersedes CQI and RI reporting.

Table 3 PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| Wideband CQI | MFM 0, 1, 4, 7 (whether to report wideband CQI for other MFM is TBD) | One average CQI across the whole band |
| Specified CQI | MFM 2, 5 | CQI specified for a certain RU indicated by DLRU parameter (defined in feedback allocation A-MAP IE in 15.3.6.4.11) |
| Rank Indicator | MFM 0, 1, 2, 4, 7 | Number of streams |
| Event-driven Indicator (EDI) | N/A | MS to indicate channel variation event |
| Bandwidth Request Indicator (TBD) | N/A | This is used to request UL bandwidth. |
| Frequency partition selection (FPS) | N/A | Inform BS the frequency partition of the reported CQI (DL diversity mode only) |
| Buffer management (TBD) | N/A | Indicates occupancy status of HARQ soft buffer |

Note: Whether to report subband CQI, subband selection, PMI in PFBCH for MFM 2,3,5,6 (M=1) is TBD.

# 15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

Feedback contents to be carried over SFBCH is described in Table 4, to support FSS and MFM 2, 3, 5, 6 (in 15.3.7.2.6.4). Feedback formats for different MFM are defined in section 15.3.6.4.1.3

SFBCH is dynamically allocated to an AMS by feedback allocation A-MAP IE (in 15.3.6.4.15). Non-periodic feedback reporting is supported in SFBCH and BS will reallocate or de-allocate SFBCH to an AMS according to the EDI being reported from PFBCH.

Table 4 SFBCH Feedback Content

| SFBCH Feedback Content | Related MIMO feedback mode | Description/Note |
| --- | --- | --- |
| Subband CQI | MFM 2, 3, 5, 6 | Reporting of average and differential CQI of selected sub-bands |
| Subband selection | MFM 2, 3, 5, 6 | Selected sub-band position. Index or bitmap to indicate sub-band selection |
| Subband PMI | MFM 3, 6 | Precoding Matrix Indicator of one sub-band for CL MIMO |
| Stream Indicator | MFM 5 | It is needed for OL MU MIMO only and used to indicate which spatial stream to estimate CQI |
| Rank Indicator | MFM 2, 3 | Number of streams |

# 15.3.9.3.1.3 Joint coded CQI and Rank reporting

CQI and rank are jointly encoded when reporting in both PFBCH and SFBCH, which specifies the spectrum efficiency. The detailed table of joint coded CQI and Rank is defined in XXX.

Mapping of CQI and rank combination and non-regular information to the sequence of PFBCH is described in Table 5.

Table 5 Mapping CQI and rank combinations and non-regular information to 6-bit payload in PFBCH (TBD)

| Sequence Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | MCS, Rank | 1. Totally number of combinations of CQI and RI is N, and N is TBD. 3. All combinations are arranged in SE increasing order and first 16 sequences will be reserved for CQI with rank=1 |
| 1 | MCS, Rank |  |
| … | … |  |
| N | MCS, Rank |  |
| N+1 | (reserved) |  |
| … | (reserved) |  |
| 51 | (reserved) |  |
| 52 | Preferred MIMO mode (TBD) | AMS indicate preferred MIMO mode |
| 53 | Preferred MIMO mode (TBD) |  |
| 54 | FBCH release confirmation | Inform BS that MS has received FBCH release signaling from BS successfully |
| 55 | Indicate 80% occupancy of HARQ soft buffer | MS assisted Buffer Management to inform BS the occupancy of MS HARQ soft buffer level |
| 56 | Indicate HARQ soft buffer overflow |  |
| 57 | BW Request Indicator | Used to request UL Bandwidth |
| 58 | Frequency partition 0 (reuse-1) | AMS informs BS the frequency partition of the reported CQI (for DL diversity mode) Refer to section 20.1 for the frequency partition indexing definition. |
| 59 | Frequency partition 1 (reuse-3) |  |
| 60 | Frequency partition 2 (reuse-3) |  |
| 61 | Frequency partition 3 (reuse-3) |  |
| 62 | Request to switch reporting in SFBCH from Event-driven mode to regular mode | - MS send request based on channel variation event - BS will reallocate its SFBCH and AMS resumes periodical CQI/MIMO feedback reporting (Regular mode) |
| 63 | Request to switch reporting in SFBCH from regular mode to event-driven mode | - MS send request based on channel variation event - BS will release SFBCH allocation and AMS will stop reporting in SFBCH (event-driven mode) |

- CQI and rank combination in SFBCH is same as that in PFBCH and is described inTable 6 CQI and RI table in SFBCH.

Table 6 CQI and RI table in SFBCH

| Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | MCS, Rank | 1. Totally number of combinations of CQI and RI is N, and N is TBD. 3. All combinations are arranged in SE increasing order and first 16 sequences will be reserved for CQI with rank=1 |
| 1 | MCS, Rank |  |
| … | … |  |
| N | MCS, Rank |  |

- Depending on MaxMt (defined in feedback allocation A-MAP IE), payload bits of CQI and RI joint reporting can be variable as listed below, to reduce overhead and improve performance.

- 1) MFM 5,6,7 (MU MIMO schemes) always use the rank-1: 4 bits for rank-1 CQI

- 2) MFM 0,1,2,3,4 (SU MIMO): it depends on the MaxMt, MaxMt <= min(Nt, Nr) and the details are as following:

- MaxMt == 1,  4bits CQI+RI reporting is conducted (16 entries)

- MaxMt == 2,  5bits CQI+RI reporting is conducted (N entries, N is TBD)

- MaxMt == 3,  5bits CQI+RI reporting is conducted (N entries, N is TBD)

- MaxMt == 4,  5bits CQI+RI reporting is conducted (N entries, N is TBD)

- MaxMt == 5,  5bits CQI+RI reporting is conducted (N entries, N is TBD)

- MaxMt == 6,  6bits CQI+RI reporting is conducted (N entries, N is TBD)

- MaxMt == 7,  6bits CQI+RI reporting is conducted N entries, N is TBD)

- MaxMt == 8,  6bits CQI+RI reporting is conducted (N entries, N is TBD)

Information bits length is fixed to 6 when reporting in PFBCH, but BS will limit the sequence searching range depending on MaxMt (defined in feedback allocation A-MAP IE in section 15.3.6.4.11) to improve performance.

Comment: How to report subband CQI in fast feedback channel is not defined in current AWD. This contribution provides the text proposal regarding definition of different CQI types and subband CQI reporting to be included in 802.16 amendment text. Suggest adopting the proposed text change #3 in C802.16m-09_0991.doc or its latest revision.

Suggested Remedy: Adopt proposed text change #3 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text change #3

# 15.3.9.3.1.4 Channel quality indicator(CQI) definition

# 15.3.9.3.1.4.1 Widband CQI

Wideband CQI is one average CQI over whole band corresponding to the MIMO mode.

# 15.3.9.3.1.4.2 Subband CQI

Subband CQI is represented by one base CQI (ave_CQI)over the selected M subbands plus a differential CQI, which is used for best-M feedback. Two types of differential CQI are defined:

- Frequency domain differential: 2 bits are used to indicate [-1,0, 1, 2] levels of SE change based on the joint coded CQI and rank reporting specified in Table 6.

The average CQI (Ave_CQI) of the selected best M CQIs and differential CQI (Diff_CQI) over Ave_CQI. The Ave_CQI is average of the index of SE of the M CQIs (floor to the integral number is required), and Diff_CQI is the offset of the SE from individual CQI to the Ave_CQI.

- Time domain differential:

The previous CQI (Pre_CQI) of same subband and differential CQI(Diff)CQI) over Pre_CQI. And 2 bits are used indicate [ -2 -1 0 1] levels of SE based on the joint coded CQI and rank reporting specified in Table 6.

For best-M feedback, the subband selection and Ave_CQI and Diff_CQI may be reported at different interval, and share the same FBCH with TDM manner. Subband selection and Ave_CQI will puncture the Diff_CQI at every [4th] report if TDM is enabled.

All subband CQIs across the whole bandwidth are organized in hierarchical tree as shows in Figure 1, where the CQI value of each node is the average (or minimum) CQI of all nodes one level below it, while CQI of leaf node is the average CQI of N (N=4) contiguous PRUs within one subband. When number of sub-bands is not power of 2, circular padding is applied (TBD).

With hierarchical tree, AMS can choose to report CQI of selected best-M nodes across the whole tree or partial tree, depending on the system bandwidth. For example, when system bandwidth is 20 MHz, AMS can report CQI of best-M nodes in left branch (or right branch, depending on higher layer configuration) while when system bandwidth is 10MHz, AMS can report CQI of best-M nodes over the whole tree. And the selected M nodes may be from different levels in the tree (or partial tree). Generally, when the channel is flat across a certain frequency band, higher layer node of the hierarchical tree can be chosen.

Figure 1 Example of hierarchical tree for CQI (TBD)

# 15.3.9.3.1.4.3 Specified CQI

Specified CQI is the CQI specified for a certain RU indicated by DLRU parameters in feedback allocation A-MAP IE.

# 15.3.9.3.1.5 Precoding Matrix Indicator (PMI) definition

(TBD)

Comment: The details of MIMO feedback modes are not defined in the current AWD text. This contribution proposes the text of MIMO feedback modes to be included in 802.16 amendment text. Suggest adopting the proposed text change #4 in C802.16m-09_0991.doc or its latest revision.

Suggested Remedy: Adopt proposed text change #4 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text change #4

- 15.3.9.3.1.6 MIMO feedback modes

Each MIMO transmission mode can have one or several kind of MIMO feedback modes. When allocating a feedback channel, the MIMO feedback mode shall be indicated to the MS, and the MS will feedback information accordingly.

The description of MIMO feedback modes and corresponding supported MIMO transmission modes is shown in Table 7. The detailed description of feedback and MS processing are in the following subsections.

Table 7 MIMO feedback modes

| Feedback mode | Description | Feedback content | Type of RU | Supported MIMO transmission mode | Parameters |
| --- | --- | --- | --- | --- | --- |
| Mode 0 | OL SU MIMO STBC/SM (Diversity) | 1. Rank 2. Wideband CQI | Diversity (DRU, Mini-band based CRU) | MIMO mode 0, and MIMO mode 1 support the flexible adaptation between the two modes Rank 1: means the SFBC CQI Rank >=2: means the SM CQI. In DRU, Mt=2 for SM. In Miniband: based CRU, Mt >=2 for SM |  |
| Mode 1 | OL SU MIMO SM (Diversity) | 1. Rank 2. Wideband CQI | Diversity (DRU, Mini-band based CRU) | MIMO mode 1 |  |
| Mode 2 | OL SU MIMO SM (localized) | 1. Rank 2. Subband CQI 3. Subband Selection | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 1 |  |
| Mode 3 | CL SU MIMO (localized) | 1. Rank 2. Subband CQI 3. Subband PMI 4.Subband selection 5.Correlation matrix (for adptive codebook mode) | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 2 |  |
| Mode 4 | CL SU MIMO (Diversity) | 1. Rank 2. Wideband CQI 3. Correlation matrix | Diversity (DRU, Mini-band based CRU) | MIMO mode 2 (Mt=1) |  |
| Mode 5 | OL MU MIMO (localized) | 1. Subband CQI 2. Suband Selection 3. Stream indicator | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 3 |  |
| Mode 6 | CL MU MIMO (localized) | 1. Subband CQI 4. Subband PMI 5. Subband Selection 6. Correlation matrix (for adptive codebook mode) | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 4 |  |
| Mode 7 | CL MU MIMO (Diversity) | 1. Wideband CQI 2. Correlation matrix | Diversity (DRU, Mini-band based CRU) | MIMO mode 4 |  |

Notes: Subband CQI may consist of the Average CQI (Ave_CQI) among the best-M CQIs and the differential CQI (Diff_CQI).

Comment: Feedback format for fast feedback channel is not defined in the current AWD text. This contribution proposes the text of feedback format to be included in 802.16 amendment text. Suggest adopting the proposed text change #5 in C802.16m-09_0991.doc or its latest revision

Suggested Remedy: Adopt proposed text change #5 in C802.16m-09_0991.doc or its latest revision (this document).

# Proposed AWD text change #5

# 15.3.9.3.1.6 Feedback Format

# 15.3.9.3.1.6.1 WB_CQI for MIMO feedback mode 0,1,4,7

MIMO feedback modes 0,1,4,7 are for diversity mode, and AMS needs to report one wideband CQI+RI through PFBCH required by MFM. The detailed sequence of P-FBCH for the wideband CQI+RI is defined in Table 5.

For feedback mode 1, 4,7, BS will additionally report WB CQI for OL MIMO STBC(diversity) every 128 frames, and this report will puncture the original CQI report.

# 15.3.9.3.1.6.2 Subband CQI for MIMO feedback 2

The detailed format is listed in Table 8. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every 4n-th reporting period, n=0,1,2,...,2d−22p(d, p are defined in feedback allocation A-MAP IE)

For feedback format 0~4, BS will additionally allocate one PFBCH for this MS to report WB CQI for OL MIMO STBC (diversity) and RI, on-demand non-regular information. Its PFBCH will report every 128 frames and puncture the report of first SFBCH.

Table 8 Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | One SFBCH | 1 | Short term | Subband Selection | 4: 10MHz 3: 5MHz | Log2(N) |
|  |  |  |  | Rank Indicator+CQI | 4-6 | See Table 6 MaxMt decides the maximum rank |
| 1 (M=2) | One SFBCH | 1 | Short term | Subband Selection | 7: 10MHz 4: 5MHz | Log2N2 |
|  |  |  |  | Ave_CQI | 4-6 | See Table 6 MaxMt decides the maximum rank |
|  |  |  |  | Diff_CQI for first subband | 2 | Frequency domain diff |
|  |  |  |  | Diff_CQI for second subband | 2 | Frequency domain diff |
| 2 (M =2) | One SFBCH | 2 | Long term | Subband Selection | 7: 10MHz 4: 5MHz | Log2N2 |
|  |  |  | Short term | Ave_CQI | 4-6 | See Table 6 |
|  |  |  |  | Diff_CQI for first subband | 2 | Frequency domain diff |
|  |  |  |  | Diff_CQI for second subband | 2 | Frequency domain diff |
| 3 (M =3) | One SFBCH | 2 | Long term | Subband Selection | 8: 10MHz 5: 5MHz | Log2N3 |
|  |  |  | Short term | Ave_CQI | 4-6 | See Table 6 |
|  |  |  |  | Diff CQI for all the 3 subbands | 3 * 2 = 6 bits | Frequency domain diff |
| 4 (M=3) | Two SFBCH | TBD | TBD | TBD | TBD |  |
| 5 | One PFBCH | 1 | short term | Rank Indicator+CQI | 4-6 bits | Specified CQI |

# 15.3.9.3.1.6.3 Subband CQI for MIMO feedback 3 and 6

The detailed format is listed in Table 9. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every 4n-th reporting period, n=0,1,2,...,2d−22p(d, p are defined in feedback allocation A-MAP IE).

For feedback format 0~3, BS will additionally allocate one PFBCH for this MS to report WB CQI for OL MIMO STBC(diversity), on-demand non-regular information. Its PFBCH will report every 128 frames and puncture the report of first SFBCH.

Table 9 Feedback formats for MIMO feedback mode 3 and 6

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | One SFBCH | 1 | Short term | Subband Selection | 4: 10MHz 3: 5MHz | Log2(N) |
|  |  |  |  | Rank Indicator+CQI | 4-6 | See Table 6 MU MIMO, only report the rank 1 CQI |
|  |  |  |  | PMI | 3-6 | Nt=2: 3 bits Nt=4 and subset disabled: 6 bits Nt=4 and subset enabled: 4 bits |
| 1 (M =2) | One SFBCH | 2 | Long term | Subband Selection | 7: 10MHz 4: 5MHz | Log2N2 |
|  |  |  |  | Ave_CQI | 4-6 | 1. See Table 6 2. MU MIMO, only report the rank 1 CQI 3. Ave_CQI is Min_CQI |
|  |  |  | Short term | PMI | 2* (3 -6) = 6-12 bits |  |
|  |  |  |  | Diff CQI for first subband | 2 | Allocation k =1: frequency domain diff over the Ave_CQI Allocation k=2,3: time domain diff over previous report. |
|  |  |  |  | Diff CQI for second subband | 2 |  |
| 2 (M =3) | One SFBCH | 2 | Long term | Subband Selection | 8: 10MHz 5: 5MHz | Log2N3 |
|  |  |  |  | Ave_CQI | 4-6 | See Table 6 MU MIMO, only report the rank 1 CQI |
|  |  |  | Short term | PMI | 9-18 |  |
|  |  |  |  | Diff CQI for all the 3 subband | 6 | Allocation k =1: frequency domain diff over the Ave_CQI Allocation k=2,3: time domain diff over previous report. |
| 3 | TBD |  |  |  |  |  |
| 4 (M=N/A) | One SFBCH | 1 | Short term | CQI+RI | 4-6 bits | Speficied CQI |

# 15.3.9.3.1.6.4 Subband CQI for MIMO feedback 5

The detailed format is listed in Table 10. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every 4n-th reporting period, n=0,1,2,...,2d−22p, (d, p are defined in feedback allocation A-MAP IE).

For feedback format 0~4, BS will additionally allocate one PFBCH for this MS to WB CQI for OL MIMO STBC(diversity), on-demand non-regular information. Its PFBCH will report every 128 frames and puncture the report of first SFBCH.

Table 10 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | One S-FBCH | 1 | Short term | Subband selection | 4: 10MHz 3: 5MHz | Log2(N) |
|  |  |  |  | CQI | 4 | See Table 6 Only rank 1 for MU MIMO |
|  |  |  |  | Stream Index | 1-2 |  |
| 1 (M =2) | One S-FBCH | 1 | Short term | Subband selection | 7 (for 10MHz) | Log2N2 |
|  |  |  |  | Ave_CQI | 4 | See Table 6 |
|  |  |  |  | Diff CQI for 2 subbands | 2*2 = 4 |  |
|  |  |  |  | Stream Index | 2*(1-2) |  |
| 2 (M =2) | One S-FBCH | 2 | long term | Subband selection | 7: 10MHz 4: 5MHz | Log2N2 |
|  |  |  | Short term | Ave_CQI | 4 | See Table 6 Ave_CQI = Min_CQI |
|  |  |  |  | Diff CQI for the 2 subbands | 2*2 = 4 | frequency domain differential over the Ave_CQI |
|  |  |  |  | Stream Index | 2*(1-2) |  |
| 3 (M =3) | One S-FBCH | 2 | long term | Subband selection | 8: 10MHz 5: 5MHz |  |
|  |  |  | Short term | Ave_CQI | 4 | See Table 6 |
|  |  |  |  | Diff CQI for all subbands | 3*2 = 6 | frequency domain differential over the Ave_CQI |
|  |  |  |  | Stream Index | 3*(1-2) = 3-6 bis |  |
| 4 | TBD |  |  |  |  |  |
| 5 | One P-FBCH | 1 | Short term | CQI + RI | 4-6 bits | Specified CQI |