| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Mihyun Lee, Hyunjeong Kang, Rakesh Taori, Heewon Kang, Hokyu Choi Samsung Electronics | mihyun.mac.lee@samsung.com |
| Re: | Category: AWD-Comments /Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.5 DL-CTRL” |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment

Mihyun Lee, Hyunjeong Kang and Rakesh Taori, Heewon Kang, Hokyu Choi

Samsung Electronics

- Introduction

The capacity of SFH is limited to 5 MHz within the first subframe. In case of 5 MHz BW, the first subframe carries not only SFH but also A-MAP within 5 MHz. However, the size of S-SFH subpacket IE in current AWD seems to be overflowed. Also, I believe that more information will be inserted into S-SFH subpackets through the decision from other topics.

I suggest that the SFH is categorized into P-SFH and S-SFH according to the transmission period. For S-SFH, the repetition over consecutive two superframe can be allowed in order to release the capacity of S-SFH.

Main changes with respect to current AWD are as follows,

- The essential system information is categorized according to the transmission period

  - P-SFH is transmitted every superframe

  - S-SFH is transmitted multiple superframe and repeated over consecutive two superframes

- For S-SFH, the S-SFH subpacket are reorganized according to two functions

  - Network Re-entry carried in S-SFH subpacket 1 IE.

  - Network Initial entry carried in S-SFH subpacket 2 IE.

  - Remaining essential system information carried in S-SFH subpacket 3 IE

- Remove current S-SFH subpacket 5 due to the limited S-SFH capacity each superframe

And, the size and the name of the parameters are tentative and will be optimized on the decision belonging to the section.

- Text proposal for inclusion in the 802.16m amendment

(Remedy #1)

-------------------------------  Text Start  ---------------------------------------------------

# P-SFH IE

The P-SFH IE contains essential system information and it is mapped to the P-SFH. The format of the P-SFH IE is shown in Table 654.

Table 654 – P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSBs of superframe number | TBD 4 | Part of superframe number |
| System Configuration Description change count | TBD 6 |  |
| S-SFH Size | TBD 4 | The units of LRU |
| S-SFH Transmission Format | 2 | Indicate the transmission format (repetition) used for S-SFH |
| S-SFH Scheduling information bitmap | TBD 3 | 0b000: no S-SFH If 1st bit = 1, S-SFH includes SP1 otherwise no SP1 If 2nd bit = 1, S-SFH includes SP2 otherwise no SP2 If 3rd bit = 1, S-SFH includes SP3 otherwise no SP3 |
| Reserved | TBD 4 | Note the size of P-FSH SFH should be fixed |
| The reserved bits are for future extension |  |  |
| } |  | [FYI, total: 24 bits] |

SCD Count

Incremented by one (modulo TBD) by the BS whenever any of the values (except MSBs of superframe number in S-SFH SP3) of the S-SFH IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decide that the S-SFH IEs have not changed and may be able to disregard the S-SFH IEs.

-------------------------------  Text End  ---------------------------------------------------

(Remedy #2)

-------------------------------  Text Start  ---------------------------------------------------

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH subpacket IEs. The S-SFH subpacket IEs are transmitted in different timing and periodicity. The S-SFH IE format are shown in Table 655. The S-SFH IE includes the S-SFH subpacket IEs in different timing and periodicity. The S-SFH IE can be repeated over consecutive two superframes.

Table 655 – S-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH IE format () { |  |  |
| If (1st bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP1 IE |  |  |
| } |  |  |
| if (2nd bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP2 IE |  |  |
| } |  |  |
| if (3rd bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP3 IE |  |  |
| } |  |  |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #3)

-------------------------------  Text Start  ---------------------------------------------------

S-SFH SP1 IE contains information for network re-entry, see Table 656

S-SFH SP2 contains information for network re-entry, see Table 656

Table 656 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| LSB of BS ID | 24 | Specifies the 24 least bit of BS ID |
| Periodicity of A-MAP | 1 | 0b0: every subframe 0b1: every 2 subframes |
| HARQ Feedback A-MAP | 2 | Indicates the number of HARQ feedback A-MAPs (Up to 2 bits, Need the decision from DL Ctrl: A-MAP section) |
| Power Control A-MAP | 2 | Indicates the number of Power Control A-MAPs (Up to 2 bits, Need the decision from DL Ctrl: A-MAP section) |
| Transmit format of A-MAP | 1 | Indicate the transmission format (repetition) used for A-MAP (Need the decision from DL Ctrl – A-MAP section) |
| DL permutation configuration (CRU, DRU partitioning and signaling related to that) | 22 | DL_CAS_SB0(4), DL_CAS_MB0(6), DL CAS_SBi (4x3) (Up to 22 bits, Need the decision from DL physical structure section) |
| UL permutation configuration (CRU, DRU partitioning and signaling related to that) | 22 | UL_CAS_SB0(4), UL_CAS_MB0(6), UL_CAS_SBi (4x3) (Up to 22 bits, Need the decision from UL physical structure section) |
| RNG location configuration | 4 | Configuration table needs to be defined (Need the decision from UL Ctrl section) |
| RNG PHY format | 2 | 0b00: RNG format 0 0b01: RNG format 1 0b10: RNG format 2 0b11: RNG format 3 (Need the decision from UL Ctrl section) |
| RNG codes information | 12 | Parameters for determining the root sequences and their cyclic shifts in the preamble set for the cell (Up to 12 bits, Need the decision from UL Ctrl section) |
| HO ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| HO Ranging backoff start | 4 | Initial backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| HO Ranging backoff end | 4 | Final backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| reserved | 7 |  |
| } |  |  |

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 | Sector ID BS ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #4)

-------------------------------  Text Start  ---------------------------------------------------

S-SFH SP2 IE contains information for initial network entry, see Table 657

S-SFH SP3 contains information for initial network entry and network discovery, see Table 657

Table 657 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 3 | 0b000 FDD, Different UL PHY mode with DL 0b001 FDD, Same UL PHY mode with DL 0b010 only E-MBS carrier If (BW == 5/10/20 MHz in TDD) { 0b011 DL:UL = 3:5 0b100 DL:UL = 4:4 0b101 DL:UL = 5:3 0b110 DL:UL = 6:2 } Else if(BW == 8.75 MHz in TDD) 0b011 DL:UL = 3:4 0b100 DL:UL = 4:3 0b101 DL:UL = 5:2 } Else if(BW == 7 MHz in TDD){ 0b011 DL:UL = 2:4 0b100 DL:UL = 3:3 0b101 DL:UL = 4:2 } |
| If (Sub-frame configuration == 0b000) { |  | In case of FDD with Different UL PHY with DL |
| UL carrier frequency | [12] | (Need more discussion) |
| UL bandwidth | [3] | (Need more discussion) |
| } |  |  |
| MS Transmit Power Limitation Level | 8 | Unsigned 8-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| MSB bytes of BSID | 24 | Specifies the Operator ID |
| FFR partitioning info for DL region | 11 | DL_SAC(4), DL_FPSC(3), DL_FPC(4) (Up to 11 bits, Need the decision from DL physical structure section) |
| FFR partitioning info for UL region | 11 | UL_SAC(4), UL_FPSC(3), UL_FPC(4) (up to 11 bits, Need the decision from UL physical structure section) |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| Initial ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Number of BR Channel | 4 | The number of BR channel per frame (Need the decision from UL Ctrl section) |
| BS EIRP | 16 | Signed in units of 1 dBm |
| reserved | [2] |  |
| } |  |  |

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet3 | Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs MSB bytes of BSID |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #5)

-------------------------------  Text Start  ---------------------------------------------------

S-SFH SP3 IE contains remaining essential system information, see Table 658. S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 658.

Table 658 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Periodic ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Periodic ranging backoff start | 4 | Initial backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Periodic ranging backoff end | 4 | Final backoff window size for periodic ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Number of Fast Feedback Channel | 4 | Number of fast feedback channel per UL subframe (Need the decision from UL Ctrl section) |
| Number of HARQ Ack Channel | 4 | Number of HARQ ACK channel per UL subframe (Need the decision from UL Ctrl section) |
| [HARQ Ack delay] | [1] | (Need the decision from HARQ protocol section) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| MSB of superframe number | [8] | Remaining bit of SFN except LSB of SFN in P-SFH |
| Default RSSI and CINR averaging parameter | 8 | Bits 0-3: Default averaging parameter αavgfor physical CINR measurements, in multiple of 1/16 Bits 4-7: Default averaging parameter αavgfor RSSI measurements, in multiple of 1/16 |
| Tx Power report | [24] | (need more discussion) Bits 0–3: Tx_Power_Report_Threshold, It is unsigned integer and shall be read in dB scale. When “0b1111” it means infinite. Bits 4–7: It is unsigned integer whose value is d. Its value d shall be interpreted as Tx_Power_Report_Interval =2d. When “0b1111” it means infinite. Bits 8–11: αp_avg in multiples of 1/16 (range [1/16,16/16]) Bits 12–15: Tx_Power_Report_Threshold, It is unsigned integer and shall be read in dB scale. When “0b1111” it means infinite. It shall be used when CQICH is allocated to the SS. Bits 16–19: It is unsigned integer whose value is d. Its value d shall be interpreted as Tx_Power_Report_Interval =2d frames. When “0b1111” it means infinite. It shall be used when CQICH is allocated to the SS. Bits 20–23: αp_avg in multiples of 1/16 (range [1/16,16/16]), It shall be used when CQICH is allocated to the SS. |
| [UL Noise and Interference level] | [TBD] | (Need the decision from Power Control section) |
| reserved | 7 |  |
| } |  | [Total: 64 bits (excluding TBD)] |

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay SP scheduling periodicity (excluding SP1) information |  |

S-SFH SP5 is transmitted in paging listening interval. It contains information for idle mode AMSs, see Table 659.

-------------------------------  Text End  ---------------------------------------------------