| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Mihyun Lee, Hyunjeong Kang, Rakesh Taori, Heewon Kang, Hokyu Choi Samsung Electronics | mihyun.mac.lee@samsung.com |
| Re: | Category: AWD-Comments /Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.5 DL-CTRL” |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Broadcast Control Information Elements (SFH-IE) for IEEE 802.16m Amendment

Mihyun Lee, Hyunjeong Kang and Rakesh Taori, Heewon Kang, Hokyu Choi

Samsung Electronics

- Introduction

For S-SFH IE, the S-SFH subpackets were organized according to two operations;

  - S-SFH SP2 IE carries the information needed for Network Re-entry

  - S-SFH SP3 IE carries the information needed for Initial Network

But, current SP 2 IE and SP3 IE didn’t include some information needed for its operation.

I suggest inserting the needed parameters as the proposed text.

- Text proposal for inclusion in the 802.16m amendment

(Remedy #1)

Superceded by the comment #200

(Remedy #2)

-------------------------------  Text Start  ---------------------------------------------------

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH subpacket IEs. The S-SFH subpacket IEs are transmitted in different timing and periodicity. The S-SFH IE format are shown in Table 655. The S-SFH IE includes the S-SFH subpacket IEs in different timing and periodicity.

Table 655 – S-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH IE format () { |  |  |
| If (1st bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP1 IE |  |  |
| } |  |  |
| if (2nd bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP2 IE |  |  |
| } |  |  |
| if (3rd bit of S-SFH Scheduling information bitmap == 1) { |  |  |
| S-SFH SP3 IE |  |  |
| } |  |  |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #3)

Insert the following parameters into current S-SFH SP2 IE

-------------------------------  Text Start  ---------------------------------------------------

Table 656 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| ……. |  |  |
| RNG location configuration | [TBD] | Configuration table needs to be defined (Need the decision from UL Ctrl section) |
| RNG PHY format | 2 | 0b00: RNG format 0 0b01: RNG format 1 0b10: RNG format 2 0b11: RNG format 3 (Need the decision from UL Ctrl section) |
| RNG codes information | 12 | Parameters for determining the root sequences and their cyclic shifts in the preamble set for the cell (Up to 12 bits, Need the decision from UL Ctrl section) |
| HO ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| HO Ranging backoff start | 4 | Initial backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |
| HO Ranging backoff end | 4 | Final backoff window size for HO ranging contention, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |
| …… |  |  |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #4)

Insert the following parameters into current S-SFH SP3 IE

-------------------------------  Text Start  ---------------------------------------------------

Table 657 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| …… |  |  |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 |
| Number of BR Channel | 4 | The number of BR channel per frame (Need the decision from UL Ctrl section) |
| … |  |  |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------

(Remedy #5)

withdrawn

-------------------------------  Text End  ---------------------------------------------------