| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up the description of Section 15.3.5.3.2 |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: ty33.kim@samsung.com |
| Re: | IEEE 802.16m-09/0020, “Call for Comments on Amendment Working Document”. |  |
| Abstract | The contribution proposes modifications on IEEE 802.16m amendment working document IEEE 80216m-09/0010r1a. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up the description of Section 15.3.5.3.2

Taeyoung Kim, Jeongho Park, Jaeweon Cho, Hokyu Choi, Heewon Kang

Samsung Electronics

# Clean-up description of DL Subcarrier permutation

In the last meeting, the contribution C80216m-09_0581r1 has been accepted. But, the current AWD does not reflect this contribution. Therefore, this contribution is to revise the current AWD based on the proposed texts in two contributions (e.g. C80216m-09_0581r1 and C80216m-09_0582r5) accepted in May meeting.

# Proposed Amendment Text

Adopt the following text changes for [1]:

Original Amendment text is shown in black. New proposed text is in blue. Removed Amendment text is shown in strike out red text.

------------------------------------------- Text Start ---------------------------------------------------------

[Change the text from line 1 to 40 on the page 41, in 15.3.5.3.2, as follows:]

For each l-th OFDMA symbol in the subframe,

- Allocate the nl pilots within each DRU as described in section (TBD). Denote the data subcarriers of DRUFPi[j] in the l-th OFDMA symbol as SCDRUj,lFPi[k]SCDRUj,l,FPi, 0 j < LDRU,FPi, and 0  k < LSC,l.which are numbered from 0 to LSC,l -1

- Renumber the LDRU,FPi ∙ LSC,l data subcarriers of the DRUs in order, from 0 to LDRU,FPi∙LSC,l -1. Group these contiguous and logically renumbered subcarriers into LDRU,FPi∙LSP,l pairs and renumber them from 0 to LDRU,FPi∙LSP,l-1. The renumbered subcarrier pairs in the l-th OFDMA symbol are denoted by RSPFPi,l RSPl,FPi.

RSPFPi,l[u]=SCDRUj,lFPi[2ν], SCDRUj,lFPi[2ν+1],		0  u < LDRU,FPiLSP,l

where j=uLSP,l and ν=umod(LSP,L)ν=umod(LSP,l)

- Apply the subcarrier permutation formula Equation (187) to the RSPl,FPi to form the permuted subcarrier pairs (PSP) from 0 to LDRU, FPi ∙ Lpair,l-1. Map PSP [s∙LSP,l, (s+1)∙LSP,l-1] map RSPFPi,l into the sth distributed LRUs, s=0,1,…, LDRU,FPi -1. The subcarrier permutation formula is given by

, 	0  m < LSP,l

SPLRUs,lFPi[m]=RSPFPi,l[k], 	0  m < LSP,l						(187)

where

k = LDRU,FPi ∙f(m,s)+g(PermSeq(),s,m,l,t)

- SPLRUs,lFPi[m] is the mth subcarrier pair in the lth OFDMA symbol in the sth distributed LRU of the tth subframe.

- m is the subcarrier pair index, 0 to LSP,l-1.

- l is the OFDMA symbol index, 0 to Nsym-1.

- s is the distributed LRU index, 0 to LDRU,FPi-1.

- t is the subframe index with respect to the frame.

- PermSeq() is the permutation sequence of length LDRU,FPi and is determined by SEED={IDcell*1367} mod 210. The permutation sequence is generated by the random sequence generation algorithm specified in Section 15.3.5.3.3.

- g(PermSeq(),s,m,l,t) is a function with value from the set [0, LDRU,FPi-1], which is defined according to Equation (188).

g(PermSeq(),s,m,l,t) = {PermSeq[{f(m,s)+s+l} mod LDRU,FPi] +DL_PermBase} mod LDRU,FPi,	(188)

where DL_PermBase is an integer ranging from 0 to 31(TBD), which is set to preamble IDcell, and f(m,s)=(m+13*s) mod LSP,l.

SCLRUs,l,FPi[m] is the mth subcarrier pair (0  m < LSP,l) in the lth OFDMA symbol (0  l < Nsym) in the sth distributed LRU of the tth subframe; t is the subframe index with respect to the frame, s is the distributed LRU index (0  s < LDRU,FPi). PermSeq() is the permutation sequence is generated by the random sequence generation algorithm specified in Section <<15.3.5.3.4>>. g(PermSeq(),s,m,l,t) is a function with value from the set [0, LDRU,FPi-1], which is defined according to Equation (188).

g(PermSeq(),s,m,l,t) = (PermSeq(f(m,s)+s+l) mod (LDRU,FPi) +DL_PermBase) mod (LDRU,FPi),

where DL_PermBase is an integer ranging from 0 to 31(TBD), which is set to preamble IDcell, and f(m,s)=(m+13*s) mod LSP.

------------------------------------------- Text End -----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”