| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Resource Allocation of the Remaining Frequency Partition in the SFH Subframe for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jinsoo Choi, HanGyu Cho, Sungho Moon, Jin Sam Kwak LG Electronics Mihyun Lee, Taeyoung Kim Samsung Electronics Yi Hsuan Intel Corporation | emptylie@lge.com mihyun.mac.lee@samsung.com yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) "Comments on AWD 15.3.6 DL-CTRL" |  |
| Abstract | The contribution provides the proposed text for resource allocation of the remaining frequency band in the SFH subframe to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Resource Allocation of the Remaining Frequency Partition in the SFH Subframe for IEEE 802.16m Amendment

Jinsoo Choi, HanGyu Cho, Sungho Moon, Jin Sam Kwak

LG Electronics

Mihyun Lee, Taeyoung Kim

Samsung Electronics

Yi Hsuan

Intel Corporation

- Introduction

In the first subframe of every superframe containing SFH (called SFH subframe throughout this contribution), resource allocation is different from other subframes. It is because the total system bandwidth consists of the bandwidth for SFH which is the physical 5MHz minimum bandwidth and the remaining system bandwidth when the system bandwidth is more than 5MHz.

Until the previous meeting, the resource allocation within the frequency partition for SFH in the SFH subframe has been agreed based on several key-design points as following.

- 5MHz physical bandwidth usage for SFH

- A predefined frequency partition (reuse-1)

- No subband partitioning and applying distributed LRU only

- LRU segmentation/mapping in order of P-SFH, S-SFH and data/control

This contribution provides the proposed text on resource allocation for the remaining frequency partition after predefined 5MHz frequency band allocation for SFH, especially in the cases of more than 5MHz system bandwidth (e.g. 10MHz, 20MHz), to be included in the DL control section of the AWD.

- Rationale of the proposal

The design criteria of the resource allocation for the remaining frequency partition is as follows:

- Basically, subchannelization is compliant to the rules defined in 15.3.5.

- Considering up to 20MHz system bandwidth, it is desirable to be able to support both CRU and DRU without any frequency partitioning.

- It is reasonable to assume that subbands are assigned to CRU and minibands are assigned to DRU for minimizing unnecessary complexity.

- For allocating both CRU and DRU within the remaining frequency partition, the information about the number of subband is required. The value can be signaled or predefined or implicitly obtained by being set to the similar value with that of the other data subframes without any signaling.

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

After the line 46 of page 48, insert following proposed text

-------------------------------  Text Start  ---------------------------------------------------

In the case of more than 5MHz system bandwidth, the remaining frequency band in which SFH is not allocated is used for data and control transmission. This frequency band consists of NLRU,remaining LRUs, which is equal to NLRU – NDLRU,SFH. The resource of the remaining frequency band is configured as DRU by default and does not require extra configuration signaling. Configuring the resource as CRU is for further study.

-------------------------------  Text End  ---------------------------------------------------