| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for UL control allocation |  |
| Date Submitted | 2009-05-06 |  |
| Source(s) | Jinyoung Chun, Bin-Chul Ihm LG Electronics. | jychun03@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) and Chapter 15.3.9 (UL-CTRL) “Comments on AWD 15.3.6 DL-CTRL and 15.3.9 UL-CTRL” |  |
| Abstract | This contribution proposes the resource allocation for UL control channels. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for UL control allocation

Jinyoung Chun, and Bin-Chul Ihm

LG Electronics

- Introduction

There is no specific allocation method of UL control channels. We have to determine which FFR region UL control channel is allocated in, and the order of UL control channels and UL data, etc. Therefore we suggest the allocation method and also define broadcast information for that. But it is limited to UL control channel using distributed LRUs such as Bandwidth request channels, Feedback channels.

- Description of the proposal

There is some information for the allocation of UL Control channels in UL distributed LRUs.

- FFR region

If UL control channels are allocated to all FFR regions, there is the overhead for the allocation and indication. Therefore we prefer BS indicates a FFR region for UL control channels by the broadcast information.

  - Frequency partition index (FPi): It indicates which FFR region UL control channel is allocated in. It is indicated by 2bits in S-SFH or ABI [TBD].

- Bandwidth Request Channel (BRCH)

Bandwidth request channel is the contention-based channel. And all users have to know the region.

  - UL_BWREQ_SIZE: BRCHs are allocated from first distributed LRU [TBD] of the FFR region. And the size of BRCHs (UL_BWREQ_SIZE) is indicated by 2bits in S-SFH or ABI [TBD] in units of LRUs.

  - UL_BWREQ_PERIOD (2p): BRCH is allocated in first UL subframe every 2p frame, which p is indicated by 2bits in S-SFH or ABI [TBD].

- Feedback Channel

Feedback channel is the user specific channel. Each user has to know the location of the allocated channel to the user by BS. And all users may know the region for the allocation of data channel or sounding channel.

  - UL_FEEDBACK_SIZE: Feedback channels are allocated after BRCHs. And the size of feedback channels is indicated by TBD bits in S-SFH or ABI [TBD] in units of LRUs.

  - [Option 1] Feedback channel is divided by two parts: fast feedback channel and HARQ feedback channel. The fast feedback channel is allocated in increasing order starting from the first feedback channel. And the HARQ feedback channel is allocated in decreasing order starting from the last feedback channel.

Figure. The allocation of UL control channels and data channels in the distributed LRUs of a FFR region of a UL subframe

  - [Option 2] Feedback channel is divided by two parts: HARQ feedback channel and fast feedback channel. The HARQ feedback channel is allocated in increasing order starting from the first feedback channel. And the fast feedback channel is allocated in decreasing order starting from the last feedback channel.

Figure. The allocation of UL control channels and data channels in the distributed LRUs of a FFR region of a UL subframe

- Reference

[1] 80216m-08/003r8, “IEEE 802.16m System Description Document (SDD)”

[2] 80216m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

[Remedy 1]

[Modify the table in line 37, page 57 as below:]

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 658.

Table 658 S-SFH SP4 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Subpacket 4 | Fast Feedback Region |  |
|  | HARQ Ack Region |  |
|  | UL_FEEDBACK_SIZE | 4bits |

  - UL_FEEDBACK_SIZE: Feedback channels include Fast feedback channels and HARQ feedback channels (See the section 15.3.8.3.3.1). The size of feedback channels is indicated in units of LRUs.

[Remedy 2]

[Modify texts in line 14, page 102 as below:]

15.3.8.3.3 Resource allocation and tile permutation for control channels

The distributed LRUs in each of uplink frequency partition may be further divided into data, bandwidth request and feedback channels. The feedback channels can be used for both HARQ ACK/NAK and fast feedback. The allocation order of data channels and UL control channels are TBD.

15.3.8.3.3.1 Bandwidth request channels

The number of bandwidth request channels in frequency partition FPi, LBWR,FPi is indicated by the (TBD)-bit field UL_BWREQ_SIZE in the S-SFH (TBD) in the unit of LRUs.

LBWR,FPi=ULBWREQSIZE					(232)

LBWR,FPi=Nbwr⋅ULBWREQSIZE					(232)

Where Nbwr is 1 in MZone and 2 in LZone with PUSC.

Bandwidth request channels are not necessarily present in all subframes and the allocation can differ from subframe to next.

In MZone, the bandwidth request channels are of same size as LRUs, i.e. three 6-by-6 tiles. In LZone with PUSC, the bandwidth request channels consist of three 4-by-6 tiles. The bandwidth request channels use LRUs constructed from the tile permutation specified in <<Section 15.3.6.3.3>>Section 15.3.8.3.2.

[Add the section number and text in line 14, page 103 as below:]

# HARQ feedback channels

Each feedback channel constructed according to Section <<15.3.8.3.4.2>> 15.3.8.3.3.2 can be used to transmit six HARQ feedback channels. The number of HARQ feedback channels is denoted by LHFB,FPi.

[Add the figure and text in line 48, page 103 as below:]

[Note: The section is already in C80216m-09/0386r1 but missed. And the figure is clarified.]

# Fast Feedback Channels

A fast feedback channel consists of one feedback channel.

Figure 438. The allocation of UL control channels and data channels in the distributed LRUs of a FFR region of a UL subframe

Figure 438 – Example of UL resource allocation for UL control channels

Figure 438 is an example of UL resource allocation for UL control channels. The displayed order of data channels and control channels is for illustrative purpose only.

-------------------------------  Text End  ---------------------------------------------------