| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for Feedback Polling IE |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Wookbong Lee, Jinyoung Chun, Bin-Chul Ihm LG Electronics. Luciano Sarperi, Mythri Hunukumbure Fujitsu | wbong@lge.com, jychun03@lge.com luciano.sarperi@uk.fujitsu.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | This contribution proposes Feedback polling IE to allocate the feedback header. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for Feedback Polling IE

Wookbong Lee, Jinyoung Chun, and Bin-Chul Ihm

LG Electronics

Luciano Sarperi, Mythri Hunukumbure

Fujitsu

- Introduction

For large amount of information or information which required to be reported semi-statically, MAC header type of feedback is required.

In this contribution, we propose feedback polling IE.

- Feedback Type

  - PMI restriction: Section 20.2 [1]

  - PMI recommendation: Section 20.2 [1]

  - Co-MIMO: Section 20.2 [1]

  - CL-MD: Section 20.2 [1]

  - Covariance matrix: Section 11.8 [1]

  - BR header for RTPS

- Reference

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

[Insert the below texts in line 60, page 67.]

15.3.6.4.2.x. Feedback Polling A-MAP IE

This IE is used by BS to schedule feedback header transmission by the MS.

Table x. Feedback Polling A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | [4] | Feedback_Polling_IE |
| MCS | [2 or 4] | MCS level for transmit feedback header |
| Resource Allocation | [TBD] | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) superframe starting from the superframe defined by allocation relevance. If d == 0b000, the prescheduled feedback header transmission is released. If d == 0b111, the prescheduled feedback header transmission shall be valid until the BS commands to release it. |
| Period (p) | [2] | Transmit feedback header every [2p superframe] |
| DLRU | [3] | - Downlink LRU indicating which RUs or which type of RU to work on for feedback 0b000 : Whole bandwidth 0b001 : Subband CRUs in frequency partition 0 0b010 : Subband CRUs in power boosted frequency partitions 0b011 : Subband CRUs in non power boosted frequency partitions 0b100 : DRU/mini-band CRU in frequency partition 0 0b101 : DRU/mini-band CRU in power boosted frequency partitions 0b110 : DRU/mini-band CRU in non power boosted frequency partitions 0b111 : Whole bandwidth over whole carriers |
| Feedback type | [3] | See table y. |
| N | [3] | If Feedback type == 0b000 to 0b011, then N indicates how many PMI and CQI for neighboring cell should be reported |
| Measured Logical Carrier Index | [3] | - If DLRU ≠ 0b111, - 0b000: Primary carrier - 0b001: Secondary carrier 1 - 0b010: Secondary carrier 2 - 0b011: Secondary carrier 3 - 0b100: Secondary carrier 4 - 0b101: Secondary carrier 5 - 0b110: Secondary carrier 6 - 0b111: Secondary carrier 7 - Else - Set to zero |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

Table y. Feedback type for Feedback Polling A-MAP IE

| Feedback type | Feedback contents | Descriptions |
| --- | --- | --- |
| 0b000 | N ⅹ(PMIs (6 bits) + CQI difference (2 bits) + Temp_BSID) | ICT (interference coordination type) == 0b00 Downlink PMI restriction feedback, strongest interfering PMI and CQI differences for neighboring cell of ID, Temp_BSID. (Refer 15.3.11.1.1.1.) Temp_BSID is diversity set member ID assigned to this BS. |
| 0b001 | N ⅹ(PMIs (6 bits) + CQI difference (2 bits) + Temp_BSID) | ICT (interference coordination type) == 0b01 Downlink PMI recommendation feedback, weakest interfering PMI and CQI differences for neighboring cell of ID, Temp_BSID. (Refer 15.3.11.1.1.1.) Temp_BSID is diversity set member ID assigned to this BS. |
| 0b010 | N ⅹ(PMIs (6 bits) + CQI difference (2 bits) + Temp_BSID) | ICT (interference coordination type) == 0b10 Downlink Co-MIMO feedback, PMI and CQI differences for neighboring cell of ID, Temp_BSID. (Refer 15.3.11.1.1.1.) Temp_BSID is diversity set member ID assigned to this BS. |
| 0b011 | N ⅹ(PMIs (6 bits) + CQI difference (2 bits) + Temp_BSID) | ICT (interference coordination type) == 0b11 Downlink CL-MD feedback, PMI and CQI differences for neighboring cell of ID, Temp_BSID. (Refer 15.3.11.1.1.1.) Temp_BSID is diversity set member ID assigned to this BS. |
| 0b100 | Quantized Correlation Matrix | 6bit for 2Tx, 28bit for 4Tx, 120bit for 8Tx |
| 0b101 | [6] byte | BR header for RTPS |
| 0b110~0b111 | Reserved |  |

-------------------------------  Text End  ---------------------------------------------------