| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m Amendment Text Proposal for Feedback Operation |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee, Bin-Chul Ihm LG Electronics. | jychun03@lge.com, wbong@lge.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) and Chapter 15.3.9 (UL-CTRL) “Comments on AWD 15.3.6 DL-CTRL and 15.3.9 UL-CTRL” |  |
| Abstract | This contribution proposes Feedback contents and Feedback allocation IE for fast feedback channel. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for Feedback Operation

Jinyoung Chun, Wookbong Lee, and Bin-Chul Ihm

LG Electronics

- Introduction

In Amendment Working Document, two kinds of feedback channel are defined. One is primary fast feedback channel (PFBCH), and the other is secondary fast feedback channel (SFBCH). For small amount of information (up to 6bits) or reliable feedback, we can use PFBCH, while for large amount of information (up to 24 bits), we can use SFBCH.

In this contribution, we will define the detail contents and feedback allocation method.

- Reference

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Text proposal for inclusion in the 802.16m amendment

[Add the texts and figures marked in blue and remove the texts and figures marked in red in the IEEE 802.16m-09/0010rla]

---------------------------------------------  Text Start  --------------------------------------------------------

[Remedy 1. Insert the below texts in line 61, page 67.]

15.3.6.4.2.9. Feedback Allocation A-MAP IE

This IE is used by BS to schedule fast feedback channel transmission by the MS.

Table x1. FB-ALLOC-A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FB-ALLOC-A-MAP IE format { |  |  |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE |
| Channel Index | Variable | The index of fast feedback channel (The number of bits depends on LFB,FPi defined in section 15.3.8.3.3.2) |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) frame starting from the frame defined by allocation relevance. If d == 0b000, the prescheduled FBCH transmission is released. If d == 0b111, the prescheduled FBCH transmission shall be valid until the BS commands to release it. |
| Allocation period (p) | [2] | Short-term feedback is transmitted on FBCH every 2p frames |
| Long-term feedback cycle (q) | [2] | Long-term feedback is transmitted every 2q allocated FBCH. If q=0, there is no long-term feedback. The first feedback is sent on the first allocated FBCH. |
| MFM (MIMO feedback mode) | [4] | This field is to decide the feedback contents and related MS processing. See table x2 |
| Second Channel Allocation | [1] | If MFM== 0b0011, 0b0100, 0b0101, 0b0110, 0b1000, 0b1001, 0b1010, 0b1011, 0b1100, follow as below. 0b0 : The second FBCH is allocated in the same subframe of first FBCH. If first allocated FBCH’s index is CI, then the other FBCH’s index is CI+1. 0b1 : The second FBCH is allocated in the next UL subframe with same FBCH index. Else, Set to zero |
| DLRU Report type | [3] | - Downlink LRU Report type indicating which RUs or which type of RU to work on for feedback 0b000 : Whole bandwidth 0b001 : Subband CRUs in frequency partition 0 0b010 : Subband CRU in power boosted reuse N frequency partition 0b011 : Subband CRU in non power boosted reuse N frequency partition 0b100 : DRU/mini-band CRU in frequency partition 0 0b101 : DRU/mini-band CRU in power boosted reuse N frequency partition 0b110 : DRU/mini-band CRU in non power boosted reuse N frequency partition 0b111 : Whole bandwidth over whole carriers (for multi-carrier) |
| CM (Codebook feedback mode) | [2] | - 0b00: standard - 0b01: adaptive - 0b10: differential - 0b11: Reserved |
| CCE (Codebook coordination enable) | [1] | - 0b0: Disable - 0b1:Enable |
| Logical Carrier Index | [3] | - For multi-carrier operation, - if DLRU ≠ 0b111, - 0b000: Primary carrier - 0b001: Secondary carrier 1 - 0b010: Secondary carrier 2 - 0b011: Secondary carrier 3 - 0b100: Secondary carrier 4 - 0b101: Secondary carrier 5 - 0b110: Secondary carrier 6 - 0b111: Secondary carrier 7 - else, - Set to zero |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } |  |  |

Table x2. MIMO feedback mode for feedback allocation A-MAP IE

| MFM | DLRU (Type of RU) | Description | Long Term Period for first FBCH (Feedback channel type) | Long Term Period for second FBCH (Feedback channel type) | Short Term Period for first FBCH (Feedback channel type) | Short Term Period for second FBCH (Feedback channel type) |
| --- | --- | --- | --- | --- | --- | --- |
| 0b0000 | 0b100, 0b101, 0b110 (Diversity - DRU) | OL SU MIMO STBC/SM (diversity/OL Region) | Sequence 0: MIMO mode 0 Sequence 1: MIMO mode 1 with Mt =2 | N/A | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | N/A |
| 0b0001 | 0b100, 0b101, 0b110 (Diversity - miniband) | OL SU MIMO SM (MIMO mode 1) (diversity) | Sequence 0~7: Rank | N/A | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | N/A |
| 0b0011 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | OL SU MIMO SM (MIMO mode 1) (localized) | Sequence 0~7: Rank | Band Selection Information (BSI) M=3 (11bit) | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | 3 ⅹ Delta CQI (2bit) [6bit] |
| 0b0100 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | OL SU MIMO SM (MIMO mode 1) (localized) | Sequence 0~7: Rank | Band Selection Information (BSI) M = 1 (5bit) | Sequence 8~39: The best subband CQI | N/A |
| 0b0101 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | CL SU MIMO (MIMO mode 2) (localized) | Sequence 0~7: Rank | Band Selection Information (BSI) M=3 (11bit) | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | 3 ⅹ (Delta CQI (2bit) + PMI (6bit)) [24bit] |
| 0b0110 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | CL SU MIMO (MIMO mode 2) (localized) | Sequence 0~7: Rank | Band Selection Information (BSI) M = 1 (5bit) | Sequence 8~39: The best subband CQI | PMI for the best subband (6bit) |
| 0b0111 | 0b100, 0b101, 0b110 (Diversity - miniband) | CL SU MIMO (MIMO mode 2) (diversity) | Sequence 0~7: Rank | N/A | The wideband CQI value over the resource unit indicated by DLRU (5bit)+ Wideband PMI (6bit) [11bit] | N/A |
| 0b1000 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | OL MU MIMO (MIMO mode 3) (localized) | Sequence 0~3: stream index | Band Selection Information (BSI) M=3 (11bit) | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | M ⅹ Delta CQI (2bit) [6bit] |
| 0b1001 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband ) | OL MU MIMO (MIMO mode 3) (localized) | Sequence 0~3: stream index | Band Selection Information (BSI) M = 1 (5bit) | Sequence 8~39: The best subband CQI | N/A |
| 0b1010 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband) | CL MU MIMO (MIMO mode 4) (localized) | N/A | Band Selection Information (BSI) M=3 (11bit) | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | 3 ⅹ (Delta CQI (2bit) + PMI (6bit)) [24bit] |
| 0b1011 | 0b000, 0b001, 0b010, 0b011 (Localized - subband/ miniband ) | CL MU MIMO (MIMO mode 4) (localized) | N/A | Band Selection Information (BSI) M = 1 (5bit) | Sequence 8~39: The best subband CQI | PMI for the best subband (6bit) |
| 0b1100 | 0b100, 0b101, 0b110 (Diversity - miniband) | CL MU MIMO (MIMO mode 4) (diversity) | N/A | N/A | Sequence 8~39: The wideband CQI value over the resource unit indicated by DLRU | The wideband PMI (6bit) |

[Remedy 2. Modify the below texts in line 1, page 120.]

# 15.3.9.3.1 Fast feedback control channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback, and MIMO feedback. Table y1 shows the encoding of payload bits for the primary fast feedback channel (PFBCH) if it is used for first feedback channel.

Table y1. The codeword to content mapping for PFBCH

| Sequence | Value | Value Description |
| --- | --- | --- |
| 0~7 | 0b000000~0b000111 | Rank or stream index for OL MU-MIMO |
| 8~39 | 0b001000~0b100111 | CQI |
| 40 | 0b101000 | Preferred MFM: 0b0001 (OL-SU - Diversity) |
| 41 | 0b101001 | Preferred MFM: 0b0011 (OL-SU - Localized) |
| 42 | 0b101010 | Preferred MFM: 0b0101 (CL-SU - Localized) |
| 43 | 0b101011 | Preferred MFM: 0b1000 (OL-MU - Localized with MaxMt == 2) |
| 44 | 0b101100 | Preferred MFM: 0b1000 (OL-MU - Localized with MaxMt == 3) |
| 45 | 0b101101 | Preferred MFM: 0b1000 (OL-MU - Localized with MaxMt == 4) |
| 46 | 0b101110 | Preferred MFM: 0b1010 (CL-MU - Localized with MaxMt == 2) |
| 47 | 0b101111 | Preferred MFM: 0b1010 (CL-MU - Localized with MaxMt == 3) |
| 48 | 0b110000 | Preferred MFM: 0b1010 (CL-MU - Localized with MaxMt == 4) |
| 49~63 | 0b110001~0b11111 | BW-REQ based on Service type and Reserved |

# 15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive. The detail contents mapping is shown in Table x2 (Section 15.3.6.4.2.9).

-------------------------------  Text End  ---------------------------------------------------