| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedure of Interference Mitigation among multiple cells |  |
| Date Submitted | 2009-4-27 |  |
| Source(s) | Honghai Zhang, Linghang Fan, Sampath Rangarajan, Nader Zein, Tetsu Ikeda, Andreas Maeder, Amir Khojastepour, Mohammad Madihian NEC | honghai@nec-labs.com |
| Re: | 802.16m AWD Target topic: Interference mitigation, Related to SDD Section 20 |  |
| Abstract | This contribution provides contributions to the interference mitigation among multiple cells. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedure of Interference Mitigation among multiple cells

# Introduction

Binary or multiple-discrete-level power control can be performed at base stations to control the interference and optimize the network throughput. The procedure in this contribution is introduced to mitigate inference among multiple cells.

# Interference mitigation

Source BSs can continuously monitor its own interference situation, decide to mitigate interference, and then collect interference information and related parameters from the MSs and the neighbor cells. Then the source BS decides which neighbor BSs are the target BSs which need to control their transmission power and sends them the power update request or necessary information to compute appropriate transmission power for interference mitigation. Each target BS receives the requests, analyzes its own interference situation, and decides whether to change its power settings, and sends back the response to the source BSs. After receiving the response or not receiving the response after a pre-specified period, Source BS can continue to send interference mitigation request or cease interference mitigation.

Source BS

Target BS

Target BS

Source BS

decision to mitigate interference

decision to mitigate interference

interference information

request for interference mitigation

response to request

for interference mitigation

Figure 1 Procedure of Interference Mitigation

Two types of management messages are supported for the interference mitigation process.

1. XXX-REQ/RSP: message between an ABS and an AMS for requesting and reporting interference information.

2. YYY-REQ/RSP: message between two ABSs for requesting and response of interference mitigation.

--------------------------------------------- Proposed text -----------------------------------------------------

++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

xx. Interference Mitigation (Related to Sec. 20 in SDD)

xx.1 Dynamic Interference Management and Optimization

Source BSs can continuously monitor its own interference situation, decide to mitigate interference, and then collect interference information and related parameters from the MSs and the neighbor cells. Then the source BS decides which neighbor BSs are the target BSs which need to control their transmission power and sends them the power update request or necessary information to compute appropriate transmission power for interference mitigation. Each target BS receives the requests, analyzes its own interference situation, and decides whether to change its power settings, and sends back the response to the source BSs. After receiving the response or not receiving the response after a pre-specified period, Source BS can continue to send interference mitigation request or cease interference mitigation.

Source BS

Target BS

Target BS

Source BS

decision to mitigate interference

decision to mitigate interference

Collect interference information

Request for interference mitigation

response to request

for interference mitigation

Figure 1 Procedure of Interference Mitigation

Two types of management messages are supported for the interference mitigation process.

1. XXX-REQ/RSP: message between an ABS and an AMS for requesting and reporting interference information.

2. YYY-REQ/RSP: message between two ABSs for requesting and response of interference mitigation.

xx.1.a XXX-REQ message (ABS requests interference information)

The XXX-REQ message shall be transmitted by an ABS to request one or multiple AMSs to report interference information. The message may be transmitted as a broadcast message. The format of XXX-REQ messages is shown in Table xx.

Table xx XXX-REQ message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| XXX-REQ_message_format() { |  |  |
| Management message type = xx [TBD] | 8 |  |
| TLV encoded information | Variable | TLV-specific |
| } |  |  |

The following parameter shall be included in the XXX-REQ message:

Interference metric type (See 11.x.2)

If it is transmitted as a broadcast message, the following parameter shall be included:

Set of target AMSs (See 11.x.1):

xx.1.b XXX-RSP message (AMS reports interference information)

The XXX-RSP message shall be transmitted by an AMS to report its interference information to the ABS that requests interference information. It may also be transmitted voluntarily. In that case, it indicates the AMS is experiencing heavy interference and suggests the ABS performing interference mitigation. The format of XXX-RSP messages is shown in Table xx.

Table xx XXX-RSP message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| XXX-RSP_message_format() { |  |  |
| Management message type = xx [TBD] | 8 |  |
| TLV encoded information | Variable | TLV-specific |
| } |  |  |

The following parameters shall be included in the XXX-RSP message:

Interference metric type (See 11.x.2)

Interference values (See 11.x.3)

xx.1.c YYY-REQ message (ABS requests interference mitigation)

The YYY-REQ message shall be transmitted by an ABS to other ABSs that create strong interference to the current ABS. The message format of YYY-REQ is shown in Table xx.

Table xx YYY-REQ message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| YYY-REQ_message_format() { |  |  |
| Management message type = yy [TBD] | 8 |  |
| TLV encoded information | Variable | TLV-specific |
| } |  |  |

The following parameters shall be included in the YYY-REQ message:

[TBD, examples are provided below]

xx.1.d YYY-RSP message (ABS responds to interference mitigation)

The YYY-RSP message shall be transmitted by an ABS to the ABS that requests interference mitigation to the current ABS. The message format of YYY-RSP is shown in Table xx.

Table xx YYY-RSP message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| YYY-RSP_message_format() { |  |  |
| Management message type = yy [TBD] | 8 |  |
| TLV encoded information | Variable | TLV-specific |
| } |  |  |

The following parameters shall be included in the YYY-RSP message:

[TBD, Examples are provided below]

11.x XXX-REQ/RSP management message encoding:

Table xx  XXX-REQ/RSP management message encodings

| Type | Parameters |
| --- | --- |
| ... |  |
| [TBD] | Set of target AMSs |
| [TBD] | Interference metric types |
| [TBD] | Interference values |
| ... |  |

11.x.1 Set of target AMSs

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | Variable | [# of AMSs in the list (each with ## bits); List of AMS IDs] | XXX-REQ |

11.x.2 Interference metric types:

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | 1 | [TBD] [Example: 0: AMS reports the ratio of the interfering power level of a target ABS to the signal power level at the serving ABS, 1-255: reserved] | XXX-REQ, XXX-RSP |

11.x.3 Interference values:

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | Variable | [TBD] [Example: {Group of subchannels}, {#of interfering BS}, {ID of Interfering BS1}; {interference value of BS1 on the group of subchannels}; ... {ID of interfering BSn}; {Interference value of BSn on the group of subchannels}; ] | XXX-RSP |

The group of subchannels may contain one or multiple subchannels. If it is for a single subchannel, the ABS performs interference mitigation on each subchannel individually. If it is for multiple subchannels (e.g., for all subchannels in a block of a partition), the ABS performs interference mitigation on the group of subchannels.

11.y YYY-REQ/RSP management message encoding:

The field in YYY-REQ includes necessary information for the target ABS to compute the benefit of interference mitigation at the source ABS if the target ABS changes its transmission power. For example, this field contains the weighted rate increase/decrease if a target ABS turns off/on on a particular group of subchannels, which may be specified by a single group ID.

Table xx  YYY-REQ/RSP management message encodings

| Type | Parameters |
| --- | --- |
| ... |  |
| [TBD] | Required information for the target ABS to compute the benefit at the source ABS if the target ABS changes its transmission power on a group of subchannels. |
| [TBD] | New power level at the target ABS |
| ... |  |

11.y.1 Required information for the target ABS to compute the benefit at the source ABS if the target ABS changes its transmission power on a group of subchannels.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | Variable | [TBD] [For example, {Group of subchannels}, {the weighted data rate increase/decrease at the source ABS if the target ABS turns off/on the specified group of subchannels}] | YYY-REQ |

11.y.2 New power level at the target ABS.

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| [TBD] | 1 | [TBD] [For example, {Group of subchannels}, {New power level: 0- zero power on the group of subchannels, 1- Full power on the group of subchannels] | YYY-RSP |

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++