| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed IEEE 802.16m Amendment Text on Secondary Carrier Management |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Juhee Kim, Eunkyung Kim, Soojung Jung, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI | E-mail: Juhee@etri.re.kr |
| Re: | Category: AWD-New contribution /Area: Multi-Carrier |  |
| Abstract | This contribution proposes IEEE 802.16m AWD on Multi-Carrier operation |  |
| Purpose | Proposed to be discussed and adopted by TGm for the 802.16 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to activate, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed IEEE 802.16m Amendment Text on Secondary Carrier Management

Juhee Kim, Eunkyung Kim, Soojung Jung, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

This contribution proposes amendment text on the secondary carrier management procedure for the WirelessMAN-OFDMA Advance System with multi-carrier support.

# New messages for the carrier management

While other actions such as handover and power management in MC operation can be performed with the same messages as in Single Carrier, the carrier management is an action specified only in MC. Therefore, we suggest that new messages should be defined for carrier management in Multi-Carrier Operation as follows.

- AAI_CC-REQ (Carrier Change Request) message

: The AAI_CC-REQ message is transmitted by an AMS to request the active carrier change such as primary carrier change and secondary carrier activation/deactivation.

- AAI_CC-CMD (Carrier Change Command) message

: The AAI_CC-CMD message is transmitted by the ABS to command for the AMS to change the configuration of the active carriers (primary carrier change and secondary carrier activation/deactivation)

- AAI_CC-IND (Carrier Change Indication) message

: The AMS shall transmit an AAI_CC-IND message upon reception of AAI_CC-CMD message.

# Secondary Carrier Activation/Deactivation

The ABS may activate/deactivate an inactive assigned secondary carrier of the AMS for load balancing, required QoS due to addition/deletion of a service flow. The secondary carrier activation/deactivation procedure can be triggered by ABS or requested by the AMS.

When an inactive assigned secondary carrier needs to be activated, the ABS may request channel measurement on the inactive carriers to the AMS through an AAI_SCN-RSP message. In this case, upon reception of the AAI-SCN-RSP message, the AMS performs scanning by using an unused transceiver. Then, the AMS reports the scanning results through an AAI_SCN-REP message to the ABS. The ABS determines the target secondary carrier to be activated, based on the measurement information, load balance, and other factors. The ABS may activate a pre-assigned carrier for E-MBS service without collecting measurement information. In order to command the AMS to activate a secondary carrier, the ABS sends an AAI_CC-CMD message which may include the following information,

- Physical Carrier Index of the target secondary carrier

- Logical Carrier Index of the target secondary carrier

- Ranging information (optional)

- Action time

On reception of the AAI_CC-CMD message, the AMS shall respond with an AAI_CC-IND message. On the Action time, the AMS shall activate the target secondary carrier and be ready to transfer data on the target secondary carrier. If the received AAI_CC-CMD message contains a dedicated ranging code, the AMS may perform initial ranging on the target secondary carrier by using the dedicated ranging code. In this case, reception of the dedicated code may give the ABS a confirmation that the secondary carrier has been successfully activated.

Figure x Example of secondary carrier activation procedure

When the ABS deactivates an active secondary carrier of the AMS, it sends an AAI_CC-CMD message to the AMS, which includes the Logical Carrier Index of the secondary carrier to be deactivated. If the AMS receives the AAI_CC-CMD message, the AMS shall deactivate the secondary carrier and respond result with an AAI_CC-IND message.

The secondary carrier activation/deactivation may be performed by using the service flow management messages during a service flow addition/deletion procedure.

# Proposed Text

----------------------------------------------------------- Text Start ---------------------------------------------------------------

#### 15.2.x.x.x Secondary Carrier Activation/Deactivation

The ABS may activate/deactivate an inactive assigned secondary carrier of the AMS for load balancing, required QoS due to addition/deletion of a service flow. The secondary carrier activation/deactivation procedure can be triggered by ABS or requested by the AMS.

When an inactive assigned secondary carrier needs to be activated, the ABS may request channel measurement on the inactive carriers to the AMS through an AAI_SCN-RSP message. In this case, upon reception of the AAI-SCN-RSP message, the AMS performs scanning by using any unused transceiver. Then, the AMS reports the scanning results through an AAI_SCN-REP message to the ABS. The ABS determines the secondary carrier to be activated, based on the measurement information, load balance, and other factors. The ABS may activate a pre-assigned carrier for E-MBS service without collecting measurement information. In order to command the AMS to activate a secondary carrier, the ABS sends an AAI_CC-CMD message which may include the following information,

- Physical Carrier Index of the target secondary carrier

- Logical Carrier Index of the target secondary carrier

- Ranging information (optional)

- Action time

On reception of the AAI_CC-CMD message, the AMS shall respond with an AAI_CC-IND message. On the Action time, the AMS shall activate the secondary carrier and be ready to transfer data on the secondary carrier. If the received AAI_CC-CMD message contains a dedicated ranging code, the AMS may perform initial ranging on the secondary carrier by using the dedicated ranging code. In this case, if the ABS receives the dedicated code, it considers that the secondary carrier has been successfully activated.

Figure x Example of secondary carrier activation procedure

When the ABS deactivates any active secondary carrier of the AMS, it sends an AAI_CC-CMD message to the AMS, which includes the Logical Carrier Index of the secondary carrier to be deactivated. If the AMS receives the AAI_CC-CMD message, the AMS shall deactivate the secondary carrier and respond result with an AAI_CC-IND message.

The secondary carrier activation/deactivation may be performed by using the service flow-related management messages during a service flow addition/deletion procedure.

----------------------------------------------------------- Text End ---------------------------------------------------------------

# References

- Shkumbin Hamiti, “IEEE 802.16m System Description Document,” IEEE 802.16m-08/003r8, April, 2009

- Juhee Kim, Eunkyung Kim, Soojung Jung, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon, “Proposed IEEE 802.16m Amendment Text on Primary Carrier Change,” IEEE 802.16m-09/0889, May, 2009

----------------------------------------------------------- Text End ---------------------------------------------------------------