| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | - Text proposal on DL MAP |  |
| Date Submitted | 2009-4-27 |  |
| Source(s) | Amir Khojastepour, Narayan Prasad, Sampath Rangarajan, Nader Zein, Tetsu Ikeda, Andreas Maeder NEC Laboratories America | amir@nec-labs.com |
| Re: | 802.16m call for comments and contribution on AWD |  |
| Abstract | This contribution provides contributions to the interference mitigation among multiple cells. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

15.3.6.4.2.2 DL basic assignment A-MAP IE

Table 661 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL .

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 661.

Table 661: DL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: Modulation aware Horizontal encoding |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b0: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between UL/DL, basic/extended IEs. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format

PSI: Allocated pilot stream index for horizontal encoding

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

RA: Resource Allocation information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

Long TTI Indicator: Indicator to signal allocations span multiple subframes in time.

Allocation Relevance: Subframe index corresponding to the A-MAP region in which the resource is allocated

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

- The following additional MEF mode is suggested. In this “modulation aware” horizontal encoding (MU-MIMO) mode each user is informed about the modulation corresponding to the other co-scheduled users (streams)

| } else if(MEF == 0b11){ |  | Parameters for “modulation aware” horizontal encoding |
| --- | --- | --- |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| If (Mt == 0b1){ |  |  |
| MDF | 2 | 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| } |  |  |
| }else |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| If (Mt ==0b01){ |  |  |
| MDF | 2 | 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: n/a |
| }else if (Mt ==0b10){ |  |  |
| MDF | 4 | 0b0000: QPSK,QPSK 0b0001: QPSK,16 QAM 0b0010: QPSK, 64 QAM 0b0011: 16QAM, QPSK 0b0100: 16QAM,16QAM 0b0101: 16 QAM,64QAM 0b0110: 64 QAM,QPSK 0b0111: 64QAM, 16QAM 0b1000: 64QAM, 64QAM other possibilities n/a |
| }else if (Mt ==0b11){ |  |  |
| MDF | 5 | 0b00000: QPSK,QPSK, QPSK 0b00001: QPSK, QPSK, 16 QAM 0b00010: QPSK, QPSK, 64 QAM 0b00011: QPSK, 16QAM, QPSK 0b00100: QPSK, 16QAM,16QAM 0b00101: QPSK, 16 QAM, 64QAM 0b00110: QPSK, 64 QAM,QPSK 0b00111: QPSK, 64QAM, 16QAM 0b01000: QPSK, 64QAM, 64QAM 0b01001: 16QAM, QPSK, QPSK 0b01010: 16QAM, QPSK, 16 QAM 0b01011: 16QAM, QPSK, 64 QAM 0b01100: 16QAM, 16QAM, QPSK 0b01101: 16QAM, 16QAM,16QAM 0b01110: 16QAM, 16 QAM,64QAM 0b01111: 16QAM, 64 QAM,QPSK 0b10000: 16QAM, 64QAM, 16QAM 0b10001: 16QAM, 64QAM, 64QAM 0b10010: 64QAM, QPSK, QPSK 0b10011: 64QAM, QPSK, 16 QAM 0b10100: 64QAM, QPSK, 64 QAM 0b10101: 64QAM, 16QAM, QPSK 0b10110: 64QAM, 16QAM,16QAM 0b10111: 64QAM, 16 QAM,64QAM 0b11000: 64QAM, 64 QAM,QPSK 0b11001: 64QAM, 64QAM, 16QAM 0b11010: 64QAM, 64QAM, 64QAM other possibilities n/a |
| } |  |  |
| } |  |  |

MDF: Modulation Field