| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Need of Primary Carrier Change/Switch for Multi-carrier Operation in IEEE 802.16m |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Eunkyung Kim, Juhee Kim, Soojung Jung, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-5415 E-mail: ekkim@etri.re.kr |
| Re: | Category: AWD-New contribution/Area: Multi-carrier, “Contribution on AWD Multi-carrier,” in response to the TGm Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content, IEEE 802.16m-09/0020 for session #61 |  |
| Abstract | Sleep mode operation in the MAC aspect of multi-carrier operation |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Need of Primary Carrier Change/Switch for Multi- carrier Operation in IEEE 802.16m

Eunkyung Kim, Juhee Kim, Soojung Jung, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon ETRI

# 1 Instructions

The draft of IEEE 802.16m System Description Document (SDD)[1] describes that all messaging are handled by the primary carrier and primary carrier change/switch are also performed for load balancing, carriers’ varying channel quality or other reasons. Those agreements are applied to the active/normal mode AMSs as well as sleep mode AMSs. In this contribution, we describe why the primary carrier change/switch is needed upon initiating sleep mode of AMS to decrease the signaling overhead during sleep mode.

# 2. Primary carrier change for multi-carrier operation

Generally primary carrier change is performed in the purpose of load balancing or carriers varying channel quality in both active mode and sleep mode. In the ABSs’ view point, some broadcast message should transmit via primary carrier of AMS which makes some duplicated broadcasting message (e.g., AAI_TRF-IND in the sleep mode) which is depicted in the Fig. 1. As shown the figure, the traffic indication should be transmitted via primary carrier of each sleep mode AMS. It makes the signaling overhead because AAI_TRF-IND message is transmitted via primary carrier which is dependent on the number of managed primary carrier between ABS and sleep mode AMSs.

In order to alleviate the signaling overhead in terms of number of transmitted signaling message, we propose the primary carrier grouping of sleep mode AMS on entering sleep mode. Fig. 2 shows the example of decreasing the traffic indication by grouping primary carrier of sleep mode AMS on entering sleep mode. As shown the figure, if the primary carrier change is performed, the signaling overhead in terms of number of transmitted AAI_TRF-IND message may be alleviated. Therefore, we need consider alleviating the signaling overhead as one of the reasons why primary carrier change/switch is performed for multi-carrier operation especially in sleep mode.

Fig. 1. Overhead of the broadcasting message (i.e., AAI_TRF-IND) in the listening window due to transmission of signaling via primary carrier in the sleep mode

Fig. 2. Grouping the primary carrier of the sleep mode AMS to alleviate the braodcast message (i.e., AAI_TRF-IND)

# 3. Text Proposal for the 802.16m AWD

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Adopt the following text in the 802.16m Amendment Working Document (AWD)]

15. Advanced Air Interface

15.2 Medium Access Control

15.2.x.x MAC aspect of OFDMA Multi-carrier operation

15.2.x.x.x Power Management

15.2.x.x.x.1 Sleep mode

Note that the serving ABS may request AMS to change or switch its primary carrier on entering the sleep mode or during the listening window for load balancing, power saving, or alleviating singling overhead.

[-------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

- Shkumbin Hamiti, “IEEE 802.16m System Description Document,” IEEE 802.16m-08/003r8, April, 2009