| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Interference Mitigation in the 802.16m AWD |  |
| Date Submitted |  |  |
| Source(s) | Clark Chen, Shilpa Talwar, Hongmei Sun, Shantidev Mohanty, Hsuan Yi, Jong-Kae Fwu, Hujun Yin; Intel Corporation; Zhaohua Lu, Kun Liu, Yanfeng Guan, Ying Liu, ZTE Corporation Kiran Kuchi, Klutto Milleth CWiT | Clark.Chen@intel.com Lu.Zhaohua@zte.com.cn |
| Re: | 802.16m AWD |  |
| Abstract | The contribution proposes text for the Interference Mitigation to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Interference Mitigation in the 802.16m AWD

- Clark Chen, Shilpa Talwar, Hongmei Sun, Shantidev Mohanty, Hsuan Yi, Jong-Kae Fwu, Hujun Yin;

- Intel Corporation

- Zhaohua Lu, Kun Liu, Yanfeng Guan, Ying Liu,

- ZTE Corporation

- Kiran Kuchi, Klutto Milleth

- CWiT

- Introduction

This contribution provides amendment text for Interference Mitigation in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The amendment text was developed using baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2], and follow the amendment text style guide [3]. Further details of the signaling procedure between ABS and AMS to support Downlink Fractional Frequency Reuse (FFR) are provided. Performance evaluation of FFR has been previously demonstrated in [4].

- References

[1]	P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

[2]	IEEE 802.16m System Description Document (SDD) [Draft], IEEE 802.16m-08/003r8.

[3]	Style guide for writing the IEEE 802.16m amendment, IEEE 802.16m-08/043

[4]	Adaptive Frequency Reuse in IEEE 802.16m, C80216m-08_702r3,

Text proposal for inclusion in the 802.16m amendment

------------------------------- Text Start -------------------------------------

# Interference Mitigation

Details of Interference Mitigation techniques are described in the following sections.

# 20.1. Downlink Fractional Frequency Reuse

All BSs that share the same frequency band in neighbor area may apply Fractional Frequency Reuse. A FFR_ENABLE bit in SFH will indicate whether FFR is enabled for this BS or not. In case when FFR is enabled, the total available frequency band is divided into 3 or 4 partitions. The FFR partition boundary is aligned with PRU units. The FFR partitions are indexed from lower Logic Resource Unit (LRU) index to highest LRU index. It always starts from reuse-1 partition and then followed by the three reuse-3 partitions. They are numbered as FFR partition(0), partition(1), partition(2), and partition(3) respectively.

Each partition may have different power level per sector.  The transmission power level on different FFR partitions is decided by BS.

FFR is supported with necessary BS/MS signaling and corresponding operation procedure as defined in sections 20.1.1 to 20.1.x.

# DL Signaling support for Fractional Frequency Reuse

[The following text should be consistent with section 15.3.5.2.]

Essential system parameters and system configuration information is carried in the S-SFH, and is categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted with different timing and periodicity.

For FFR, the S-SFH SPx IE includes FFR partition size information needed for MS to conduct correct measurements. The period to transmit S-SFH SPx IE is N superframes, where the value of N is FFS.  Table x represents the S-SFH SPx IE.  If FFR is enabled, a frequency partition configuration indicator is transmitted in the SFH, and is termed the Downlink Frequency Partition Configuration Indicator (DFPCI).  The DFCPI s either 4, 3 or 2-bits depending on the system bandwidth. The number of frequency partitions which is defined by a parameter Frequency Partition Count (FPCT), and the number of PRUs allocated to frequency partition i which is defined by a parameter Frequency Partition Size (FPSi),  are determined from DFPCI as shown in Table 651 and Table 652 and Table BB.

[This section should be consistent with section 15.3.5.2.1.]

Table x S-SFH SPx IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SPx IE format () { |  |  |
| … |  |  |
| FFR_ENABLE | 1 | 0 if FFR is not enabled (pure reuse-1 system) 1 if FFR is enabled (4 FFR partitions or 3 partitions where reuse-1 partition size is zero) |
| If (FFR_ENABLE == 1) { |  |  |
| DFPCI | 4~2 | The Downlink FFR partition configuration is defined by Downlink Frequency Partition Configuration Indicator (DFPCI) The sequence is from low logical index of frequency partition to high logical index of frequency partition. For 20MHz bandwidth, DFPCI is 4 bits which defines 16 configurations; for 10MHz, 8.75Mhz, and 7MHz bandwidth, DFPCI is 3 bits, which defines 8 configurations. For 5MHz, DFPCI is 2 bits, defining 4 configurations. The FPCI and FFR partitions size mapping is defined in table 651, BB, 652. |
| } |  |  |
| … |  |  |
| } |  |  |

[Raplace Table 651 in line 16 on page 37 , in 15.3.5.2.3, as following]

Table 651– Mapping between DFPCI and frequency partitioning for 20MHz

| DFPCI | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 96：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：32：32：32 | 3 | 0 | NPRU * 32/96 |
| 2 | 12：28：28：28 | 4 | NPRU * 12/96 | NPRU * 28/96 |
| 3 | 15：27：27：27 | 4 | NPRU * 15/96 | NPRU * 27/96 |
| 4 | 18：26：26：26 | 4 | NPRU * 18/96 | NPRU * 26/96 |
| 5 | 21：25：25：25 | 4 | NPRU * 21/96 | NPRU * 25/96 |
| 6 | 24：24：24：24 | 4 | NPRU * 24/96 | NPRU * 24/96 |
| 7 | 27：23：23：23 | 4 | NPRU * 27/96 | NPRU * 23/96 |
| 8 | 30：22：22：22 | 4 | NPRU * 30/96 | NPRU * 22/96 |
| 9 | 36：20：20：20 | 4 | NPRU * 36/96 | NPRU * 20/96 |
| 10 | 42：18：18：18 | 4 | NPRU * 42/96 | NPRU * 18/96 |
| 11 | 48：16：16：16 | 4 | NPRU * 48/96 | NPRU * 16/96 |
| 12 | 54：14：14：14 | 4 | NPRU * 54/96 | NPRU * 14/96 |
| 13 | 60：12：12：12 | 4 | NPRU * 60/96 | NPRU * 12/96 |
| 14 | 66：10：10：10 | 4 | NPRU * 66/96 | NPRU * 10/96 |
| 15 | 72：8：8：8 | 4 | NPRU * 72/96 | NPRU * 8/96 |

Table BB– Mapping between DFPCI and frequency partitioning for 7MHz ,8.75MHz and 10MHz

| DFPCI | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 48：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：16：16：16 | 3 | 0 | NPRU * 16/48 |
| 2 | 9：13：13：13 | 4 | NPRU * 9/48 | NPRU *13/48 |
| 3 | 12：12：12：12 | 4 | NPRU * 12/48 | NPRU * 12/48 |
| 4 | 18：10：10：10 | 4 | NPRU * 18/48 | NPRU * 10/48 |
| 5 | 21：9：9：9 | 4 | NPRU * 21/48 | NPRU * 9/48 |
| 6 | 24：8：8：8 | 4 | NPRU * 21/48 | NPRU * 8/48 |
| 7 | 30：6：6：6 | 4 | NPRU * 30/48 | NPRU * 6/48 |

[Change Table 652 in line 39 on page 37 , in 15.3.5.2.3, as following]

Table 652—Mapping between DFPCI and frequency partitioning for 5MHz

| DFPCI | Freq. Partitioning (FP0:FP1:FP2:FP3) | FPCT | FPS0 | FPSi (i>0) |
| --- | --- | --- | --- | --- |
| 0 | 24：0：0：0 | 1 | NPRU | 0 |
| 1 | 0：8：8：8 | 3 | 0 | NPRU * 8/24 |
| 2 | 6：6：6：6 | 4 | NPRU * 6/24 | NPRU * 6/24 |
| 3 | 12：4：4：4 | 4 | NPRU * 12/24 | NPRU * 4/24 |

In addition to FFR configuration information, S-SFH SPx IE includes resource metric information needed for MS to select DL frequency partition in case of distributed permutation, or to select subchannel in case of localized Permutation, as indicated in Table y. The period to transmit S-SFH SPx IE is N superframes, where value of N is FFS.

Table y S-SFH SPx IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SPx IE format () { |  |  |
| … |  |  |
| If (FFR_ENABLE == 1){ |  |  |
| for (i=0; i < 2; i++) { |  |  |
| Resource_Metric | 8 | Resource metric for two power de-boosted frequency partitions in reuse 3 frequency region. Resource metric of the partition with lower partition index is sent first, followed by the resource metric of the other partition. Example: Partition(0) is reuse-1 partition, partition(1) is reuse-3 with power boosting, partitions(2) and partition(3) reuse-3 with power de-boosting; the resource metric of partition(2) will be sent first, and partition(3) will be sent after that. The resource metric is defined as a fractional number between 0 and 1. It is encoded as unsigned digital as following with resolution 1/ 256. Resource metric of FFR partition(0) with reuse-1 is fixed as ‘1’; Resource metric of FFR partition(i) with power boosting is calculated as following: resource metric(i) = 3 – sum(resource metric of partition with power de-boosting); |
| } |  |  |
| … |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

# UL Signaling support for Fractional Frequency Reuse

# Long-Term Feedback to support FFR

AMS should be able to do Signal/Interference measurements on different FFR partitions, and feedback these to home BS, as requested.

The following measurements should be supported:

The average Signal Level of the three preamble groups corresponding to home ABS;

The average Interference and Noise Level of the three preamble groups;

[TBD]

# Fast Feedback to support FFR

[This section should be consistent with section 15.3.9.3.1.]

A Frequency Partition Selection Indicator (FPSI) is required for MS using Distributed Permutation to feedback preferred partition. The FPSI is sent in P-CQICH channel, using 4 codewords in the 6-bit CQICH code space. The FPSI is defined as following:

| 58 | 0b111010 | Select FFR partition 1 (reuse-1) | PFBCH | The feedback CQI afterward is for this FFR partition. |
| --- | --- | --- | --- | --- |
| 59 | 0b111011 | Select Frequency partition 2 (reuse-3) | PFBCH | Same as above |
| 60 | 0b111100 | Select Frequency partition 3 (reuse-3) | PFBCH | Same as above |
| 61 | 0b111101 | Select Frequency partition 4 (reuse-3) | PFBCH | Same as above |

For MS using localized permutation, it reports the preferred sub-channels and their corresponding CQI in CQICH channel according to the signaling defined in section 15.3.9.3.x.  The selection of preferred partition in case of distributed permutation or of preferred subchannels in case of contiguous permutation is described below.

[Descriptive text: When AMS is using DRU permutation, it indicates to BS its preferred FFR partition for downlink transmission. BS can then allocate DL resources in this preferred FFR partition.

AMS selects the preferred FFR partition as following:

AMS estimates the average SINR on each FFR partition, and then estimates the MCS Rate according to Link Adaptation procedure. The Estimated MCS Rate and PER, is used to calculate the expected SE on the next frame as follow:

Expected_SE = MCS_Rate * (1-PER);

AMS then calculates the normalized SE of each partition as follows:

Normalized_SE(i) = Expected_SE(i) / Resource_Metric(i);

AMS compares the Normalized_SE(i) of each partition, and selects the Partition with maximum Normalized_SE(i). The Normalized SE(i) can be smoothed (filtered) to avoid rapid changes in partition selection.  If this Normalized_SE(i) is of a different FFR partition than current partition choice, and it is x% (FFS) higher than current partition’s normalized SE, AMS should report a switch of FFR partition and report CQI in CQICH channel according to the signaling defined in section 15.3.9.3.1.

When AMS is using LRU permutation, and Best-M feedback is enabled, it indicates to BS its preferred sub-channels for downlink transmission. BS can allocate DL resources in this preferred sub-channel, and also can allocate DL transmission in other sub-channel as needed.

AMS selects the preferred sub-channels as following:

AMS estimates the average SINR on each sub-channel of all FFR partitions, and then estimates the MCS Rate according to Link Adaptation procedure. The Estimated MCS Rate and PER of each sub-channel, is used to calculate the expected SE on the next frame as follow:

Expected_SE = MCS_Rate * (1-PER);

AMS then calculates the normalized SE of each sub-channel as follow:

Normalized_SE(m) = Expected_SE(m) / Resource_Metric(i);

Here, ‘i’ is the FFR partition index of the corresponding sub-channel ‘m’.  AMS then selects the sub-channel with maximum Normalized_SE(m).

End Descriptive text]

# Signaling operation procedure between ABS and AMS

The following sections define the signaling operation procedure between ABS and AMS to support FFR.

# BS Broadcast and Update of DL FFR information

Figure. Xxx

# AMS system entry procedure in case of FFR

Figure xxxx.

AMS always selects reuse-3 with high transmission power when it enters the system. Only after AMS has received one superframe which has the resource metric information, it will start to use the resource metric to select the preferred partition in case of DRU, or select the preferred sub-channels in case of LRU, as defined in section 20.1.2.2.

AMS sends back the CQI of corresponding partitions during at the initial stage of system entry, and after it receives the resource metric of each partition, it shall select the preferred partition and send back corresponding FPSI and CQI via the CQICH channel, according to signaling defined in section 15.

------------------------------- Text End -------------------------------------