| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed IEEE 802.16m Amendment on Handover with Multi-Carrier Support |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee Chulsik Yoon ETRI | E-mail: sjjung@etri.re.kr |
| Re: | Category: AWD-New contribution /Area: Multi-Carrier |  |
| Abstract | This contribution proposes IEEE 802.16m AWD on Multi-Carrier operation |  |
| Purpose | Proposed to be discussed and adopted by TGm for the 802.16 AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed IEEE 802.16m Amendment on Handover with multi-carrier support

Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha, Kwangjae Lim, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

This contribution proposes amendment text on handover procedure for the WirelessMAN-OFDMA Advance System with multi-carrier support.

# Handover procedure with multi-carrier support

When an AMS, serving ABS and target ABS can support multi-carrier operation, the information about the network topology may be carried using the broadcast information. Therefore messages broadcasting system information shall include the information of neighbor ABS’s multi-carrier configuration. And the physical carrier index may be used to inform the candidate carrier to be assigned for performing scanning and network re-entry procedure. Therefore the messages in relation with Handover procedure may include the physical index of carrier.

# Proposed Text

----------------------------------------------------------- Text Start ---------------------------------------------------------------

#### 15.2.x.x.x AMS scanning of neighbor ABS

An ABS shall broadcast information about the network topology using the AAI_NBR-ADV message. The message shall include the information of neighbor ABSs’ multi-carrier configuration.

An AMS capable of concurrently processing multiple radio carriers may perform scanning with neighbor ABSs using one or more of its available radio carriers while maintaining normal operation with the serving ABS on the primary carrier and secondary carriers. An AMS scans each fully configured carrier of the neighbor ABSs in AAI_NBR-ADV.  To avoid resource allocation on these active carriers, the AMS may inform the serving ABS through AAI_SCN-REQ its one or more candidate carriers to be assigned for performing scanning operation. For each candidate carrier, the information related to scanning interval may be included.  Upon reception of the AAI_SCN-REQ messages, the serving ABS shall respond with an AAI_SCN-RSP message to grant or assign other carriers. The physical carrier index is used to inform the carrier index to be assigned for scanning operation in AAI_SCN-REQ/RSP/REP.

#### 15.2.x.x.x Handover process

HO process in multi-carrier operation follows the process defined in section 15.2.5.2.  An AMS capable of concurrently processing multiple radio carriers negotiates with serving ABS and the target ABSs whether they use the Multi-carrier capability or not by using Multi-carrier_ Mode in AAI_HO-REQ, AAI_HO-CMD, AAI_HO-IND. Upon reception of the AAI_HO-REQ with Multi-carrier_ Mode=1, the serving ABS shall negotiates with target ABS selected for HO. If the target ABS can support multi-carrier handover, the serving ABS shall response with AAI_HO-CMD message with Multi-carrier_Mode=1 and Reentry_mode=1. In this case, Disconnect_time has to be set to sufficient value such that network reentry procedure to target ABS is completed before the expiration of Disconnect_time. And one or more physical carrier indices of active carrier in serving ABS and one or more physical carrier indices of target ABS are included to inform the carrier Indices to be assigned for network reentry operation. If the serving ABS completed exchange and negotiation with the target ABS for the information needed for multi-carrier operation in target ABS during HO negotiation, the logical carrier Indices related to those carriers may also be included in AAI_HO-CMD message. In case of AAI_HO-CMD message with multiple target ABS and carriers information, the target ABS’s index and physical index of each candidate carrier may be used to distinguish each other. And one or more action times may be included to avoid resource allocation on those carriers to be assigned for performing network re-entry procedure. When multiple target ABSs and carrier Indices are included in AAI_HO-CMD message, the AMS may inform the serving ABS of target ABS and the carrier assigned for network reentry operations through AAI_HO-IND message.

Figure xxa shows an example call flow of HO with Multi-carrier_Mode=1 in which the AMS maintain data communication with the serving ABS on one radio carrier while performing network reentry to the target ABS on another radio carrier.

Figure xxa — A call flow for multi-carrier HO in which the AMS performs network reentry to the target ABS on one radio carrier while maintaining communication with the serving ABS on another.

In case that AMS is capable to process multiple carriers at the same time and Multi-carrier_Mode is set to 1 in HO related messages, the target primary carrier can be different from the one chosen in serving cell. Figure xxb shows an example HO call flow of the case in which AMS is capable to process multiple carriers at the same time and the target primary carrier is different from the serving primary carrier.

Figure xxb — A call flow for multi-carrier HO in which the AMS performs network reentry on the target primary carrier which is different from the serving primary carrier

After the serving ABS received a message which is used to notify the completion of network reentry by target ABS through backbone, the serving ABS releases the AMS’s context in serving ABS and transfer that to target ABS if it is need. In this case, the target ABS shall start the data communication with AMS upon receiving AMS’s context

----------------------------------------------------------- Text End ---------------------------------------------------------------