| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Construction & Transmission of MAC PDU section for AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sungjin Lee, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics Youngjoo Tcha KT | steve.lee@samsung.com yjtcha@kt.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: MAC PDU formats |  |
| Abstract | This contribution proposes text for MAC PDU Formats section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC PDU Formats in 16m

Sungjin Lee, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

Youngjoo Tcha

KT

# Introduction

This contribution proposes the text for MAC PDU Formats section based on SDD [2].

# Text Proposal for inclusion in the 802.16m amendment working document

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.x as follows:]

15.2.x MAC PDU Formats

Each MAC PDU contains a MAC header. The MAC PDU may contain payload. The MAC PDU may contain one or more extended headers.

Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU.

15.2.x.1 Generic MAC Header

| FlowID(4) | EH (1) | Length (3) |
| --- | --- | --- |
| Length (8) |  |  |

Figure xx Generic MAC Header Format

- FlowID (Flow Identifier): This field indicates the service flow that is addressed. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length (or BR MSB): Length of the payload. This field is 11bits long. This field may be interpreted as BR MSB if a BR extended header follows this MAC header format. For more specific see 15.2.x.4 MAC Header without payload

  - Bandwidth Request (BR MSB): The number of bytes of UL bandwidth requested by the MS. The BR is for the FlowID. The request shall not include any PHY overhead.

Table xx.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAC Header () { |  |  |
| FlowID | 4 |  |
| EH | 1 |  |
| if (EH==0) { |  | If there is no extended header |
| Length | 11 | Length of the payload |
| } |  |  |
| Else { |  |  |
| if (EHT ==0000){ |  | Refer to the value of EHT field in Extended Header |
| BR MSB | 11 |  |
| } |  |  |
| Else { |  |  |
| Length | 11 | Length of the payload |
| } |  |  |

15.2.x.2 Compact MAC Header

Compact header is used for connections with persistent allocation and group allocation.

| EH (1bit) | Length (7bits) |
| --- | --- |

Figure xx Compact header format

EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this header.

Length (7): Length of the payload.

15.2.x.3 Extended Headers

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in Figure xx and will be used unless specified otherwise.

| L(1) | EHT(4) | EH Contents(3) |
| --- | --- | --- |
| EH Contents(8) |  |  |

Figure xx Extended Header Format

- L (Last): When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set this extended header is the last one.

- EHT (Extended Header Type) indicates the type of extended header. The length is TBD.

- Body Contents: EHT dependent contents.

15.2.x.3.1 MAC Header without payload

This extended header is used for MAC header without payload. see 15.2.x.4 MAC Header without payload

15.2.x.3.2 FPEH

15.2.x.3.3 Grant Management EH

Table xx.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Grant Management EH { |  |  |
| L | 1 |  |
| EHT | 4 |  |
| Reserved | 3 |  |
| if (scheduling service type == UGS) { |  |  |
| SI | 1 |  |
| PM |  |  |
| FLI |  |  |
| FL |  |  |
| Reserved | 9 |  |
| } else if (scheduling service type == ertPS){ |  |  |
| Extended piggyback request | 11 |  |
| FLI | 1 |  |
| FL | 4 |  |
| } else { |  |  |
| Piggyback Request | 16 |  |
| } |  |  |
| } |  |  |

15.2.x.4 MAC Header without payload

15.2.x.4.1 Bandwidth Request

| L | EHT | RT | SI | TI |
| --- | --- | --- | --- | --- |
| BR LSB (8) |  |  |  |  |
| STID (length=TBD) |  |  |  |  |
| Tx Power |  |  |  |  |

- L (Last): When the “Last” bit is set, another extended header will follow the current extended header. If this bit is not set this extended header is the last one.

- EHT= 0000

- RT (Request Type) : [0] Incremental [1] Aggregate

- SI (STID presence indicator) : [0] Included [1] Not included

- TI (UL Tx Power Report Indicator) : [0] Included [1] Not included

- BR LSB : LSB 8 bits of BR

- STID : Station ID of the MS transmitting this message

- UL TX POWER : UL Tx power level in dBm for the burst that carries this header (as described in x.x.x). The value shall be estimated and reported for the burst.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Bandwidth Request EH{ |  |  |
| L | 1 |  |
| EHT | 4 | Extended Header Type = (TBD) |
| RT | 1 | Request Type |
| SI | 1 | STID presence indicator |
| TI | 1 | Tx Power Report indicator |
| BR LSB | 8 |  |
| if (SI=0){ |  |  |
| STID | TBD |  |
| } |  |  |
| if (TI=0) { |  |  |
| UL TX POWER | 8 |  |
| } |  |  |
| } |  |  |

# Reference

[1] IEEE 802.16m-09/0010r1a, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r8, “802.16m System Description Document (SDD)”