| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Interference Mitigation for the IEEE 802.16m Amendment Working Document |  |
| Date Submitted | 2009-05-04 |  |
| Source(s) | Jihyung Kim, Rong Ran, Wooram Shin, Suchang Chae, Dong Seung Kwon ETRI | Voice: +82-42-860-6687 E-mail: sunny_rr@etri.re.kr savant21@etri.re.kr |
| Re: | 802.16m AWD IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “Interference Mitigation”. |  |
| Abstract | The contribution proposes the text of a interference mitigation section to be included in the IEEE 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

PMI Combination Proceeding in Uplink for Improvement of Throughput in A Multi-Cell Environment

Jihyung Kim, Rong Ran, Wooram Shin, Suchang Chae, Dong Seung Kown

ETRI

- Introduction

Beamforming (BF) is the one of popular techniques for reducing inter-cell interference (ICI) for multi-cell MIMO systems. Since a users always wants to select the proper precoding matrix index (PMI) for maximizing signal to interference plus noise ratio (SINR), which perhaps cause very serious interference to some cell-edge users of other cells, a combination of the PMI which maximizes SINR to its own cell and one which minimizes ICI to other users of neighboring cells should be concerned in order to improve the overall system throughput and the edge area throughput.

- Proposed Scheme

To increase the overall system throughput and edge area throughput in UL closed-loop MIMO, we propose a PMI combination scheme in the multi-cell environments. This scheme actually takes a tradeoff between maximizing a target user’s SINR of its own cell and minimizing ICI to users of neighboring cells.

In order to support PMI combination to mitigate inter-cell interference, the ABSs are capable of exchanging the interference measurement results such as the recommended PMI subset to be restricted or to be applied in neighboring cells with each other or some control element in the backhaul network. Furthermore, for UL PMI combination, the ABS is capable of measuring the channel from the interfering AMS using sounding signals.

Fig.1. Two-tier cell structure

Fig.1 shows two-tier cell structure. Each cell is partitioned into three sectors. Channels between AMSs and ABSs in multi-cell environments are divided into three categories. The first category of channels from the  to the BS of its own cell () is named as an own channel (OC), the second one from the  of the cth cell to the  or from the  of the cth cell to the  is called as an imported interference channel (IIC) and the final one from  to  or from  to is called as an exported interference channel (EIC). In this contribution, one-stream signal, one antenna of a BS and two antennas of a user are considered. The extension to multiple streams of signals and multiple antennas of BSs is straightforward.

The received signal at the BS can be written as follows:

,                              (1)

where , and  with  denote the received and transmitted signals of the kth subcarrier in the cth cell, respectively.  is an additive Gaussian noise variable with a zero mean and variance . Since a two-tier system is considered, the total number of cells which may suffers ICI is 18, i.e.,.  is a uniform BF value corresponding to the ith transmitted antenna. with denotes the coefficient of the channel from the ith transmitted antenna to the BS. The SINR of (1) can be described as follows:

.                                        (2)

Recalling of the combination of BF vectors [1][2],

,                           (3)

where  and  denote a BF vector for maximum-ration transmission (MRT) corresponding to OC and one for zero-forcing (ZF) corresponding to EIC, respectively. The MRT gain is increased as  is increased, while ICI to other users of neighboring cells is also increased. Thus,  should be properly selected. Based on (3), a combination of PMIs in UL is motivated.

Time division duplex (TDD) mode, in which channels of UL and DL are reciprocal, is firstly considered. First, a user, e.g. , estimates OC, IIC and the propagation loss at adjacent sectors by using common pilot signals such as preamble, midamble etc.

If the propagation loss of the channel from  to the  of  is denoted as . Similarly, denotes the propagation loss of the channel from  to  of  with , a sector and the corresponding BS of neighboring cells with the largest potential interference caused by the will be selected through the following criteria:

,                                   (4)

where  denotes a threshold which is decided based on the range for valid IIC estimation. Note that only one is determined because the spatial degree of freedom is two. PMIs of  and  in a predefined codebook can be selected by the target user through the following equations:

,                               (5)

whereis the PMI in the codebook,  is the subcarrier index and  is the symbol index. The  resource block can be a minimum resource unit or a subband allocated to the target user. Then,  and  will be transmitted to the user’s own BS through the feedback channel. Each BS broadcasts the information of its interference level, e.g., interference over thermal (IoT) level, to users of adjacent sectors. So the user can determine  according to the received interference level of the  of the . Since the value of  is listed in a predefined table which is also known by the BS, the BS need to find which  is to be used by the user through the information of interference level known from the backhaul connection. Another way is BSs determine  in advance and transmit it to users instead of broadcasting the information of their interference level. If interference level is low,  is closed to 1. Note that interference level at the nth frame is calculated by taking the average interference value from the (n-S)th frame to the (n-1)th frame, where S is the window size.

The procedure of PMI combination in frequency division duplex (FDD) mode is different with that in TDD mode due to the lack of reciprocity of channels, which is listed as follows:

- The user () determines by using (4).

- The BS () estimates OC and IIC by using common pilot signal such as sounding channel and calculates and  through (5). The IIC at the  of  is equivalent to the EIC of the user in another sector.

- BSs exchange the IoT level and .

- is determined by the BS and transmitted to the user with  and .

- Analysis and Comparison

Upon the described procedure, the range of users who can use PMI combination for a specified BS is limited in one-tier. Moreover, only users who have can use PMI combination. Thus, SINR of (2) can be rewritten as follows:

,                                 (6)

where is the controlled ICI with PMI combination of users who have and is the uncontrolled ICI of users who have or have no . If is zero, and are closed to zero but not zero because of the difference between the codebook and the optimal BF vector.  If is one, is closed to two. In other words, when a user in  of  uses  with a low value, the transmitted power becomes low as well as the ICI power for .

- Performance Evaluation

The same codebook as that of IEEE802.16Rev2 [3] is used for link level simulation (LLS) and system level simulation (SLS) and given in Table I. Based on simulation parameters in [4][5], channels are extended ITU-R Ped-B (velocity:3km/h) and Veh-A (30km/h).  is given as .

Table I. Codebook for two transmitted antennas and one stream

| Vector index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| v1 | 1 | 0.7940 | 0.7940 | 0.7941 | 0.7941 | 0.3289 | 0.5112 | 0.3289 |
| v2 | 0 | -0.5801+j0.1818 | -0.0576+j0.6051 | -0.2978+j0.5298 | 0.6038+j0.0689 | 0.6614+j0.6740 | 0.4754-j0.7160 | -0.8779-j0.3481 |

Fig.2. shows the transmitted signal power of OC and ICI power of EIC with respect to PMIs. If BF vectors which match the channel are used, the signal power should vary from 0 to 2. Note that the power of signal varies from 0.18 to 1.82 in Fig.2. This is because of the difference between BF vectors and the codebook. Meanwhile, the ICI power of EIC keeps constant as one because EIC is independent to OC. Thus, if any factor concerned by EIC is not considered, using the PMI maximizing the signal power of OC is the best for increasing SINR.

- Fig.2. Signal power according to PMI

Fig.3. shows the signal power of OC and EIC with respect to  of PMI combination. In this figure, denotes the case that using  and  is delayed for 3 subframes (18 symbols) due to feedback. The power is increased as  is increased for all cases. For the Veh-A, the power is marginally larger than that for the Ped-B. The power for the Veh-Ad is reduced compared to that for the Veh-A due to the delay. The performances of three cases are deduced by Fig.3.

The first case is that two users affect each other. For example,  and , which use the same band, cause ICI to  of  and ICI to  of , respectively. In this case PMI combination has the performance gain according to different, which depends on the propagation loss such as the user location.

The second one is the extension of the first case, in which two users don’t affect each other directly. However, they affect each other through other users. For example,  and  use the same band.  causes ICI to  of ;  causes ICI to  of ;  causes ICI to  of ;  causes ICI to  to  and  causes ICI to  of . Thus  and  don’t affect each other directly but the effect does exist through other users. As users are connected like a ring, PMI combination () provides the same performance gain as that of the first case.

The final one is that there is no effect between two users. For example,  and  have the same band.  causes ICI to  of  but there is no  selected for . In this case the best performance can be achieved when  is equal to 1.

Fig.3. Signal power according to PMI combination

Table II shows the overall system throughput and the edge area throughput. The results show that the introduction of PMI combination in uplink multi-cell systems provides gains on the overall system throughput and edge area throughput as amount of 3-6% and 9-15%, respectively. In the Table II, ‘Case2’ denotes that the used bands for the cell-edge of all cells are the same.

Table II. Performance gain with PMI combination

|  | according to user location |  |  | Edge throughput (compared with 0.28Mbps) | Total throughput (compared with 3.86Mbps) |
| --- | --- | --- | --- | --- | --- |
|  | Cell-edge | Cell-mid | Cell-center |  |  |
| Case1 | 0.9 | 0.1 | 1 | 9% | 4% |
| Case2 | 0.5 | 0.1 | 1 | 15% | 3% |
| Case3 | 0.1 | 1 | 1 | -10% | 6% |

- Necessary Signaling

AMS transmits information of monitoring sectors to ABS:

ABS transmits information of PMI combination to AMS: , ,

ABS transmits information of PMI to another ABS:

- 6. Conclusion

A PMI combination proceeding for uplink multi-cell systems is proposed to improve throughput, which leads to a tradeoff between maximizing SINR of the user to its own cell and minimizing ICI caused by it to other users in neighboring cells. The simulation results show improvement of the overall system throughput and edge area throughput. Based in the above observation, we propose PMI combination for uplink multi-cell environment as one of technologies to achieve target requirements for IEEE 802.16m.

- 7. Reference

[1] Eduard A. Jorswieck, Erik G. Larsson, and Danyo Danev, “Complete Characterization of the Pareto boundary for the MISO Interference Channel”, IEEE Trans. on Signal Processing, Vol.56, Issue.10, pp.5292-5296, Oct. 2008.

[2] Zuleita ka Ming Ho and David Gesbert, “Spectrum sharing in multiple-antenna channels: A distributed cooperative game theoretic approach”, PIMRC 2008, pp.1-5, Sept. 2008.

[3] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems”.

[4] IEEE 802.16m-08/004r5, “IEEE 802.16m Evaluation Methodology Document (EMD)”.

[5] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”.

- 8. Text proposal for inclusion in the 802.16m amendment

--------------------------------------------------------------------- Text Start --------------------------------------------------------------------------------

15.x. Interference Mitigation using Advanced Antenna Technology

15.x.1. Single Cell Antenna Processing with Multi-ABS Coordination

15.x.1.x. Uplink Single Cell Antenna Processing

The serving ABS instructs the AMS to choose the combination of PMIs for maximizing SINR to its own cell (PMIOC) and minimizing the interference to neighboring cells (PMIEIC). For this, the AMS feeds back the ID of the dominant interfering BS when the interference level is over the threshold. The AMS chooses PMIOC and PMIEIC and feeds back to its own ABS in TDD mode. For FDD mode, the ABS chooses PMIOC and PMIEIC. The ABS is capable of measuring the channel from the interfering AMS using sounding signals. Neighboring ABS calculates the PMIEIC and forward them to the serving ABS with resource allocation information. The combination ratio (4 bits) between PMIOC and PMIEIC is determined by the ABS according to IoT level of neighboring cells and operation modes, and transmitted to AMS.

--------------------------------------------------------------------- Text End --------------------------------------------------------------------------------