| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to MAC PDU formats for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Eunjong Lee, Doo-hyun Sung, Jeongki Kim and Kiseon Ryu LG Electronics | Voice: +82-31-450-1910 E-mail: mehappy3@lge.com, dh.sung@lge.com, chegal@lge.com, ksryu@lge.com |
| Re: | Category: AWD New contribution / Area: MAC PDU Formats |  |
| Abstract | The contribution proposes the text of MAC PDU formats section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to MAC PDU formats Section for the IEEE 802.16m Amendment

Eunjong Lee, Doo-hyun Sung, Jeongki Kim and Kiseon Ryu

LG Electronics

# Introduction

This contribution includes the proposed text for MAC PDU formats in the IEEE 802.16m Amendment. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. In this contribution, amendment text including several illustrations is proposed for the section in which 16m members have enough consensus.

# Modifications from the SDD and key descriptions

  - The main concepts of the MAC PDU formats from “10.12 MAC PDU formats” in the IEEE 802.16m SDD [2] are maintained in this amendment text.

  - Key descriptions and motivations/rationales for our proposals are as follows.

    - ARQ feedback IE indication (section 15.2.1.3.2.1 and 15.2.1.3.2.3)

Generally, the MAC entity receiving the MPDU decodes the MAC header. After that, it delivers the MAC SDU(s) to higher layer without knowledge of the formats or bit patterns of those messages. In IEEE 802.16, the ARQ feedback information can be transmitted as a packed payload (“piggybacked”) within a packed MAC PDU or as a payload of a standalone MAC PDU [3]. If it is transmitted as a packed payload, the MAC entity should perceive the presence of ARQ feedback IE because it is control information of MAC layer, but not MAC management message. Therefore, we propose to define an ARQ feedback IE indication bit within the FPEH and MEH to inform the presence of a piggybacked payload.

    - ARQ feedback polling request (section 15.2.1.3.2.1.1 and 15.2.1.3.2.3.2)

In current IEEE 802.16m SDD [2], the ARQ feedback can be transmitted from the receiver when the ARQ feedback polling request is received from the transmitter or an ARQ block has been missing for a predetermined period. However, the detailed method for ARQ feedback polling request is not yet discussed in IEEE 802.16m. Therefore, we propose to define the ARQ feedback polling request bit in FPEH and MEH only for ARQ enabled connection.

    - Bandwidth stealing for PA and GRA region (section 15.2.1.3.2.3.3)

In current IEEE 802.16m SDD [2], Compact Header is defined for the connections with persistent allocation or group resource allocation to reduce the MAC header overhead. AMS/ABS should be able to transmit any MPDUs via the allocated resource region regardless of the specific resource allocation method (e.g. PA or GRA). So, we propose the bandwidth stealing extended header which is used to transmit other MPDUs which don’t belong to the connection with PA or GRA via the resource region which is allocated by PA or GRA. When EH field is set to 1 in compact header and a bandwidth stealing is needed at the resource region where the compact header is used, the bandwidth stealing extended header follows the compact header. The BS EH indicating the GMH does not include an EH field and a length field because the compact header contains the length field and EH field.

# References

- IEEE 802.16m-07/002r4, “802.16m System Requirements.”

- IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document.”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment.”

# Text proposal for the IEEE 802.16m amendment

----------------------------------------------------- Start of the text ----------------------------------------------------------

## Advanced Air Interface

## Medium Access Control

## Data/Control Plane

## MAC PDU formats

MAC PDUs shall be of the form illustrated in Figure xx-1. Each MAC PDU shall contain a MAC header and the header may be followed by the payload of the MAC PDU. The header may be followed by the payload of the MAC PDU. If present, the payload shall consist of zero or more extended headers and zero or more MAC SDUs and/or MAC SDU fragments thereof. The payload information may vary in length, so that a MAC PDU may represent a variable number of bytes. This allows the MAC to tunnel various higher layer traffic types without knowledge of the formats or bit patterns of those messages. Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS may be multiplexed into a single MAC PDU. All reserved fields shall be set to zero on transmission and ignored on reception.

Figure xx-1. MAC PDU format

## MAC header formats

## Generic MAC Header (GMH)

The generic MAC header is illustrated in Figure xx-2. Every header is encoded with the 4-bit Flow ID field. Two Flow IDs are reserved for MAC management messages. MAC management or CS data shall be transmitted with generic MAC header. One or more extended headers (described in section 15.2.1.x.2) may follow along after GMH depending on the value of EH field.

Figure xx-2. Generic MAC Header format

The fields of the generic MAC header are defined in Table xx-1.

Table xx-1. Generic MAC header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| FlowID | 4 | Flow Identifier. This field indicates the service flow that is addressed. This field is 4 bits long. |
| EH | 1 | Extended Header Presence Indicator. When set to ‘1”, this field indicates that an Extended Header is present following this GMH. |
| Length | 11 | Length of the payload. This field is 11 bits long. |

## Compact MAC Header (CMH)

The data for persistent allocation and group allocation shall be transmitted with compact MAC header. The compact MAC header is illustrated in Figure xx-3.

Figure xx-3. Compact MAC Header format

The fields of the compact MAC header are defined in Table xx-2.

Table xx-2. Compact MAC header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| EH | 1 | Extended Header Presence Indicator. When set to ‘1”, this field indicates that an Extended Header is present following this GMH. |
| Length | 7 | Length of the payload. This field is 7bits long. |

## Extended header formats

The inclusion of extended header is indicated by EH indicator bit in generic MAC header. The extended header format is shown in Figure xx-4.

Figure xx-4. Extended Header format

The fields of the extended header are defined in Table xx-3. The per-PDU extended headers may be inserted in MPDUs immediately following the generic MAC header. The fragmentation and packing extended header

Table xx-3. Extended Header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | When the “Last” bit is set, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header. |
| Type | 5 | This field indicates the type of extended header. 0b00000: Fragmentation Extended Header (FEH) 0b00001: Multiplexing Extended Header (MEH) 0b00010: Bandwidth Stealing Extended Header (BSEH) 0b00011 ~ 0b11111: TBD |
| Body Contents | Variable size | Type-dependent contents. |

## Fragmentation and Packing Extended Header for transport connection (FPEH)

Fragmentation and packing extended header is shown in Figure xx-5. This header shall be used when MAC PDU contains single transport connection payload. This header shall be located after the last extended header (i.e. extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’.

Figure xx-5. Fragmentation and Packing Extended Header format

The fields of the fragmentation and packing extended header are defined in Table xx-4.

Table xx-4. Fragmentation and Packing Extended Header Fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control bits definition is given in table xx-5. |
| AFI | 1 | ARQ feedback IE Indication. If a packed ARQ feedback payload presents within this MAC PDU, this field is set to ‘1’ and the ARQ feedback payload shall be transported as the first packed payload. |
| End | 1 | If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment. |
| Length | 11 | This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header. |
| Reserved bit |  | Reserved bits for byte alignment. |

Table xx-5. Fragmentation control information

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

## Fragmentation and Packing Extended Header for ARQ-enabled connection

The FPEH for ARQ-enabled connection shall contain the ARQ Polling Request field, as shown in Figure xx-6. This field is used to request the receiver to send the ARQ Feedback for received blocks.

Figure xx-6. Fragmentation and Packing Extended Header format

The fields of the fragmentation and packing extended header for ARQ-enabled connection are defined in Table xx-6.

Table xx-6. Fragmentation and Packing Extended Header fields for ARQ-enabled connection

| Name | Length (bit) | Description |
| --- | --- | --- |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control bits definition is given in table yyy. |
| AFI | 1 | ARQ feedback IE Indication. If a packed ARQ feedback payload presents within this MAC PDU, this field is set to ‘1’ and the ARQ feedback payload shall be transported as the first packed payload. |
| APR | 1 | ARQ feedback Polling Request. If this bit set to ‘1’, the receiver shall transmit the ARQ feedback IE for this connection to transmitter. |
| End | 1 | If this bit set to ‘0’, another ‘Length’ and ‘End’ field are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment. |
| Length | 11 | This field represents the length of SDU/SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 length fields are present in the header. |
| Reserved bit |  | Reserved bits for byte alignment. |

## Fragmentation Extended Header for management connection (FEH)

This fragmentation extended header is shown in Figure xx-7. This header shall be used when MAC PDU contains single management message payload.

Figure xx-7. Fragmentation Extended Header format

The fields of the fragmentation extended header are defined in Table xx-7.

Table xx-7. Fragmentation Extended Header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | When the “Last” bit is set, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header. |
| Type | 5 | Extended header type field. Shall be set to ‘0b00000’. |
| SN | 8 | Payload sequence number. |
| FC | 2 | Fragmentation control bits definition is given in table yyy. |

## Multiplexing Extended Header (MEH)

Multiplexing Extended Header (MEH) is used when SDUs or SDU fragments from different connections are included in the same MPDU.

As shown in the figure xx-8, Multiplexing Extended Header contains multiple Multiplexing Extended Header Blocks (MEHBs). The SDUs or SDU fragments belonging to the same connection are packed together and the information related to these SDUs or SDU fragments is included in one MEHB. The M bit in MEHB indicates if there is more MEHB followed. If the SDUs or SDU fragment(s) included belong to the same connection, only one MEHB is present.

Figure xx-8. Multiplexing Extended Header format

The fields of the multiplexing extended header are defined in Table xx-8.

Table xx-8. Multiplexing Extended Header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | This field always sets to ‘1’. |
| Type | 4 | Extended header type field. Shall be set to ‘0b00001’. |
| AFI | 1 | ARQ feedback IE Indication. If a packed ARQ feedback payload presents within this MAC PDU, this field is set to ‘1’ and the ARQ feedback payload shall be transported as the first packed payload. |
| MEHB | variable | Multiplexing Extended Header Blocks (described in section 15.2.1.x.2.3.1) |

## Multiplexing Extended Header Block (MEHB)

The format of MEHB is shown in Figure xx-9, except the first MEHB. The first MEHB doesn’t contain the Flow ID and the length for the first SDU or SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU or SDU fragment associated with the first MEHB.

Figure xx-9. Multiplexing Extended Header Block format

The fields of the multiplexing extended header are defined in Table xx-9.

Table xx-9. Multiplexing Extended Header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| M | 1 | This field indicates if there is more MEHB follows. |
| Flow ID | 4 | Flow id of the SDUs/SDU fragments identified in the MEHB. |
| FC | 2 | Multiplexing Control Information (as shown in Table yyy). |
| SN | 10 | ARQ BSN for ARQ enabled connection or Fragment SN for non-ARQ enabled connection. |
| L | 1 | This field indicates if there is more length field follows. |
| Length | 11 | Length for each SDU identified in the MEHB |
| Reserved |  | Reserved bits for byte alignment |

## Multiplexing Extended Header Block for ARQ-enabled connection

The MEH for ARQ-enabled connection shall contain the ARQ Polling Request field, as shown in Figure xx-10. This field is used to request the receiver to send the ARQ Feedback for received blocks.

Figure xx-10. Multiplexing Extended Header Block format

The fields of the multiplexing extended header for ARQ-enabled connection are defined in Table xx-10.

Table xx-10. Multiplexing Extended Header fields for ARQ-enabled connection

| Name | Length (bit) | Description |
| --- | --- | --- |
| M | 1 | This field indicates if there is more MEHB follows. |
| Flow ID | 4 | Flow id of the SDUs/SDU fragments identified in the MEHB. |
| FC | 2 | Multiplexing Control Information (as shown in Table yyy). |
| SN | 10 | ARQ BSN for ARQ enabled connection or Fragment SN for non-ARQ enabled connection. |
| APR | 1 | ARQ feedback Polling Request. If this bit set to ‘1’, the receiver shall transmit the ARQ feedback IE for this connection to transmitter. |
| L | 1 | This field indicates if there is more length field follows. |
| Length | 11 | Length for each SDU identified in the MEHB |
| Reserved |  | Reserved bits for byte alignment |

## Bandwidth Stealing Extended Header (BSEH)

The Bandwidth stealing extended header (BSEH) is used to transmit other MPDUs which don’t belong to the connection with persistent allocation or group resource allocation through the resource region which is allocated by PA or GRA. When a bandwidth stealing is needed at the resource region where the compact header is used, EH field in CMH shall be set to ‘1’ and this BSEH follows the CMH. The format of BSEH is as shown in Figure xx-11.

Figure xx-11. Bandwidth Stealing Extended Header format

The fields of the bandwidth stealing extended header are defined in Table xx-11.

Table xx-11. Bandwidth Stealing Extended Header fields

| Name | Length (bit) | Description |
| --- | --- | --- |
| Last | 1 | When the “Last” bit is set, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header. |
| Type | 5 | Extended header type field. Shall be set to ‘0b00010’. |
| Flow ID | 4 | Flow Identifier. This field indicates the service flow that is addressed. This field is 4 bits long. |

---------------------------------------------------------- End of the text --------------------------------------------------------------