| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Fast Feedback Channel Allocation A-MAP Information Elements for the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | IEEE 80216m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | The contribution proposes text for the FBCH allocation A-MAP IEs in 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Fast Feedback Channel Allocation A-MAP Information Elements for the IEEE 802.16m AWD

Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides amendment text for fast feedback channel allocation A-MAP IEs in subclause 15.3.6.4.2 in the 802.16m Amendment Working Document [1].

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Proposed text for inclusion in the IEEE 802.16m AWD

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.6.4.2.9 Fast feedback channel allocation A-MAP IE

FBCH-allocation-A-MAP IE is introduced to dynamically allocating or de-allocating UL fast feedback control channels, PFBCH and SFBCH, to an AMS. The format of the IE is shown in Table 667.

Table 667- FBCH allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| FBCH-allocation-A-MAP_IE() { |  |  |
| A-MAP IE Type | 4 | FBCH allocation A-MAP IE |
| MFM | 3 | According to MFM, feedback contents are decided. 0b000: MFM0 ~ 0b110: MFM6 0b111: MFM7 |
| Allocation index | 6 | Index to feedback channel region |
| Short period (p1) | 2 | FBCH with short period is transmitted every 2^p1 frames |
| Long period (p2) | 2 | FBCH with long period is transmitted every 2^(p2+1) FBCHs of short period |
| Duration (d) | 3 | Feedback is transmitted for 10x2^d frames |
| Frame offset | 2 |  |
| Max Mt | 2 | 0b00: 1 stream 0b01: 2 streams For MFM == 5,6,7 0b10: 3 streams 0b11: 4 streams For other MFMs 0b10: 4 streams 0b11: 8 streams |
| If (MFM == 1) { |  |  |
| DLRU[TBD] | [1] | Indicating DRU or CRU 0b0: DRU 0b1: CRU |
| } |  |  |
| If (MFM == 2,3,5,6) { |  | For localized RU |
| DLRU_flag | 1 | Indicating use of DLRU field |
| If (DLRU_flag == 0) |  |  |
| Best_M [TBD] | [3] |  |
| } |  |  |
| Else { |  |  |
| DLRU[TBD] | [4] | Downlink RU, indicating which Rus to work on for feedback |
| } |  |  |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | For adaptive precoding modes |
| CM | 2 | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: adaptive with CCE disabled 0b10: differential with CCE disabled 0b11: standard with CCE enabled |
| PMI_type | 1 | (For MFM 2 and 3, 0b0 is set) 0b0: wideband PMI 0b1: subband PMI |
| } |  |  |
| reserved | TBD |  |
| Padding | Variable |  |
| MCRC | 16 | 16 bit CRC masked by Station ID |
| } |  |  |

Best M: This field specifies the number of subband to be reported.

CM: This field specifies codebook feedback mode and codebook coordination in adaptive precoder mode.

PMI type: This field specifies the type of PMI, wideband PMI and subband PMI.

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------