| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Uplink Control Information Contents for the IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | IEEE 80216m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | The contribution proposes the text of UL control information contents in the UL Control section of the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Uplink Control Information Contents for the IEEE 802.16m AWD

Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes amendment text of UL control for the feedback information contents to be included in subclause 15.3.9.3.1 of the IEEE 802.16m Amendment Working Document Error: Reference source not found.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Proposed text for inclusion in the IEEE 802.16m AWD

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3 Uplink control information content

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified into the following categories:

1) Channel quality feedback

2) MIMO feedback

3) HARQ feedback (ACK/NACK)

4) Uplink synchronization signals

5) Bandwidth requests

6) E-MBS feedback

# 15.3.9.3.1 Fast feedback control channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback, and MIMO feedback, and other event-driven messages. The information contents carried in the PFBCH support MFM 0, 1, 2, 3, 4 and 7, and two contents may be carried by single PFBCH with different periods, period_1 and period_2. The PFBCH for period_2 punctures one for period_1, and period_1 and period_2 are given by FBCH allocation IE. For MFM 0 and 1, wideband CQI and rank information are carried over single PFBCH with period_1 and period_2, respectively. For MFM 2, PFBCH is used for M=1 and carries subband CQI and subband indication. The support of PFBCH for MFM 3 (M=1) is TBD. For MFM 4 and 7, wideband CQI and wideband PMI are carried in the PFBCH. Table UL_1 describes the information contents to be carried over PFBCH. The PFBCH will carry the sequences corresponding to the event-driven messages, such as BW request indicator, the preferred MIMO mode, and etc. on-demand by puncturing the CQI or rank information. Detail mapping contents to sequences of PFBCH is defined in Table UL_2[TBD]. Mapping PMI index to sequences of PFBCH is TBD. In section 15.3.9.3.1.3, mapping contents to PFBCH corresponding to MFM is described in detail.

Table UL_1-Feedback Contents of PFBCH

| PFBCH Feedback Content | Supported MIMO Feedback Mode | Period |
| --- | --- | --- |
| Wideband CQI | MFM 0, 1, 4, 7 | Period 1 |
| Wideband PMI | MFM 4, 7 | Period 2 |
| Rank Indicator | MFM 0, 1, 2, 3, 4, 7 | Period 2 |
| Subband CQI | MFM 2, 3 | Period 1 |
| Subband PMI [TBD] | MFM 3 | Period 1 |
| Preferred MIMO mode indicator |  | Event-driven |
| Multi-cell MIMO mode indicator |  | Event-driven |
| Bandwidth Request Indicator |  | Event-driven |

Table UL_2-Sequence allocation in PFBCH

| Sequence Value | Content | Description |
| --- | --- | --- |
| 0 | CQI | N codewords are used for CQI. N is TBD |
| 1 | CQI |  |
| … | … |  |
| N-1 | CQI |  |
| N | SFBC or rank 1 | To indicate SFBC or rank 1 according to MFM |
| N+1 | Rank 2 |  |
| N+2 | Rank 3 |  |
| … |  |  |
| N+7 | Rank 8 |  |
| … | (reserved) |  |
| 53 | (reserved) |  |
| 54 | SU-OL in diversity | Preferred MIMO mode |
| 55 | SU-CL in diversity |  |
| 56 | SU-OL in localized |  |
| 57 | SU-CL in localized |  |
| 58 | Recommendation set indication [TBD] | To indicate the necessity of multi-cell MIMO mode |
| 59 | Restriction set indication [TBD] |  |
| 60 | Multi-cell MIMO indication [TBD] |  |
| 61 | Multi-cell MIMO indication [TBD] |  |
| 62 | BW Request Indicator | To request UL Bandwidth |
| 63 | BW Request Indicator |  |

# 15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive. Support for event-driven messages is TBD. The information contents carried in the SFBCH are related with MFM 2, 3, 5, and 6, and single SFBCH carries two groups of contents with different periods, period_1 and period_2. Each group consists of 1~3 information contents. The SFBCH for period_2 punctures one for period_1, and period_1 and period_2 are given by FBCH allocation IE. Table UL_3 describes the information contents to be carried in the SFBCH. In section 15.3.9.3.1.3, mapping contents to SFBCH corresponding to MFM is described in detail.

Table UL_3-Feedback Contents of SFBCH

| SFBCH Feedback Content | Supported MIMO Feedback Mode | Period |
| --- | --- | --- |
| Subband indication | MFM 2, 3, 5, 6 | Period 2 |
| Rank Indicator | MFM 2, 3 | Period 2 |
| Subband CQI | MFM 2, 3, 5, 6 | Period 1 or Period 2 |
| Subband PMI | MFM 3, 6 | Period 1 |
| Stream Indicator | MFM 5 | Period 1 or Period 2 |
| Event-driven messages [TBD] |  | Event-driven |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------