| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Mapping Contents to Fast Feedback Channel for IEEE 802.16m AWD |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | IEEE 80216m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | The contribution proposes text of UL-control for mapping contents to fast feedback channel in the UL control section of the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Mapping Contents to Fast Feedback Channel for IEEE 802.16m AWD

Sangheon Kim, David Mazzarese, Hwasun Yoo, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes amendment text of UL control for mapping the feedback contents to fast feedback channel in subclause 15.3.9.3.1 of the IEEE 802.16m Amendment Working Document Error: Reference source not found.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Proposed text for inclusion in the IEEE 802.16m AWD

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.3 Mapping contents to fast feedback control channel

Mapping contents to fast feedback control channel is controlled by MFM, type of PMI, and the number of best M subbands, which are given in FBCH allocation IE. Table xx1, Table xx2, and Table xx3 describe mapping for MFM 0~ 2(OL-SU-MIMO), for MFM 3 and 4(CL-SU-MIMO), and for MFM 5~7(MU-MIMO), respectively.

Table xx1-Mapping contents for OL-SU-MIMO

| MFM | MIMO mode | Type of FBCH | Contents |  |  |  | Max Bits |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Period 1 (short) |  | Period 2 (long) |  |  |
|  |  |  | Contents | Bitwidth | Contents | Bitwidth |  |
| MFM 0 | OL-SU, diversity | P-FBCH | W-CQI |  | MEF/rank |  |  |
| MFM 1 | OL-SU, diversity | P-FBCH | W-CQI |  | rank |  |  |
| MFM 2 | OL-SU, Best M=1 | P-FBCH | Subband CQI |  | Subband indication |  |  |
|  | OL-SU, Best M=2 | S-FBCH | Subband CQI 2x4 = 8 bits | 2x4 = 8 | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M=3 | S-FBCH | Avg CQI subband diff. CQI | 4 + 3x2 = 10 bits | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M=4 | S-FBCH | Avg CQI subband diff. CQI | 4 + 4x2 = 12 bits | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M=5 | S-FBCH | Avg CQI subband diff. CQI | 4 + 5x2 = 14 bits | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M=6 | S-FBCH | Avg CQI subband diff. CQI | 4 + 6x2 = 16 bits | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M=7 | S-FBCH | Avg CQI subband diff. CQI | 4 + 7x2 = 18 bits | Subband indication rank | 12 + 1~3 = 13~15 bits |  |
|  | OL-SU, Best M≥8 | S-FBCH | subband diff. CQI | 2xM bits | Subband indication rank avg CQI | 12 +1~3 + 4 = 17~19 bits |  |

Table xx2- Mapping contents for CL-SU-MIMO

| MFM | Type of PMI | MIMO mode | Type of FBCH | Contents |  |  |  | Max Bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Period 1 (short) |  | Period 2 (long) |  |  |
|  |  |  |  | Contents | Bitwidth | Contents | Bitwidth |  |
| MFM 3 | Subband PMI | CL-SU, Best M=1 | P-FBCH | Subband CQI Subband PMI |  | Subband indication |  |  |
|  |  | CL-SU, Best M=2 | S-FBCH | Subband diff. CQI Subband PMI | 2x(2+3~6) = 10~18 bits | Subband indication rank, avg CQI | 12+1~3+4= 17~19 bits |  |
|  |  | CL-SU, Best M=3 | S-FBCH | Subband diff. CQI Subband PMI | 3x(2+3~6) = 15~24 bits | Subband indication rank avg CQI | 12+1~3+4= 17~19 bits |  |
|  |  | CL-SU, Best M=4 | S-FBCH | Subband diff. CQI Subband PMI | 4x(2+3~4) = 20~24 bits | Subband ind, rank, avg CQI 12 + 3 + 4= 19 bits | 12+1~3+4= 17~19 bits |  |
|  | Wideband PMI | CL-SU, Best M=2 | S-FBCH | Subband CQI Wideband PMI | 2x4 + 3~6 = 11~14 bits | Subband indication 12 bits | 12 bits |  |
|  |  | CL-SU, Best M=3 | S-FBCH | Avg CQI, diff. CQI Wideband PMI | 4+ 3x2 + 3~6 = 13~16 bits | Subband indication 12 bits | 12 bits |  |
|  |  | CL-SU, Best M=4 | S-FBCH | Subband diff. CQI Wideband PMI | 4x2 + 3~6 = 11~14 bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
|  |  | CL-SU, Best M=5 | S-FBCH | Subband diff. CQI Wideband PMI | 5x2 +3~6 = 13~16 bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
|  |  | CL-SU, Best M≥6 | S-FBCH | Subband diff. CQI Wideband PMI | 2M + 3~6bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
| MFM 4 | Wideband PMI | CL-SU, diversity | P-FBCH | Wideband CQI |  | Wideband PMI |  |  |

Table xx3- Mapping contents for MU-MIMO

| MFM | Type of PMI | MIMO mode | Type of FBCH | Contents |  |  |  | Max Bits |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Period 1 (short) |  | Period 2 (long) |  |  |
|  |  |  |  | Contents | Bitwidth | Contents | Bitwidth |  |
| MFM 5 | n.a. | OL-MU | S-FBCH | TBD |  | TBD |  |  |
| MFM 6 | Subband PMI | CL-MU, Best M=2 | S-FBCH | Subband diff. CQI Subband PMI | 2x(2+3~6) = 10~18 bits | Subband indication avg CQI | 12+4= 16 bits |  |
|  |  | CL-MU, Best M=3 | S-FBCH | Subband diff. CQI Subband PMI | 3x(2+3~6) = 15~24 bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
|  |  | CL-MU, Best M=4 | S-FBCH | Subband diff. CQI Subband PMI | 4x(2+3~4) = 20~24 bits | Subband ind, rank, avg CQI | 12 + 4= 16 bits |  |
|  | Wideband PMI | CL-MU, Best M=2 | S-FBCH | Subband CQI Wideband PMI | 2x4 + 3~6 = 11~14 bits | Subband indication 12 bits | 12 bits |  |
|  |  | CL-MU, Best M=3 | S-FBCH | Avg CQI diff. CQI Wideband PMI | 4+ 3x2 + 3~6 = 13~16 bits | Subband indication 12 bits | 12 bits |  |
|  |  | CL-MU, Best M=4 | S-FBCH | Subband diff. CQI Wideband PMI | 4x2 + 3~6 = 11~14 bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
|  |  | CL-MU, Best M=5 | S-FBCH | Subband diff. CQI Wideband PMI | 5x2 + 3~6 = 13~16 bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
|  |  | CL-MU, Best M≥6 | S-FBCH | Subband diff. CQI Wideband PMI | 2M + 3~6bits | Subband indication avg CQI | 12 + 4= 16 bits |  |
| MFM 7 | Wideband PMI | CL-MU, diversity | P-FBCH | Wideband CQI |  | Wideband PMI |  |  |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------