| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on MAC PDU Formats |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Dongyao Wang, Kaibin Zhang, Gang Shen, Shan Jin Alcatel-Lucent Shanghai Bell | Dongyao.wang@alcatel-sbell.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0020: Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content: 15.2.x MAC PDU Formats |  |
| Abstract | Proposed Contents for the P802.16m Amendment on the MAC PDU Formats |  |
| Purpose | For review, discussion and adopt in the Project 802.16m AWD document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on MAC PDU Formats

Dongyao Wang, Kaibin Zhang, Gang Shen, Shan Jin

Alcatel-Lucent Shanghai Bell

Introduction

This contribution provides amendment text for MAC PDU formats in the Advanced Air Interface to be included in Section 15.2 of the IEEE 802.16m Amendment Working Document [1]. The text was developed with the baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2].

In addition, since the extended header plays an important role for the MAC PDU processing in 16m SDD, it is required to determine the type of extended header as early as possible. And when defining the type field of extended header, enough length should be reserved to support different purposes for high system efficiency while keeping reasonable overhead. And in this contribution, we choose 5bit long for this type field.

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-09/0010r1a.

- IEEE 802.16m System Description Document (SDD), IEEE 802.16m-08/003r8.

Proposed Amendment Text

The following text is proposed for inclusion in Section 15.2 of the IEEE 802.16m Amendment Working Document [1].

# 15.2.x MAC PDU Formats

MAC PDUs shall be of the form illustrated in Figure xx. Each MAC PDU contains a MAC header. And this header may be generic MAC header or compact header. And this header may be followed by one or more extended headers. Also the MAC PDU may contain payload. If present, the payload shall consist of zero or more MAC SDUs and/or fragments thereof. Multiple MAC SDUs and/or SDU fragments from different unicast connections belonging to the same AMS can be multiplexed into a single MAC PDU. The MAC PDU may vary in length, and shall be byte aligned.

Figure xx MAC PDU formats

# 15.2.x.1 MAC header formats

#### 15.2.x.1.1 Generic MAC Header

The generic MAC header is illustrate in Figure xx.

Figure xx Generic MAC header format

- FlowID (Flow Identifier): This field indicates the service flow that is addressed.. This field is 4bits long.

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this GMH.

- Length: Length of the payload. This field is 11bits long

#### 15.2.x.1.2 Compact header

Compact header is used for connections with persistent allocation and group allocation.

Figure xx Compact header format

- EH (Extended Header Presence Indicator): When set to ‘1”, this field indicates that an Extended Header is present following this header.

- Length (7): Length of the payload.

#### 15.2.x.1.3 Extended header

The inclusion of extended header is indicated by EH indicator bit in MAC Header. The EH format is shown in xx and will be used unless specified otherwise.

Figure xx Extended Header Format

- Last: When the “Last” bit is set, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header.

- Type: indicates the type of extended header. The length is 5 bits long and Table xx describes the encodings of the 5-bit Type field.

- Body Contents: Type-dependent contents.

Table xx  Type field encodings for extended header

| Type field (5 bits) | Extended header type | Notes |
| --- | --- | --- |
| 00000 | Fragmentation and packing | for transport connection |
| 00001 | Fragmentation | for management connection |
| 00010 | Multiplexing Extended Header | Multiplexing SDUs or SDUs fragments from different connections |
| 00011 | Standalone bandwidth request | Incremental bandwidth request |
| 00100 | Standalone bandwidth request | aggregate bandwidth request |
| 00101 | ARQ feedback | used by the receiver to signal positive or negative acknowledgments |
| 00110~11111 | Reserved | Others for future definition |