| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for DL A-MAP IE for MU-MIMO mode in IEEE802.16m Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Takaaki KISHIGAMI, Isamu YOSHII Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: kishigami.takaaki@jp.panasonic.com yoshii.isamu@jp.panasonic.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7.2.5.2 Unicast information” |  |
| Abstract | We propose the DL A-MAP IE for MU-MIMO mode. |  |
| Purpose | For 802.16m discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for DL A-MAP IE for MU-MIMO mode in IEEE802.16m Amendment

Takaaki KISHIGAMI, Isamu YOSHII

Panasonic Corporation

# Introduction

The unicast information contents for MIMO modes are not defined clearly in current AWD [1].

In this contribution, we propose DL A-MAP IE for MU-MIMO mode based on IEEE802.16 Rev2/D8 [2].

# References

[1]IEEE 802.16m-09/0010r1a, 802.16m Amendment Working Document (AWD)”

[2]IEEE 802.16Rev2/D8

# Proposed Text

We propose the following text for the IEEE802.16m Amendment Working Document in Section 15.3.7.2.5.2

-------------------------------  Start text proposal  ---------------------------------------------------

- 15.3.7.2.5.2 Unicast information

Some parameters necessary for DL MIMO operation shall be unicast by the BS to a specific MS. The unicast

information is carried by A-MAP IEs or feedback allocation IEs

For DL MU-MIMO mode, A-MAP IE is specified in Table x.

Table x –DL basic assignment A-MAP for DL MU-MIMO information contents

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| DL basic assignment A-MAP IE{ |  |  |
| A-MAP IE Type | [4] | Type for DL MU-MIMO A-MAP IE |
| MM | [4] | MIMO Mode: TBD Additional modes may be specified in extended IEs. |
| Resource Allocation | Variable | Determines the start, length and number of subframes spanned the allocation. |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| Boosting | [3] | 0b000: Normal (not boosted) 0b001: +6dB; 0b010: –6dB 0b011: +9dB; 0b100: +3dB 0b101: –3dB; 0b110: –9dB 0b111: –12dB |
| Number of users | 2 | The number of MU-MIMO multiusers ( upto 4 users) [The number of streams in transmission] |
| For (jj = 1: Num_Users) | - | - |
| RCID_IE( ) | variable | Specifies the station ID in RCID format |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| Stream Index | 2 | Index of pilot stream allocation 0b00:1 0b01:2 0b10:3 0b11:4 |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } | - | - |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit masked CRC [TBD] |
| } | - | - |

MCRC: MCRC may be used by the Multiuser MIMO group ID which assign MAC management message or Group Configuration A-MAP IE.

-------------------------------  End text proposal  ---------------------------------------------------