| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Wideband Beamforming in the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Wideband Beamforming in the IEEE 802.16m Amendment

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7. The proposal resolves the “TBD” for long-term PMI report for the serving cell in Table 690 page 239 in [1]. It is shown that the feedback of a wideband PMI is a more economical way to perform long-term wideband beamforming than using the feedback of the quantized correlation matrix.

As such, we recommend to allow the feedback of a wideband PMI to ensure performance at the cell edge, while keeping the feedback of the quantized correlation matrix for the main purpose of adaptive codebook feedback mode, and allowing the base station to use the feedback of the quantized correlation matrix for wideband beamforming when supported by the uplink SNR.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Proposal for wideband beamforming

The support of wideband beamforming is mandated in the SDD [3] (closed-loop single-user and multi-user MIMO in distributed allocations). It is a recognized technique to achieve downlink beamforming gains in highly correlated channels, for both fixed and mobile users. The technique relies on the fact that the transmit spatial correlation changes slowly over time, and is approximately constant over the bandwidth.

Wideband beamforming is implemented by applying a single rank-1 precoder over all the distributed subcarriers in a diversity allocation. Usually, that rank-1 precoder is only changed over a long period of time (several frames).

There are two feedback methods for supporting wideband beamforming in the AWD:

- Long-term wideband quantized correlation matrix

- Rank-1 wideband PMI from the base codebook or a subset of the base codebook

Table 1 summarizes the characteristics of these two feedback methods and their usage for wideband beamforming.

Table 1 – Wideband correlation matrix vs. wideband PMI feedback

|  | Long-term wideband quantized correlation matrix | Rank-1 wideband PMI |
| --- | --- | --- |
| Estimation at MS | Over several midambles (e.g. 16 midambles every 80 ms) | Over 1 midamble Over several midambles is also possible |
| Feedback period | Several frames (e.g. 16 frames) | Several frames (e.g. 16 frames) |
| Transmission method | Keep the same rank-1 beamformer for several frames | Keep the same rank-1 beamformer for several frames |
| DL control signaling for allocation uplink feedback channel | Polling_IE FBCH_Alloc_IE | FBCH_Alloc_IE |
| Uplink feedback channel type | - One MAC management message (feedback header) with long period - One P-FBCH (CQI with short period) | One P-FBCH - PMI (long period, puncturing short period) - CQI (short period) |
| Payload size (bits) | Matrix: 6 (2Tx), 28 (4Tx), 120 (8Tx) | PMI: 3 (2Tx), 4 or 6 (4Tx), 3 (8Tx) |
| Uplink feedback channel size | 3 or 6 UL tiles every frame (5 ms) (packet fragmentation could be used for 8Tx with a transmission over 3 UL subframes with 3 tiles in each subframe) | 1 UL tile every frame (5 ms) |
| Robustness at cell-edge | Link budget may be tight for large payload, it may require packet fragmentation. Otherwise | No problem, link budget of P-FBCH is optimized for the cell-edge |
| Computational complexity at MS | Estimate the wideband correlation matrix, quantization and SVD of RT Cf Table 3 | Find the best PMI over the whole band, or computed it from the estimated wideband correlation matrix Cf Table 3 |
| Computational complexity at BS | SVD of RT | No additional computation required |
| BS-MS interoperability | BS and MS could do different things on matrix RT, depending on implementation, especially in CL MU MIMO. | Yes, since the rank-1 PMI that is feedback cannot be further transformed |
| Quantization effect | RT is quantized. Matrix quantization to convey the information of 1 vector is unnecessary. | PMI is quantized. The base codebooks have been optimized to quantize a correlated channel vector with a small error. |

Feedback overhead

A wideband PMI with a maximum size of 6 bits can be effectively transmitted in a P-FBCH, by time multiplexing with CQI. The long-period feedback of PMI punctures the short-period feedback of CQI. One P-FBCH takes up 6 subcarriers in one uplink subframe (i.e. 1 UL tile). A wideband PMI with 4 bits could also be used without much performance loss for cell-edge, due to the limitation in the accuracy of the channel estimation at low SNR. The P-FBCH requires an UL SNR of about -1.5 dB for achieving a PER of 1%.

The quantized correlation matrix is fed back to the BS via a MAC management message, i.e. using an extended header. As such, it requires a 2-byte generic MAC header and a 2-byte CRC, independently of the payload size. An additional 2-byte fragmentation and packing header is required in case of packet fragmentation, which may be required if the uplink SNR doesn’t meet the link budget of the feedback channel, so that the MS can multiplex the payload over several uplink subframes. Without fragmentation, it may be difficult to guarantee the successful transmission of the quantized correlation matrix by cell-edge users.

Assuming a robust MCS (QPSK rate 71/256 of Table 744 in [1]) for the transmission of the extended header, the actual header bit count and physical feedback resource size are summarized in table 2. The minimum data allocation on uplink in DRU is one LRU (three 6x6 tiles), which can carry 96 symbols once the pilot overhead has been accounted for.

Table 2 – Overhead of Feedback Header

|  | 2Tx | 4Tx | 8Tx |
| --- | --- | --- | --- |
| Bit count | MAC header: 2 bytes CRC: 2 bytes Payload: 6 bits Total: 38 bit | MAC header: 2 bytes CRC: 2 bytes Payload: 28 bits Total: 60 bit | MAC header: 2 bytes CRC: 2 bytes Payload: 120 bits Total: 152 bit |
| Encoded bits | 95.3239 bits | 174.6479 | 506.3662 |
| Byte alignment | 96 bits | 176 bits | 512 bits |
| Number of QPSK symbols | 48 | 88 | 256 |
| Size of the allocated feedback uplink resource (6 symbols UL subframe with QPSK rate 1/2) | 1 LRU = 3 UL tiles | 1 LRU = 3 UL tiles | 3 LRUs = 9 UL tiles |

This is to be compared with 1 UL tile for the feedback of the wideband PMI using one P-FBCH.

A minimum of 3 UL tiles are required for the feedback header in the case of 4 transmit antennas at the BS, so the power spectral density per subcarrier of the uplink transmission is decreased, and the UL SNR is decreased by 4.77 dB. Therefore the feedback reliability decreases for cell-edge users. For 8Tx, packet fragmentation could be used to transmit the information over multiple UL subframes.

Computational complexity at MS

It is assumed that the MS can obtain a channel estimate Hk on every subcarrier k from the midamble, and will use these estimates as a basis for computing the wideband correlation matrix or for finding the best wideband PMI.

The wideband PMI is found by maximizing  over all rank-1 codewords with index i in the base codebook of rank 1. In general, the channel matrix on subcarrier k can be expressed as of size, where  is the transmit side correlation matrix of size, where the BS transmitter is equipped with n antennas and the MS receiver is equipped with m antennas. Finally the optimal precoder from the codebook (with L precoders) is expressed as:

With the assumptions of stationarity and ergodicity of the fading process:

.

The estimate of the wideband correlation matrix is precisely obtained as]

.

It can be further averaged over several midambles (i.e. several frames).

The well-know solution to the problem  is the principal eigenvector of.

So  can be approximated by finding the closest PMI to the principal eigenvector of, but it requires to perform a singular value decomposition (SVD) of, then a quantization to the closest vector in the codebook.

Alternatively,can be found directly as

,

and the CQI is obtained as

,

where  and  are the AWGN and interference powers, respectively.

Table 2 – Computational Complexity

| Wideband PMI (approach 1) |  | Wideband PMI from correlation matrix (approach 2) |  | Eigenvector from correlation matrix (approach 3) |  |
| --- | --- | --- | --- | --- | --- |
| operation | Complexity | operation | complexity | operation | complexity |
|  | NFFTLmn multiplications |  |  |  |  |
|  | NFFTLm multiplications |  | NFFTmn2 multiplications |  | NFFTmn2 multiplications |
|  | L sums of NFFT complex numbers |  | n2 sums of NFFT complex numbers |  | n2 sums of NFFT complex numbers |
|  |  |  | 1 division |  | 1 division |
|  |  |  | Ln2 multiplications | SVD of (,) | n3 multiplications |
|  |  |  | Ln multiplications |  |  |
|  | Comparison |  | Comparison |  |  |
| Quantization | Already done | Quantization | Already done | Quantization | Quantize |
| CQI computation | 2 divisions | CQI computation | 1 division | CQI computation | 1 division |
| BS computation | Not required | BS computation | Not required | BS computation SVD of | n3 multiplications |
| Total number of multiplications at MS | NFFTLm(n+1) multiplications |  | n2(NFFTm+L)+Ln multiplications |  | n2(NFFTm+n) multiplications |

The complexity of approach 1 can be higher when a large codebook is used (large L), whereas the complexities of approaches 2 and 3 are of the same order at the MS. The complexity of approach 3 is higher at the BS, and it requires the implementation of an SVD at both the MS and BS.

- Text proposal for inclusion in the 802.16m amendment

<Delete “[TBD]” in Table 690 on page 239 lines 28>

PMI report for serving cell [TBD]