| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for MIMO feedback modes |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for MIMO feedback modes

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7. We propose a definition of MIMO feedback modes in Table 671, along with a description of the MIMO operation for each sub-mode, indicated by DL control parameters and feedback parameters.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2.6.4. MIMO feedback modes” on line 62 page 75 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Each MIMO transmission mode can be supported by one or several MIMO feedback modes. When allocating a feedback channel, the MIMO feedback mode shall be indicated to the MS, and the MS will feedback information accordingly.

The description of MIMO feedback modes and corresponding supported MIMO transmission modes is shown in Table 671. The detailed description of feedback and MS processing are in the following subsections.

Table 671 – MIMO feedback modes

| MIMO feedback mode | Type of MIMO operation | Type of RU (possible allocation) | Supported MIMO transmission mode | Content in fast feedback channels | DL control parameters in FBCH_Alloc_IE |
| --- | --- | --- | --- | --- | --- |
| 0 | OL SU MIMO STBC/SM switching (diversity allocation) | DRU | MIMO mode 0, and MIMO mode 1 (Mt=2) | Wideband CQI STC rate | MaxMt |
|  | OL SU MIMO STBC/SM switching (diversity allocation) | Miniband-based CRU | MIMO mode 0, and MIMO mode 1 (Mt≥2) |  |  |
| 1 | OL SU MIMO SM (diversity allocation) | DRU | MIMO mode 1 (Mt = 2) | Wideband CQI Rank | MaxMt DRU or CRU type of assumed allocation |
|  | OL SU MIMO SM (diversity allocation) | Miniband-based CRU | MIMO mode 1 (8≥Mt≥1) |  |  |
| 2 | OL SU MIMO SM (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 1 (Mt=1) (applicable to cell-edge) | BSI (1 subband) 1 subband CQI | MaxMt Number M of best subbands to feedback |
|  | OL SU MIMO SM (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 1 (not optimized for cell-edge) | BSI (M subbands) M subbands CQI Rank |  |
| 3 | CL SU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 2 (Mt=1) (applicable to cell-edge) | BSI (1 subband) 1 subband CQI 1 subband PMI | CM MaxMt Number M of best subbands to feedback PMI type (wideband or subband) |
|  | CL SU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 2 (Mt=1) (not optimized for cell-edge) | BSI (M subbands) M subbands CQI Wideband PMI |  |
|  | CL SU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 2 (not optimized for cell-edge) | BSI (M subbands) M subbands CQI M subbands PMI Rank |  |
| 4 | CL SU MIMO (diversity allocation) | Mini-band based CRU | MIMO mode 2 (Mt=1) (applicable to cell-edge) | Wideband CQI Wideband PMI | CM |
| 5 | OL MU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 3 (not optimized for cell-edge) | BSI (M subbands) M subbands CQI M streams indication | MaxMt Number M of best subbands to feedback |
| 6 | CL MU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 4 (Mt=1) (not optimized for cell-edge) | BSI (M subbands) M subbands CQI Wideband PMI | CM MaxMt Number M of best subbands to feedback PMI type (wideband or subband) |
|  | CL MU MIMO (localized allocation) | Subband or contiguous mini-bands allocation in CRU | MIMO mode 4 (Mt=1) (not optimized for cell-edge) | BSI (M subbands) M subbands CQI M subbands PMI |  |
| 7 | CL MU MIMO (diversity allocation) | Mini-band based CRU | MIMO mode 4 (Mt=1) (not optimized for cell-edge) | Wideband CQI Wideband PMI | CM MaxMt |

BSI: band selection information.

The feedback of the quantized wideband correlation matrix shall be requested by the BS for operation with adaptive codebook-based feedback mode. The BS may request the feedback of the quantized wideband correlation matrix independently of the MIMO feedback mode requested in the FBCH_Alloc_IE. The quantized wideband correlation matrix may be used for wideband beamforming.

-------------------------------  Text End  ---------------------------------------------------