| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for MIMO feedback modes |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for MIMO feedback modes

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7. We propose a definition of MIMO feedback modes in Table 671, along with a description of the MIMO operation for each sub-mode, indicated by DL control parameters and feedback parameters.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2.6.4. MIMO feedback modes” on line 62 page 75 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Each MIMO transmission mode can be supported by one or several MIMO feedback modes. When allocating a feedback channel, the MIMO feedback mode shall be indicated to the MS, and the MS will feedback information accordingly.

The description of MIMO feedback modes and corresponding supported MIMO transmission modes is shown in Table 671. The detailed description of feedback and MS processing are in the following subsections.

Table 671 – MIMO feedback modes

| Feedback mode | Description | Feedback content | Type of RU | Supported MIMO transmission mode |
| --- | --- | --- | --- | --- |
| Mode 0 | OL SU MIMO SFBC/SM (Diversity) | 1. STC Rate 2. Wideband CQI | Diversity (DRU, Mini-band based CRU) | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate ≥ 2: SM CQI In DRU: Mt=2 for SM. In Miniband based CRU: Mt >=2 for SM |
| Mode 1 | OL SU MIMO SM (Diversity) | 1. STC Rate 2. Wideband CQI | Diversity (Mini-band based CRU) | MIMO mode 1 |
| Mode 2 | OL SU MIMO SM (localized) | 1. STC Rate 2. Subband CQI 3. Subband Selection | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 1 |
| Mode 3 | CL SU MIMO (localized) | 1. STC Rate 2. Subband CQI 3. Subband PMI 4.Subband selection [5. Wideband PMI] 6. Wideband correlation matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 2 |
| Mode 4 | CL SU MIMO (Diversity) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation matrix | Diversity (Mini-band based CRU) | MIMO mode 2 (Mt=1) |
| Mode 5 | OL MU MIMO (localized) | 1. Subband CQI 2. Subband Selection 3. Stream indicator | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 3 |
| Mode 6 | CL MU MIMO (localized) | 1. Subband CQI 2. Subband PMI 3. Subband Selection [4. Wideband PMI] 5. Wideband correlation matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 4 |
| Mode 7 | CL MU MIMO (Diversity) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation matrix | Diversity (Mini-band based CRU) | MIMO mode 4 |

The feedback of the quantized wideband correlation matrix shall be requested by the BS for operation with adaptive codebook-based feedback mode. The BS may request the feedback of the quantized wideband correlation matrix independently of the MIMO feedback mode requested in the FBCH_Alloc_IE. The quantized wideband correlation matrix may be used for wideband beamforming.

            - MIMO feedback 0

MIMO feedback mode 0 is used for the OL-SU SFBC and SM adaptation in diversity permutation. The MS estimates the wideband CQI for both SFBC and SM, and reports the CQI and STC Rate. STC Rate 1 means SFBC with rank-2 SM with precoding.

            - MIMO feedback 1

MIMO feedback mode 1 is used for the OL-SU SM with rank adaptation in diversity permutation. The transmission rank is determined by the STC Rate. STC Rate 1 means the rank-1 precoding.

            - MIMO feedback 2

MIMO feedback mode 2 is used for the OL-SU SM in localized permutation for frequency selective scheduling. The STC Rate indicates the preferred number of streams for SM. The subband CQI shall correspond to the selected rank.

            - MIMO feedback 3

MIMO feedback mode 3 is used for the CL-SU SM in localized permutation for frequency selective scheduling. The STC Rate indicates the preferred number of streams for SM. The subband CQI shall correspond to the selected rank.

            - MIMO feedback 4

The MIMO feedback mode 4 is used for the CL SU MIMO using long-term beamforming with rank 1. In this mode, MS shall feedback the wideband CQI. The wideband CQI shall be estimated at the MS assuming long-term precoding at the BS. The channel state information may be obtained at the BS via the feedback of the correlation matrix. The feedback of a wideband PMI is TBD.

            - MIMO feedback 5

The MIMO feedback mode 5 is used for OL MU MIMO in localized permutation with frequency selective scheduling. In the mode, MS shall feedback the subband selection, stream indicator and the corresponding CQI.

            - MIMO feedback 6

The MIMO feedback mode 6 is used for CL MU MIMO in localized permutation with frequency selective scheduling. In the mode, MS shall feedback the subband selection, corresponding CQI and subband PMI. The subband CQI refers to the CQI of the best PMI in the subband. Rank-1 base codebook (or its subset) is used to estimate the PMI in one subband.

            - MIMO feedback 7

The MIMO feedback mode 7 is used for CL MU MIMO in diversity permutation (long-term beamforming MU MIMO). In this mode, MS shall feedback the wideband CQI. The wideband CQI shall be estimated at the MS assuming long-term precoding at the BS. The channel state information may be obtained at the BS via the feedback of the correlation matrix. The feedback of a wideband PMI is TBD.

-------------------------------  Text End  ---------------------------------------------------