| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to Table 670 on MIMO feedback information |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to Table 670 on MIMO feedback information

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7. Missing descriptions are added in Table 670 (MIMO feedback information). In particular, we specify the nature of the event-driven feedback relative to MIMO (preferred MIMO feedback mode), and we propose to remove the TBD for the feedback of “stream index” with MIMO mode 3.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

< Modify section “15.3.7.2.6.3. MIMO feedback information” on line 26 page 75 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 670 specifies the feedback information required for MIMO operation.

Table 670 – MIMO feedback information

|  | Feedback information type | Description |
| --- | --- | --- |
| Long period feedback | Rank information | For MIMO modes 1 and 2 |
|  | Subband selection | For CRU allocations, indicating which subbands are preferred |
|  | Stream index (TBD) | For MIMO mode 3, indicating which streams are preferred. |
|  | Quantized Correlation matrix | For adaptive codebook feedback mode or long term wideband beamforming [TBD] |
|  | Time Correlation coefficient information [TBD] | For differential codebook feedback mode |
|  | PMI report for serving cell [TBD] | For long-term wideband beamforming |
|  | PMI report for neighboring cell | For PMI coordination among multiple BSs |
|  | CQI | For link adaptation (MCS selection) |
| Short period feedback | CQI | For link adaptation (MCS selection) |
|  | PMI report for serving cell | For short-term beamforming with MIMO modes 2 and 4 |
| Event-driven feedback | Preferred MIMO feedback mode | For MS reporting of its preferred MIMO mode in unsolicited manner |

-------------------------------  Text End  ---------------------------------------------------