| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for the STC rate and allowed values of DL and UL MIMO parameters |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for the STC rate and allowed values of DL and UL MIMO parameters

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7 and subclause 15.3.10. The definition of space-time coding “rate” is clarified to avoiding confusion with others uses of “rate”, such as FEC coding rate. “rate” is renamed “STC rate”. An appropriate definition is given for DL MU-MIMO, such that the “STC rate” is defined per layer. Undefined DL MIMO parameters in Table 668 for DL MU-MIMO are defined accordingly. A similar table missing from the UL MIMO section 15.3.10 is proposed. A clarification on the definition of the number of streams Mt is given to differentiate the SU-MIMO and MU-MIMO cases. Some editorial changes to the text are also proposed.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify the definition on “rate” on line 65 page 1 as follows>

-------------------------------  Text Start  ---------------------------------------------------

3. Definitions

3.110 STC rate: The number of QAM symbols signaled per array channel use.

-------------------------------  Text End  ---------------------------------------------------

<Modify the text starting at line 28 on page 69 in section 15.3.7.1.1 Layer to stream mapping >

-------------------------------  Text Start  ---------------------------------------------------

For SU-MIMO transmissions, the STC rate is defined as in equation (193).

R=MNF						(193)

For MU-MIMO transmissions, the STC rate per layer (R) is equal to 1.

-------------------------------  Text End  ---------------------------------------------------

< Modify the second row of Table 670 on line 32 page 75 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 670 – MIMO feedback information

|  | Feedback information type | Description |
| --- | --- | --- |
| Long period feedback | Rank information STC rate | For MIMO modes 0, 1 and 2 |

-------------------------------  Text End  ---------------------------------------------------

<Modify the text starting at line 44 on page 71 in section 15.3.7.1.3 Downlink MIMO transmission modes>

-------------------------------  Text Start  ---------------------------------------------------

The allowed values of the Some parameters for each DL MIMO mode are shown in Table 668.

Table 668 – DL MIMO Parameters

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |

Mt refers to the number of streams transmitted to one MS with MIMO modes 0, 1, and 2.

Mt refers to the total number of streams transmitted to multiple MS on the same RU with MIMO modes 3 and 4.

-------------------------------  Text End  ---------------------------------------------------

<Insert the following text at line 20 on page 122 after Table 695 in section 15.3.10.1.3 Uplink MIMO transmission modes>

-------------------------------  Text Start  ---------------------------------------------------

The allowed values of the parameters for each UL MIMO mode are shown in Table 696.

Table 696 – UL MIMO Parameters

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 1 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |

Mt refers to the number of streams transmitted by one MS.

-------------------------------  Text End  ---------------------------------------------------