| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text modifications for non-adaptive precoding MIMO sections |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text modifications for non-adaptive precoding MIMO sections

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7 (DL MIMO) and subclause 15.3.10 (UL MIMO). The non-adaptive precoding rules for OL MIMO modes (OL SU-MIMO and OL MU-MIMO) are specified according to the MIMO encoder format (MEF) and according to whether the allocation is inside or outside the OL region.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

< Modify section “15.3.7.1.2.1. Non-adaptive precoding“ on line 55 page 70 as follows>

-------------------------------  Text Start  ---------------------------------------------------

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. W belongs to one of the subsets of the base codebook specified in Section 15.3.7.2.6.6.2.4.1, according to the type of allocation, MEF, Nt and Mt, as specified in Tables X and Y.

Table X – Codebook subsets used for non-adaptive precoding in DL diversity allocations (DRU and distributed minibands)

| MEF | RU with Mt pilot streams outside OL region | RU in OL region with MaxMt streams |
| --- | --- | --- |
| SFBC | CDLOLSU(Nt, Mt, Nw), Mt = 2 | CDLOLSU(Nt, 2, Nw), MaxMt = 2 |
| VE | CDLOLSU(Nt, Mt, Nw), Mt = 1, …, MaxMt | CDLOLSU(Nt, 2 Nw), MaxMt= 2 |
| HE | n.a. | n.a. |

Table X – Codebook subsets used for non-adaptive precoding in DL localized allocations

| MEF | RU with Mt pilot streams outside OL region | RU in OL region with MaxMt streams |
| --- | --- | --- |
| SFBC | n.a. | n.a. |
| VE | Nt=2: C(2, Mt, 3), Mt = 1, …, MaxMt Nt=4: C(4, Mt, 4), Mt = 1, …, MaxMt Nt=8: C(8, Mt, 4), Mt = 1, …, MaxMt | Nt=2: C(2, MaxMt, 3), MaxMt=1 or 2 Nt=4: C(4, MaxMt, 4), MaxMt=1 or 2 Nt=8: C(8, MaxMt, 4), MaxMt=1 or 2 |
| HE | Nt=2: C(2, MaxMt, 3), Mt = 1, …, MaxMt Nt=4: C(4, MaxMt, 4), Mt = 1, …, MaxMt Nt=8: C(8, MaxMt, 3), Mt = 1, …, MaxMt | Nt=2: C(2, 2, 3), MaxMt = 2 Nt=4: C(4, 2, 4), MaxMt = 2 Nt=8: C(8, 2, 4), MaxMt = 2 |

Non-adaptive precoding outside the OL region

In a RU allocated outside the OL region, with MEF = 0b00 or 0b01 and non-adaptive precoding, the matrix W changes every N1PSC contiguous physical subcarriers according to equation (10), and it does not depend on the subframe number. W belongs to the subset of the base codebook specified in Section 15.3.7.2.6.6.2.4.1. The Nt×Mt precoding matrix W(k) applied on subcarrier k in subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

.				(1)

, s = 0… KSB.				(9)

In a RU allocated outside the OL region, with MEF = 0b10 and non-adaptive precoding, the matrix W changes every N1PSC contiguous physical subcarriers according to equation (10), and it does not depend on the subframe number. The Nt×Mt precoding matrix W(k) applied on subcarrier k in subband s is selected as any Mt unordered columns of the codeword of index i in the open-loop codebook subset of rank MaxMt, where i is given by equation (9).

Non-adaptive precoding inside the OL region

In a RU allocated in the MaxMt-streams OL Region [TBD], with MEF = 0b00, 0b01 or 0b10 and non-adaptive precoding, the matrix W changes every NPsc contiguous physical subcarriers. The default value of N is N1. N2 is optional [TBD]. Use of N2 does not require additional signaling.

When N = N1, The Nt×MaxMt precoding matrix W(k) applied on subcarrier k in subband s is selected as the codeword of index i in the open-loop codebook subset of rank MaxMt, where i is given by

, s = 0… KSB.				(10)

When N = N2, The Nt×MaxMt precoding matrix W(k) applied on subcarrier k in PRUMB s is selected as the codeword of index i in the open-loop codebook subset of rank MaxMt, where i is given by

, s = 0… KSB.				(11)

-------------------------------  Text End  ---------------------------------------------------

< Modify section “15.3.7.2.2.3.1 MIMO mode 3 “ on page 73 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Non-adaptive precoding of section 15.3.7.1.2.1 shall be used with MIMO mode 3.

The definition of W is the same as OL SU MIMO (mode 0 and mode 1).

Inside the OL region with OL MU MIMO, the precoder W with 2 streams is predefined and fixed over time.

Outside the OL region with OL MU MIMO, the precoder W is an NtMt sub-matrix of a predefined NtMaxMt matrix.

The precoding matrixWused by the BS is represented in equation (X).

W(k)=[v1(k)v2(k)⋯vM(k)]				(X)

Wherevi(k)is the precoding vector for the i-th MS on the k-th subcarrier.

vi(k) shall be used for precoding the pilot symbols on the i-th pilot stream on the k-th subcarrier.

-------------------------------  Text Start  ---------------------------------------------------

< Modify section “15.3.10.1.2.1 Non-adaptive Precoding “ on page 121 as follows>

-------------------------------  Text Start  ---------------------------------------------------

There is no precoding if there is only one transmit antenna at the MS.

With non-adaptive precoding, the precoder W is predefined and selected from the base codebook. The changes of the precoder W is [TBD].

The base codebook is defined in [TBD]. Details of selected codebook are [TBD].

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. W belongs to one of the subsets of the base codebook specified in Section 15.3.7.2.6.6.2.4.1, , according to the type of allocation, MEF, Nt and Mt, as specified in Tables X and Y.

Table X – Codebook subsets used for non-adaptive precoding in UL diversity allocations (DRU and distributed minibands)

| MEF | RU with Mt pilot streams |
| --- | --- |
| SFBC | CULOLSU(Nt, Mt, Nw), Mt = 2 |
| VE | CULOLSU(Nt, Mt, Nw), Mt = 1, …, 4 |
| HE | Not allowed on UL |

Table Y – Codebook subsets used for non-adaptive precoding in UL localized allocations (minibands and subbands CRU)

| MEF | RU with Mt pilot streams |
| --- | --- |
| SFBC | n.a. |
| VE | C(Nt, Mt, Nw), Nw =2NB, Mt = 1, …, 4 |
| HE | Not allowed on UL |

In a RU allocated in a subframe with MEF = 0b00 or 0b01 and non-adaptive precoding, the matrix W changes every N1PSC contiguous physical subcarriers according to equation (Y), and it does not depend on the subframe number. The Nt×Mt precoding matrix W(k) applied on subcarrier k in subband s is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

.						()

-------------------------------  Text End  ---------------------------------------------------