| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications on the use of MIMO Encoding Format |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Hyunkyu Yu, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications on the use of MIMO Encoding Format

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7 and subclause 15.3.10. We clarify the allowed uses of the MIMO encoding formats (MEF) on the DL and UL. We propose a clearer presentation of Tables 667 and 695, which follows more closely the descriptions of MIMO encoding and MIMO precoding for each MIMO mode. Finally, we specify the use of MIMO mode 1 (OL SU-MIMO with VE) with Mt =1 streams on the UL for a MS with a single transmit antenna.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

< Add the following text on line 26 page 69 in section “15.3.7.1.1. Layer to stream mapping”>

-------------------------------  Text Start  ---------------------------------------------------

The three MIMO encoder formats (MEF) are SFBC, vertical encoding (VE), and horizontal encoding (HE).

-------------------------------  Text End  ---------------------------------------------------

< Modify Table 667 on page 71 in section “15.3.7.1.3. Downlink MIMO modes” as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 667 – Downlink MIMO modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | HE | non-adaptive |
| Mode 4 | CL MU-MIMO (SM) | HE | adaptive |

-------------------------------  Text End  ---------------------------------------------------

< Add the following text on line 9 page 121 in section “15.3.10.1.1 Layer to stream mapping”>

-------------------------------  Text Start  ---------------------------------------------------

Horizontal encoding (MEF = 0b10) is not supported for uplink transmissions.

A MS with 1 transmit antenna shall use vertical encoding (MEF = 0b01) for uplink transmissions.

-------------------------------  Text End  ---------------------------------------------------

< Add the following text on line 16 page 121 in section “15.3.10.1.1.1 SFBC encoding”>

-------------------------------  Text Start  ---------------------------------------------------

SFBC encoding format shall not be allocated to an MS with 1 transmit antenna.

-------------------------------  Text End  ---------------------------------------------------

< Add the following text on line 29 page 121 in section “15.3.10.1.1.2 Vertical encoding”>

-------------------------------  Text Start  ---------------------------------------------------

Vertical encoding with 1 stream (Mt = 1) format shall be allocated to an MS with 1 transmit antenna.

-------------------------------  Text end  ---------------------------------------------------

< Modify Table 695 on page 122 in section 15.3.10.1.3 Uplink MIMO transmission modes” as>

-------------------------------  Text Start  ---------------------------------------------------

Table 695 – Uplink MIMO modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL Collaborative spatial multiplexing (MU-MIMO) | VE | non-adaptive |
| Mode 4 | CL Collaborative spatial multiplexing (MU-MIMO) | VE | adaptive |

-------------------------------  Text End  ---------------------------------------------------