| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications to the notations used for the base codebooks and on Table 680 |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications to the notations used for the base codebooks and on Table 680

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7. We propose clarifications in the terminologies used for describing the base codebooks for DL 4Tx and DL 8Tx:

- We correct Table 680 according to the resolution of comment 12 in 80216m-09_0014r3

- We specify the terminology “rank-Mt base codebook”

- We specify that the codebook element C(Nt, Mt, NB) is a complex matrix of dimension Nt by Mt

- We accordingly correct the missing “transpose” signs in Tables 674, 675, 676 and 677

- We clarify the meaning of the last 4 columns of Table 674, which represent the BS antennas

- We introduce the binary index in Tables 679 and 680 to be consistent with Tables 674-677

- We add the missing “m” in the notations C(,,,m) on the first rows of Tables 679 and 680

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2.6.6.2. Base mode for codebook-based feedback” on line 55 page 76 as follows>

-------------------------------  Text Start  ---------------------------------------------------

The base codebook is a unitary codebook. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix.

The MS selects its preferred matrix from the base codebook based on the channel measurements. The MS feedbacks sends the index of the preferred codeword by feedback, and the BS computes the precoder W according to the index. Both BS and MS use the same codebook for correct operation.

For the base mode, the PMI feedback from a mobile station shall represent an entry of the base codebook, where the base codebooks are defined as follows for two, four, and eight transmit antennas at the BS.

The notation C(Nt, Mt, NB) denotes the rank-Mt base codebook, which consists of 2NB complex, matrices of dimension Nt by Mt, and Mt denotes the number of streams.

The notation C(Nt, Mt, NB, i) denotes the i-th codebook entry of C(Nt, Mt, NB), which is a complex matrix of dimension Nt by Mt.

-------------------------------  Text End  ---------------------------------------------------

<Modify the first row of Table 674 on page 80 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 674 – C(4,1,6)

| # Binary Index | # m |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Antenna 0 | Antenna 1 | Antenna 2 | Antenna 3 |

-------------------------------  Text End  ---------------------------------------------------

<Modify the first row of Table 675 on page 82 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 675 – C(4,2,6)

-------------------------------  Text End  ---------------------------------------------------

<Modify the first row of Table 676 on page 83 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 676 – C(4,3,4)

-------------------------------  Text End  ---------------------------------------------------

<Modify the first row of Table 677 on page 83 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 677 – C(4,4,3)

------------------------------  Text End  ---------------------------------------------------

<Modify Table 679 on page 86 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 679 – Rank 1 of SU MIMO 4bit 8Tx base codebook

| Binary index | m | Base Matrix | C(8,1,4,m) |
| --- | --- | --- | --- |
| 0000 | 1 | V8(:,:,3) | V8(:,1,3) |
| 0001 | 2 |  | V8(:,2,3) |
| 0010 | 3 |  | V8(:,3,3) |
| 0011 | 4 |  | V8(:,4,3) |
| 0100 | 5 |  | V8(:,5,3) |
| 0101 | 6 |  | V8(:,6,3) |
| 0110 | 7 |  | V8(:,7,3) |
| 0111 | 8 |  | V8(:,8,3) |
| 1000 | 9 |  | V8(:,9,3) |
| 1001 | 10 |  | V8(:,10,3) |
| 1010 | 11 |  | V8(:,11,3) |
| 1011 | 12 |  | V8(:,12,3) |
| 1100 | 13 |  | V8(:,13,3) |
| 1101 | 14 |  | V8(:,14,3) |
| 1110 | 15 |  | V8(:,15,3) |
| 1111 | 16 |  | V8(:,16,3) |

-------------------------------  Text End  ---------------------------------------------------

<Correct Table 680 on page 88 according to the resolution of comment 12 in 80216m-09_0014r3, and modify the first row and first column as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 680 – Ranks 2 to 8 of SU MIMO 4bit 8Tx base codebook

| Binary index | m | Base Matrix | C(8,2,4,m) | C(8,3,4,m) | C(8,4,4,m) | C(8,5,4,m) | C(8,6,4,m) | C(8,7,4,m) | C(8,8,4,m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0000 | 1 |  | 1 5 | 1 3 5 | 1537 | 12357 | 123567 | 1234567 | 12345678 |
| 0001 | 2 |  | 2 6 | 2 4 6 | 2648 | 12468 | 124568 | 1234568 | n/a |
| 0010 | 3 |  | 3 7 | 2 3 7 | 3726 | 23467 | 234678 | 1234678 | n/a |
| 0011 | 4 |  | 4 8 | 1 4 8 | 4815 | 13458 | 134578 | 1234578 | n/a |
| 0100 | 5 |  | 5 3 | 3 5 7 | 5372 | 23567 | 234567 | 2345678 | n/a |
| 0101 | 6 |  | 4 6 | 4 6 8 | 6481 | 14568 | 134568 | 1345678 | n/a |
| 0110 | 7 |  | 2 7 | 2 6 7 | 7264 | 24678 | 124678 | 1245678 | n/a |
| 0111 | 8 |  | 8 1 | 1 5 8 | 8153 | 13578 | 123578 | 1235678 | n/a |
| 1000 | 9 |  | 1 3 | 1 2 3 | 1234 | 12345 | 123456 | 1234567 | 12345678 |
| 1001 | 10 |  | 2 4 | 1 2 4 | 1246 | 12456 | 124567 | 1245678 | n/a |
| 1010 | 11 |  | 2 3 | 2 3 4 | 2437 | 23478 | 123478 | 1234578 | n/a |
| 1011 | 12 |  | 1 4 | 1 3 4 | 1348 | 13478 | 134678 | 1234678 | n/a |
| 1100 | 13 |  | 5 8 | 5 7 8 | 3578 | 23578 | 235678 | 1235678 | n/a |
| 1101 | 14 |  | 6 7 | 6 7 8 | 4678 | 14678 | 145678 | 1345678 | n/a |
| 1110 | 15 |  | 5 7 | 5 7 6 | 5678 | 35678 | 345678 | 2345678 | n/a |
| 1111 | 16 |  | 6 8 | 5 6 8 | 1568 | 13568 | 123568 | 1234568 | n/a |

-------------------------------  Text End  ---------------------------------------------------

<Since CMI was deleted from Tables 679 and 680 and it is not used elsewhere (PMI is a more commonly accepted acronym), delete the definition of CMI on line 18 page 3.>

-------------------------------  Text Start  ---------------------------------------------------

CMI	codebook matrix index

-------------------------------  Text End  ---------------------------------------------------