| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to section “Downlink signaling support of UL-MIMO modes” |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim, Hyunkyu Yu, Heewon Kang, Hokyu Choi Samsung Electronics Wookbong Lee LG Electronics Alexei Davydov Intel | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.10 (UL-MIMO) “Comments on AWD 15.3.10 UL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to section “Downlinklink signaling support of UL-MIMO modes”

David Mazzarese et Al.

Samsung Electronics

Wookbong Lee

LG Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.10. We propose editorial changes in the text of section 15.3.10.2.6 (downlink signaling support of UL-MIMO modes). We also propose changes to Table 698, which specifies the DL control parameters required to control the UL MIMO.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.10.2.6 Downlink signaling support of UL-MIMO modes” on line 1 page 124 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.10.2.6 Downlink signaling support of UL-MIMO modes

15.3.10.2.6.1 Broadcast information

The BS shall send parameters listed in Table 697 which are necessary for UL MIMO operation in a control channel with a broadcast CID. The parameters may be transmitted depending on the type of operation.

The parameters may be transmitted depending on the type of operation. The broadcast information is carried by BCH or in DCD/UCD.

15.3.10.2.6.2 Unicast information

The BS shall send parameters listed in Table 698, which are necessary for MIMO operation, by unicast in a control channel to a specific MS. The parameters may be transmitted depending on the type of operation.

The BS shall send parameters necessary for UL MIMO operation in a unicast message. The parameters may be transmitted depending on the type of operation. The unicast information is carried in the A-MAP IE, in the FBCH_Alloc_IE, on in the Sounding_ IE. feedback allocation IEs.

Table 698 specifies the DL control parameters required for UL MIMO operation.

Table 698 – Unicast information for UL MIMO operation control parameters

| Parameter | Description | Value | Control channel (IE) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| Unicast information |  |  |  |  |  |
| MEF | MIMO Encoding Format | 0b0: SFBC 0b1: Vertical encoding 0b10: CSM 0b11: No encoding [One TX antenna MS] |  | A-MAP IE (unicast) | MIMO encoding format [MEF bit-field may not be explicitly indicated in DL A-MAP IE] |
| CSM | Collaborative Spatial Multiplexing | Disabled or enabled |  | A-MAP IE | SU MIMO if CSM is disabled MU MIMO if CSM is enabled |
| Mt | Number of streams | 1 to 4 0b00: 1 0b01: 2 0b10: 3 0b11: 4 (Mt <= Nt ) |  | A-MAP IE (unicast) | Number of streams in the MS transmission. |
| RU allocation (TBD) | LRU allocation | TBD |  | A-MAP IE (unicast) | Refer to DL control group |
| TNS MaxMt | Total number of streams in the LRU | 1 to 4 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |  | A-MAP IE (unicast) | Enabled when MEF=0b10 CSM is enabled Indication of the total number of streams in the LRU |
| SI | First pilot index | 1 to 4 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |  | A-MAP IE (unicast) | Enabled when MEF=0b10 CSM is enabled 1 bit for 2Tx, 2 bit for 4Tx |
| PF | Precoding flag | 0b0: non adaptive precoding or 0b1: adaptive codebook precoding |  | A-MAP IE (unicast) | Disabled when MEF=0b11 It cannot be applied to MS with 1 transmit antenna |
| PMI Indicator | PMI indicator | 0b0: the MS shall use the precoder of rank Mt of its choice 0b1: the indicated PMI of rank Mt shall be used by the MS for precoding |  | A-MAP IE | This field is relevant only when PF indicates adaptive codebook precoding.= 0b1 . PMI indication = 0b0 may be used in TDD. When PMI indication = 0b1, the BS selects the precoder to use at the MS. |
| PMI | Precoding matrix index in the UL base codebook | 0 to 7 when Nt=2 0 to 63 when Nt =4 TBD |  | A-MAP IE (unicast) | Enabled when PF indicates adaptive codebook precoding.= 0b1 and PMI indication = 0b1. [Bit-field length is variable, depending on the number of code matrices] |

-------------------------------  Text End  ---------------------------------------------------