| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed modifications to section “Downlink signaling support of DL-MIMO modes” |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | David Mazzarese, Bruno Clerckx, Sangheon Kim, Hyunkyu Yu, Heewon Kang, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) “Comments on AWD 15.3.7 DL-MIMO” |  |
| Abstract | The contribution proposes text for the MIMO section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed modifications to section “Downlink signaling support of DL-MIMO modes”

David Mazzarese et Al.

Samsung Electronics

- Introduction

This contribution proposes amendment text for MIMO, to be included in the IEEE 802.16m Amendment Working Document [1], subclause 15.3.7 and subclause 15.3.10. We first propose to move the content of mis-placed sections 15.3.7.2.5.1 (Broadcast information) and 15.3.7.2.5.2 (Unicast information) into section 15.3.7.2.6.5 (Downlink signaling support of DL-MIMO modes). Some editorial changes are proposed to this content, such as the renaming of the control channels according to the AWD naming conventions. The modified content of Table 672 is proposed.

- References

[1] IEEE 802.16m-09/0010r1a, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”, 2009-04-10.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

- Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2.6.5. Downlink signaling support of DL-MIMO modes” on line 19 page 76 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.6.5. Downlink signaling support of DL-MIMO modes

Some parameters necessary for DL MIMO operation shall be broadcast by the BS. The BS shall send some parameters necessary for DL MIMO operation in a broadcast message. The broadcast information is carried in the SFH or in the additional broadcast information by BCH or in DCD/UCD.

Some parameters necessary for DL MIMO operation shall be unicast by the BS to a specific MS. The BS shall send some parameters necessary for DL MIMO operation in a unicast message. The unicast information is carried in the A-MAP IE, in the FBCH_Alloc_IE, on in the Sounding_ IE. feedback allocation IEs.

Table 672 specifies the DL control parameters required for MIMO operation.

Table 672 – DL MIMO control parameters

| Parameter | Description |  | Value | DL Control channel (IE) | Notes |
| --- | --- | --- | --- | --- | --- |
| Broadcast Information |  |  |  |  |  |
| Nt | Number of transmit antennas at the BS |  | 2, 4, 8 | SFH (system information) | Nt must be known before decoding the DL A-MAP IE |
| OL_Region [TBD] | To indicate the location of the OL region and the number of streams |  | TBD | Additional broadcast information | OL MIMO transmission region with improved intercell interference statistics |
| SU_CT | SU base codebook type for CL MIMO modes 2 |  | Base codebook or base codebook subset | Additional broadcast information | All MSs shall use the SU base codebook subset of Table 681 when indicated by the BS when Nt=4 SU base codebook subset indication |
| MU_CT (TBD) | MU base codebook type for CL MIMO modes 4 |  | base codebook or base codebook subset | Additional broadcast Information | All MSs shall use the MU base codebook subset of Table 681 or Table TBD when indicated by the BS when Nt=4 or Nt=2, respectively MU base codebook subset indication |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation |  | BitMAP: 8 bits if Nt=2 16 bits if Nt=4 8 bits if Nt=8 Or enhanced subset indication (1 bit) | Additional broadcast information | Rank-1 codebook element restriction/recommendation information It shall be ignored if CM = 0b00, 0b01 or 0b10 |
| MaxMt (TBD) | Maximum number of streams |  | 0b00: 2 0b01: 3 0b10: 4 0b11: reserved | Broadcast information | If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| Unicast Information |  |  |  |  |  |
| MEF | MIMO encoder format |  | SFBC Vertical encoding Horizontal encoding | A-MAP IE (unicast) | MIMO encoder format. [MEF bitfield may not be explicitly indicated in DL A-MAP IE] |
| Mt | Number of streams in transmission |  | 1 to 8 | A-MAP IE (unicast) | Number of streams in the transmission. When MEF is SFBC: Mt =2 MEF is HE, Mt <= 4 [Bit-field length is variable, depending on the number of Tx at BS] |
| RU allocation (TBD) | RU [and stream] indicator for the burst of data |  | TBD | A-MAP IE (unicast) | Refer to DL control group. |
| SI (TBD) | Index of pilot stream allocation |  | 1 to 4 | A-MAP IE (unicast) | SI shall be indicated if MEF is HE [Bit-field length is variable, depending on the number of Tx at BS] RU allocation and SI can be merged together depending on other DG’s decision |
| Feedback Allocation IEs |  |  |  |  |  |
| MFM | MIMO feedback mode | Refer to Table 671 |  | FBCH_Alloc_IE | To decide the feedback content and related MS processing |
| MaxMt | Maximum number of streams | If MFM indicates a SU feedback mode: 1, 2, 4 or 8. If MFM indicates a MU feedback mode: 1, 2, 3, or 4. |  | FBCH_Alloc_IE | If MFM indicates a SU feedback mode: the maximum number of streams scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| DLRU (TBD) | Downlink RU, indicating which RUs or which type of RU (DRU or miniband-based CRU) to work on for feedback | TBD (Tree structure, bit map etc) |  | FBCH_Alloc_IE | To process CQI (PMI) estimation for the indicated RUs Refer to other DG |
| Feedback allocation IEs | MIMO feedback type for CL MIMO modes 2 and 4 | 0b00: codebook 0b01: sounding |  | FBCH_Alloc_IE Sounding_Alloc_IE | FBCH_Alloc_IE for codebook-based feedback Sounding_Alloc_IE for sounding-based feedback |
| CM | Codebook feedback mode for CL MIMO modes 2 and 4 | 0b00: standard mode with codebook coordination disabled 0b01: adaptive mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: standard mode with codebook coordination enabled |  | FBCH_Alloc_IE | In codebook-based feedback mode: Codebook coordination disabled: MS finds PMI within whole broadcasted codebook type entry Codebook coordination enabled: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |
| CCE | Codebook Coordination Enable | 0b0: Disable 0b1: Enable |  |  | CCE = 0b0: MS finds PMI within whole broadcasted codebook type entry CCE = 0b1: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |

-------------------------------  Text End  ---------------------------------------------------