| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal for UL Fast Feedback Allocation for IEEE 802.16 Amendment |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Rajni Agarwal, Kevin Power Fujitsu | Rajni.agarwal@uk.fujitsu.com Kevin.power@uk.fujitsu.com |
| Re: | AWD Comment for UL Fast Feedback Allocation |  |
| Abstract | This contribution provides the Fast Feedback Signaling Details including Feedback Content and reporting method for Primary as well Secondary Fast Feedback Channels |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fast Feedback Signaling Mechanism

Rajni Agarwal, Kevin Power

Fujitsu

- Introduction

This contribution provides the AWD text covering Fast Feedback Signaling Details, specifically Information Elements for Fast Feedback Allocation for Primary as well Secondary Fast Feedback Channels.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# Feedback Allocation A-MAP IE

Table 1 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in 1.

Table 1 Feedback Allocation A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE == [TBD] |
| Channel Index | Variable | Physical channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in section 15.3.8.3.3.2) |
| Number of Reports (n) | [1] | 0: Same Feedback reported on each FFB opportunity 1: Different Feedback reported in a time-interleaved fashion over the FFB opportunities |
| Short Allocation Period (p) | [3] | A feedback is transmitted on the FBCH every (2^p) frames |
| If(Number of Reports>0) { |  |  |
| Long Allocation Period (q) | [2] | Long Period Fast Feedback reports are transmitted on the FBCH channel every [2*(2^q)]*[2^p] frames |
| } |  |  |
| MaxMt | Variable [1-3] | Variable number of bits - depends on number of transmit antenna Nt - (MaxMt <= min(Nt,Nr)) - If Nt=2: - 0b0: 1 - 0b1: 2 - If Nt=4: - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8: - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 |
| For (i==0 to n){ |  | For specified Number of Reports (n) |
| MFM | [3] | - MIMO Feedback Mode as defined in Table XXX [TBD] |
| FPS | [2] | - Frequency Partition and Reuse Factor as follows: - 0b00: Frequency Partition 1 (Reuse 1) - 0b01: Frequency Partition 2 (Reuse 3) - 0b10: Frequency Partition 3 (Reuse 3) - 0b11: Frequency Partition 4 (Reuse 3) |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

FCT: Feedback Channel Type

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

Number of Reports (n): Indicates whether the FFB channel allocated to the AMS carries 1 or 2 FFB Reports.

Long/Short Period Reports: Applies when the Number of Reports is more than 1. By default, the first report description in the Allocation IE is the Short-Period Report while the subsequent report description is the Long-Period Report. This can switch implicitly every time based the on the mode/zone used for the last data-allocation.

Short Allocation Period (p): Default allocation period for the FFB report when Number of reports is equal to 1; short allocation period for the Short-Period report when Number of reports is more than 1.

Long Allocation Period (q): Indicates the allocation period for Long-Period report and these puncture the Short-Period reports.

MFM: MIMO feedback mode, defined in XXX [TBD].

FPS: Frequency Partition Selection & Reuse Factor Information.

Feedback Format: This field specifies the feedback format index when reporting fast feedback information in FBCH. Feedback format definitions for different MIMO feedback modes are described in table xxx in section xxx

MaxMt: This field specifies the maximum rank to be fed back by the AMS if MFM=0,1,2,3,4 (which indicates a SU MIMO feedback mode for SM), or it specifies the maximum number of users scheduled on each RU at the BS if MFM=5,6,7 (which indicates a MU MIMO feedback mode)

CM: this field specifies codebook feedback mode, which is needed only for MFM 3 and 6.

-------------------------------  Text End  -------------------------