| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on DL Control Channel for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-4-27 |  |
| Source(s) | Yanfeng Guan, Wei Xia, Huiying Fang, Ying Liu ZTE Corporation | E-mail: guan.yanfeng@zte.com.cn |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text for DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on DL Control Channel for IEEE 802.16m Amendment

Yanfeng Guan,Wei Xia, Huiying Fang,Ying Liu

ZTE Corporation

- Introduction

This contribution proposes text to be included in the DL control section of the 802.16m amendment working document (AWD). Some essential system parameters related to legacy systemt support, EMBS support and Multicarrier support for network entry should be included in S-SFH IE.

Add three information indicators into S-SFH SP1 IE, and corresponding information into SP for network entry.

References

[1] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[2] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[3] IEEE 80216m-09_0010r1a. Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems. Advanced Air Interface(working document), March, 2008.

[4] IEEE 80216m-09_558r3. “Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment”

- Text proposal for inclusion in the 802.16m amendment

- Proposal 1: Add the legacy information indicator into SP1 and related legacy information into SP for network entry.

- ------------------------------- Text Start  ---------------------------------------------------

# S-SFH IE

Table 655-S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| … | … | … |
| Legacy information indicator | 1 | 0b0: Absence of Legacy System Information Element 0b1: Presence of Legacy System Information Element |
| … | … | … |
| } |  |  |

Legacy Information

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| If(Duplex Mode==TDD){ |  |  |
| If(Legacy Information Indication==1){ |  |  |
| Frame_Offset | 2 | 0b001: 1 subframe 0b010: 2 subframes 0b011: 3 subframes 0b100: 4 subframes |
| DL subframe number for the Advanced Air Interface | TBD | TBD |
| Number of UL Legacy subframes | 2 | 0b00: 1 subframe 0b01: 2 subframes 0b10: 3 subframes 0b11: 4 subframes |
| UL Duplex Mode in Legacy | 1 | 0b0: TDM 0b1: FDM |
| If(UL Duplex Mode in Legacy==0b1){ |  |  |
| Resource Information | TBD | TBD |
| } |  |  |
| } |  |  |
| … | … | … |

- ------------------------------- Text End  ---------------------------------------------------

- Proposal 2: Add the Multicarrier information indicator into SP1 and related multicarrier information into SP for network entry.

- ------------------------------- Text Start  ---------------------------------------------------

# S-SFH IE

Table 655-S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| … | … | … |
| Multicarrier information indicator | 1 | 0b0: Absence of Multicarrier Information Element 0b1: Presence of Multicarrier Information Element |
| … | … | … |
| } |  |  |

Multicarrier Information

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| If(Multicarrier information indicator==1){ |  |  |
| Number of Partially Configured Carrier | TBD | TBD |
| For (i=0;i++;i< Number of Partially Configured Carrier){ |  |  |
| Partially Configured Carrier Information | TBD | Essential system parameters related to other partially configured carriers. |
| } |  |  |
| } |  |  |
| … | … | … |

- Proposal 3: Add the EMBS information indicator into SP1 and related EMBS information into SP for network entry.

- ------------------------------- Text Start  ---------------------------------------------------

# S-SFH IE

Table 655-S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| … | … | … |
| MBS information indicator | 1 | 0b0: Absence of MBS Information Element 0b1: Presence of MBS Information Element |
| … | … | … |
| } |  |  |

EMBS IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| If(EMBS information indicator==1){ |  |  |
| Number of EMBS subframe | 2 | 0b00: 1 subframe 0b01: 2 subframes 0b10: 3 subframes 0b11: 4 subframes |
| Multiplex Mode in EMBS subframe | 1 | 0b0: TDM 0b1: FDM |
| If (Multiplex Mode in EMBS subframe==0b1){ |  |  |
| Resource Information | TBD |  |
| } |  |  |
| } |  |  |
| … | … | … |

-------------------------------  Text End  ---------------------------------------------------