| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | IEEE 802.16m AWD Text Proposal for Security |  |
| Date Submitted | 2009-4-27 |  |
| Source(s) | Whai-En Chen, Shih-Yuen Cheng National Ilan University Shiann-Tsong Sheu, Chih-Cheng Yang National Central University Kanchei (Ken) Loa, Yung-Ting Lee, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Hsueh Tsai, Tsung-Yu Tsai Institute for Information Industry Yang-Han Lee, Yih Guang Jan Tamkang University | Voice: +886-3-9357400#255 Fax: +886-3-9353804 wechen@niu.edu.tw Voice: +886-2-66000100 Fax: +886-2-66061007 loa@iii.org.tw |
| Re: | Category: AWD-New contribution / Area: Security “Comments on AWD 10.6. Security” |  |
| Abstract | The contribution describes the AWD text of IEEE 802.16m security section. |  |
| Purpose | Discussion and Approval |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m AWD Text Proposal for Security

Whai-En Chen, Shih-Yuen Cheng

National Ilan University

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University

Kanchei (Ken) Loa, Yung-Ting Lee, Chiu-Wen Chen, Chun-Yen Hsu, Youn-Tai Lee, Yi-Hsueh Tsai,

Tsung-Yu Tsai

Institute for Information Industry (III)

Yang-Han Lee, Yih Guang Jan

Tamkang University

# 1. Introduction

To enable identity privacy, the permanent IEEE 802 MAC address shall be protected by using temporary Station ID (T-STID) prior to the initiation of authentication/authorization phase. The AMS and the ABS exchange the permanent IEEE 802 MAC address through the encrypted messages during the authorization phase.

In order to protect the mapping between the STID and the AMS’s IEEE 802 MAC Address, a T-STID is used until the successful authentication process.

First, an AMS randomly selects a T-STID and utilizes the T-STID to send a RNG-REQ message. Upon receipt of the RNG-REQ message, the ABS checks whether the T-STID has been used by other AMS. If yes, the ABS sends a RNG-RSP message with a new T-STID. Otherwise, the ABS sends a RNG-RSP message without assigning a new T-STID. Then the AMS and the ABS can use the T-STID to establish a secure channel. After successful authentication process, the AMS sends encrypted IEEE 802 MAC address to the ABS through the secure channel. After the ABS verifies the received IEEE 802 MAC address, the ABS sends STID to the AMS and releases the T-STID. Figure x illustrates the example of AMS privacy.

Fig. x. Example of Identity Privacy.

# 2. Text Proposal

[--------------------------------------------------------Start of Text Proposal---------------------------------------------------]

10.6 Security

[add new subsection to section 10.6]

# 10.6.x AMS privacy

[Insert the following texts into the section 10.6.x]

In order to protect the mapping between the STID and the AMS MAC Address, temporary STID (TSTID) is used until the successful authentication process. AMS selects a temporary STID and sends it in RNG-REQ message. After receiving RNG-REQ message, if the TSTID is already used in the system, ABS sends RNG-RSP message with new TSTID. Otherwise, ABS sends RNG-RSP message without new TSTID. AMS and ABS establish a secure channel by using this TSTID. After successful authentication process, AMS sends encrypted MAC address to ABS through the secure channel. After ABS verifies the received MAC address, ABS sends STID to AMS and release the TSTID. Figure x illustrates the example of AMS privacy.

Fig. x. Example of Identity Privacy.