| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Broadcast Control Information Elements (SFH-IE) |  |
| Date Submitted | 2009-04-29 |  |
| Source(s) | Doo-hyun Sung LG Electronics YihShen Chen MediaTek Ming-Hung Tao ITRI Maheshwari Shashikant NSN Kiran Kuchi, J. Klutto Milleth CeWit Sassan Ahmadi, Shantidev Mohanty Intel | dh.sung@lge.com yihshen.chen@mediatek.com minghung02@gmail.com shashi.maheshwari@nsn.com kkuchi@cewit.org.in Sassan.ahmadi@intel.com shantidev.mohanty@intel.com |
| Re: | Category: AWD-Comments /Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.5 DL-CTRL” |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Broadcast Control Information Elements (SFH-IE)

Doo-hyun Sung

LG Electronics

YihShen Chen

MediaTek

Ming-Hung Tao

ITRI

Maheshwari Shashikant

NSN

Kiran Kuchi, J. Klutto Milleth

CeWit

Sassan Ahmadi, Shantidev Mohanty

Intel

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# P-SFH IE

Table 654 – P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSBs of superframe number | TBD 4 | Part of superframe number |
| System Configuration Description change count | TBD 4 |  |
| S-SFH Size | TBD 4 | The units of LRU |
| S-SFH Transmission Format | 2 | Indicate the transmission format (repetition) used for S-SFH |
| S-SFH SP Scheduling information bitmap | 3 | 0b000: no S-SFH If 1st bit = 1, S-SFH includes SP1 otherwise no SP1 If 2nd bit = 1, S-SFH includes SP2 otherwise no SP2 If 3rd bit = 1, S-SFH includes SP3 otherwise no SP3 |
| S-SFH SP change bitmap | 3 |  |
| Reserved | TBD | Note the size of P-FSH SFH should be fixed. The reserved bits are for future use. |
| The reserved bits are for future extension |  |  |
| } |  |  |

SCD Count

Incremented by one (modulo TBD 16) by the BS whenever any of the values of the S-SFH IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decide that the S-SFH IEs have not changed and may be able to disregard the S-SFH IEs.

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 656 655 includes essential information needed for select and access the network.

Table 655 – S-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| Additional broadcast information indicator | TBD |  |
| HO information (Open BS/Close BS, UL load indicator, cell type) | TBD | For uncontrolled HO scenarios |
| Cell bar info | TBD | If Cell Bar bit = 1, this cell is not allowed for any new initial entry |
| Reserved | TBD | The reserved bits are for future extension |
| } | TBD |  |

S-SFH SP2 contains information for network re-entry, see Table 656

Table 656 – S-SFH SP1 IE format

| Channel S-SFH Sub-packet | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 1 2 | Sector ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile for A-MAP BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Cell bar information Uplink_Burst_Profile | 8 1 6 26 26 8 8 TBD 2 4 TBD 8 TBD TBD 3 1 |

S-SFH SP2 contains information for initial network entry and network discovery, see Table 657

Table 657 – S-SFH SP2 IE format

| Channel S-SFH Sub-packet | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 3 | Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs MSB bytes of BSID Additional broadcast information indicator (ABI) | 8 2 8 4 4 8 8 1 TBD TBD 24 16 5 |

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 658.

Table 658 – S-SFH SP3 IE format

| Channel S-SFH Sub-packet | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 3 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay SP scheduling periodicity information | 8 8 8 8 8 8 8 TBD TBD TBD TBD 12 |

S-SFH SP5 is transmitted in paging listening interval. It contains information for idle mode AMSs, see Table 659.

Table 659 – S-SFH SP5 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 5 | N_PGID Paging Indicator Usage Flag PGID List If( Paging Indicator Usage Flag ==1) ( Paging Indicator Bitmap } |  |

-------------------------------  Text End  ---------------------------------------------------