| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for Authentication and Security Context |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | GeneBeck Hahn, KiSeon Ryu and Ronny YongHo Kim LG Electronic Inc. LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7188 E-mail: gbhahn@lge.com, ksryu@lge.com and ronnykim@lge.com |
| Re: | IEEE 802.16m-09/xxx. ”Call for Comments and Contributions on Project 802.16m Amendment Working Document” Target topic "Authentication", “Security Context” |  |
| Abstract | This contribution proposes the text of authentication and security context sections to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment working document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Authentication and Security Context

Gene Beck Hahn, Ki Seon Ryu and Ronny Yong Ho Kim

LG Electronics

# 1. Introduction

This contribution proposes amendment text to explain the 802.16m authentication and is intended as a section to be included in 802.16m amendment. The proposed text is developed so that it can be combined with 802.16 Rev2/D9 [1], it is compliant to 802.16m SRD [2] and 802.16m SDD [3]. This contribution follows the tentative outline and style guidelines described in [4]. The text proposal is based on current 802,16m SDD [3]. In Section 2, the main changes with regard to 802.16m SDD are outlined, which is aimed at helping the understanding of amendment text.

# 2. Modification from the SDD and Key Descriptions

The text proposed in this contribution is based on Subclause 10.6.2 in 802.16m SDD [3]. The modifications to the current SDD text are summarized below:

- Updated the authentication of 802.16m.

Subclause 10.6.2 of IEEE 802.16m SDD [3] defined a high level view of 802.16m authentication. We added the detailed descriptions on the pre-authentication capabilities negotiation, authentication and its resultant basic key derivation of 802.16m. Also, more illustrations related to the re-authentication have been made.

- Inserted Subclauses 15.2.3.7, 15.2.3.7.1, 15.2.3.7.2 and 15.2.3.7.3.

We added the Subclauses 15.2.3.7, 15.2.3.7.1, 15.2.3.7.2 and 15.2.3.7.3 to explain the security context, AK, PMK and NONCE respectively. The AK and PMK contexts of 16m essentially follow the AK and PMK contexts of 16e except the modifications made within PKMv3 [1][3]. The modifications are the exclusion of HMAC, PAK sequence number, KEK and EIK from the AK context. Also, GKEK context is not treated in 16m security context. Finally, NONCE context is a newly propsed context to support the local derivation of TEK between AMS and ABS. We describe the NONCE context in 15.2.3.7.3.

# 3. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-08/050, “IEEE 802.16m Amendment Working Document”

# 4. Text Proposal for Inclusion in the 802.16m Amendment Working Document

============================= Start of Proposed Text =============================

# 15.2.3.2 Authentication

Pairwise mutual authentication of user and device identities takes place between AMS and ABS entities using EAP. The choice of EAP methods and selection of credentials that are used during EAP-based authentication are outside of the scope of this specification.

Authentication is executed during the initial network entry after the pre-authentication capability negotiation. Security Capabilities, policies etc. are negotiated in this pre-authentication capability negotiation. The remaining AMS capability negotiation is performed together with registration after the successful completion of the authentication and the authorization. The product of EAP exchange that is transferred to IEEE 802.16m layer is the master session key (MSK), which is 512 bits in length. This key is known to the AAA server, to the Authenticator (transferred from AAA server) and to the AMS. The AMS and the Authenticator derive PMK (pairwise master key) by truncating the MSK to 160 bits.

The PMK derivation from the MSK during first EAP method is as follows:

PMK <= truncate (MSK, 160)

Re-authentication should be made before lifetime of authentication materials/credentials expires. Specifically, after the successful initial authentication, the AMS shall initiate re-authentication prior to expiration of PMK lifetime by sending the PKM EAP Start Message signed by CMAC_KEY_U derived from the AK. Either the ABS or AMS may initiate re-authentication at any time prior to expiration of PMK lifetime. After expiration of the PMK lifetime, authentication shall be performed using initial authentication procedures. Data transmission may continue during re-authentication process, by providing AMS with two sets of authentication/keying material with overlapping lifetimes. Authentication procedure is controlled by authorization state machine, which defines allowed operations in specific states.

# 15.2.3.7 Security Context

The security context is a set of parameters linked to a key in each hierarchy that defines the scope while the key usage is considered to be secure.

Examples of these parameters are the key lifetime and counters ensuring the same encryption will not be used more than once. When the context of the key expires, a new key should be obtained to continue working.

The scope of this subclause is to define the context that belongs to each key, how it is obtained and the scope of its usage.

# 15.2.3.7.1 AK Context

The PMK key has two phases of lifetime: the first begins at the PMK creation and the second begins after the validation by the 3-way handshake.

The phases ensure that when the PMK is created it will be defined with the PMK and after successful 3-way handshake, this lifetime may be enlarged using the PMK lifetime parameter within the 3-way handshake.

Cached AKs that were derived from the PMK caqn continue to be used in HO.

Reauthentication is required to obtain a new PMK so new AKs can be derived.

The AK context is described in Table 1.

Table 1. AK Context in PKMv3

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| AK | 160 | The authorization key, calculated as defined in 15.2.3.3.1.1 |
| AKID | 64 | Authorization Key Identifier = Dot16KDF(AK, 0b0000 ¦ AK SN ¦ AMS MAC Address ¦ BSID ¦ ”AK”, 64) The AK SN in the Dot16KDF function is encoded in MSB first order. |
| AK Sequence Number | 4 | Sequence number of PMK for the AK. AK SN = PMK SN |
| AK Lifetime |  | AK Lifetime (equals to PMK lifetime) Before this expires, when AK Gracetime expires, re-authentication is needed |
| PMK Sequence Number | 4 | The sequence number of the PMK from which this AK is derived. If EAP authentication is used, this value shall be set to zero |
| CMAC_KEY_U | 128 | The key which is used for signing UL management messages |
| CMAC_PN_U | 22 | Used to avoid UL replay attack on the management connection before this expires, re-authentication is needed. The initial value of CMAC_PN_U is zero and the value of CMAC_PN_U is reset to zero whenever CMAC_KEY_COUNT is increased |
| CMAC_KEY_D | 128 | The key which is used for signing DL management messages |
| CMAC_PN_D | 22 | Used to avoid DL replay attack on the management connection before this expires, re-authentication is needed. The initial value of CMAC_PN_D is zero and the value of CMAC_PN_D is reset to zero whenever CMAC_KEY_COUNT is increased |
| KEK | 128 | Used to encrypt NONCEs used to derive and update TEK between the ABS to AMS |
| CMAC_KEY_COUNT | 16 | Value of Entry Counter that is used to guarantee freshness of computed CMAC_KEY_* with every entry and provide replay protection |

# 15.2.3.7.2 PMK Context

The PMK context includes all parameters associated with the PMK. This context is created when EAP based Authentication completes. The PMK context is described in Table 2.

Table 2. PMK Context in PKMv3

| Parameter | Size (bit) | Usage |
| --- | --- | --- |
| PMK | 160 | A key yielded from the EAP-based authentication |
| PMK Sequence Number | 4 | PMK sequence number, when the EAP-based authorization is achieved and a key is generated. The 2 LSBs are the sequence counter. And the 2 MSBs set to 0 |
| PMK Lifetime |  | The time this PMK is valid. Before this expires reauthorization is needed |

# 15.2.3.7.3 NONCE Context

The NONCE context is generated and managed by the ABS. The NONCE context is used only to derive and update TEK. Whenever AK is verified via 3-way handshake, NONCE is updated.

============================= End of Proposed Text = m============================