| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on Carrier Reselection during Network Entry with Multi-Carrier Support |  |
| Date Submitted | 2009-04-28 |  |
| Source(s) | Yuqin Chen, Nan Li, Hongyun Qu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD-Comments/ Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | This contribution proposes the text for carrier reselection during initial entry to be included in 802.16m amendment text. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Carrier Reselection during Network Entry with Multi-Carrier Support

Yuqin Chen，Nan Li, Hongyun Qu

ZTE Corporation

# Introduction

In current SDD document 80216m-008_003r8, for initial entry in multi-carrier deployment, it is claimed that “Once the AMS detects the A-PREAMBLE on a fully configured carrier, the AMS may proceed with reading SFH or Extended system parameters and system configuration information where the ABS indicates its configuration and its support for multicarrier feature. The AMS can decide on proceeding with network entry with the current carrier or going to alternative carriers based on this information.”

In current AWD, a “Cell Bar bit” is used to reject the AMS’ initial entry if the carrier loading is heavy. Besides that, to expedite the carrier reselection procedure, it is proposed that ABS suggests another carrier for AMS to perform initial entry in SFH IE.

# Proposed Carrier Reselection process

The carrier re-selection procedure is proposed to take place after acquiring the synchronization with the ABS via Advanced Preamble and before ranging during initial network entry. The procedure is shown as Figure 1.

To expedite carrier reselection procedure, it is proposed to add a field of “suggested carrier information” to suggest AMS to switch to. This field is used associated with “Cell Bar bit”. When Cell bar bit sets to 1, current carrier is not allowed for any more access and then ABS recommends another carrier by the new filed.

Figure 1 Carrier reselection procedure during initial entry

# Text proposal in the 802.16m amendment

---------------------------------------------------------- Text Start ----------------------------------------------------------------

# 15.2.x Network entry and initialization

Network entry is the procedure by which an AMS finds and establishes a connection with the network. The network entry has the following steps:

- AMS synchronizes with the ABS via Advanced Preamble (A-Preamble).

- AMS obtains necessary information e.g. ABS ID, NSP ID for initial network entry, and performs network selection and carrier selection. If the “cell bar Info” sets to 1, AMS may perform carrier reselection based on ABS’ indication.

- AMS obtains necessary uplink information.

- AMS starts ranging process.

- Pre-authentication capability negotiation.

- Authentication

- Capability exchange and registration.

- AMS enters Advanced WirelessMAN-OFDMA network and sets up service flows.

# 15.3.6.4.1.2 S-SFH IE

Table 655-S-SFCH SP1 IE

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format() { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| Additional broadcast information indicator | TBD |  |
| HO information (Open BS/Close BS, UL load indicator, cell type) | TBD | For uncontrolled HO scenario |
| Cell bar info | TBD | If Cell Bar bit=1, this cell is not allowed for any new initial entry. |
| If(Cell bar info=1){ |  |  |
| Suggested carrier for initial entry | TBD | A carrier is suggested for AMS to switch to. |
| } |  |  |
| Reserved | TBD | The reserved bits are for future extension. |
| } |  |  |

------------------------------- ----------------------------Text End ------------------------------------------------------------

# References

[1] IEEE 802.16-09/0010r1a, “IEEE 802.16m Amendment Working Document”, March 31, 2009.

[2] IEEE 802.16m-07/003r8, “802.16m System Description Document”