| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on scanning with Multi-Carrier Support |  |
| Date Submitted | 2009-04-28 |  |
| Source(s) | Nan Li , Yuqin Chen，Kaiying Lv, Wenhuan Wang, Hongyun Qu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD-New contribution/ Multi-carrier and MAC management message Area: Multi-carrier and MAC management message |  |
| Abstract | This contribution proposes the text for MAC Management message for scanning with multi-carrier support to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on scanning with Multi-Carrier Support

Nan Li , Yuqin Chen，Kaiying Lv, Wenhuan Wang, Hongyun Qu

ZTE Corporation

# Introduction

It has been described in SDD that during multi-carrier operation, an AMS could either handover to another ABS which has better channel quality, QoS, load balancing condition or location, or it could change its primary carrier or secondary carrier within the serving ABS.

The serving ABS will broadcast neighbor ABS’s information to the AMSs. Meanwhile, the serving ABS informs its available carrier set in SFH or in other broadcast information. By capability negotiation, the serving BS could get each AMS’s multi-carrier capability and will provide it with an assigned carrier set.

The AMS keeps the neighbor ABS’s carrier list and its own assigned carrier set for the purpose of inter-BS HO or primary /secondary carrier change. However, the AMS will be required to perform scanning on other carriers which is not serving it before this.

This contribution proposes a general scanning process with multi-carrier system for IEEE 802.16m.

# Proposed scanning process

It is proposed that intra-BS carrier scanning process could be consistent with which in inter-BS HO. The ABS sets scanning trigger for intra-BS carrier scanning and inter-BS carrier scanning separately. When specific trigger is achieved, the corresponding scanning is performed by the AMS. The ABS may also send control signaling to start scanning. Intra-BS scanning shall reuse the SCN-REQ message and SCN-RSP message to initiate scanning.

The AMS sends an AAI-SCN-REQ MAC management message to initiate scanning procedure. The ABS will send an AAI-SCN-RSP in response. The AAI-SCN-REQ message shall contain parameters such as scanning carrier list, scanning start time and scanning interval, etc.

AAI-SCN-REQ message is transmitted through the primary carrier of the AMS. An example of the AAI-SCN-REQ message is as follows:

Table XXX- AAI-SCN-REQ message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_ SCN-REQ_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Scan interval | TBD | When AMS is using multi RFs, scan interval is the total scanning length, in which case scan iteration and interleaving interval is not needed. |
| Interleaving interval indicator | 1 | 0：no scan iteration and interleaving interval 1：scan iteration and interleaving interval exist |
| If (Interleaving interval indicator) |  |  |
| { Scan Iteration | 8 |  |
| Interleaving interval } | 8 |  |
| Scan RF change indicator | 1 | To indicate if the RF used to scan is the current RF 1：Yes 0：No |
| If(Scan RF change indicator==0) |  |  |
| {scan RF ID} | TBD | Indicate the RF will be used to scan |
| N_Recommended_BS_Index | 8 | Number of neighboring BS to be scanned or associated, value 0 means the carriers to be scanned is within the serving ABS. |
| If(N_Recommended_BS_Index == 0){ |  |  |
| N_Recommended_BS_carrier_Index | 8 | Number of carriers to be scanned in the serving ABS |
| for(i==0,i<N_Recommended_BS_carrier_Index; i++){ |  |  |
| Recommended_BS_carrier_Index | TBD | carrier physical index |
| } |  |  |
| } |  |  |
| else if( N_Recommended_BS_Index == 0xFF){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index){ |  |  |
| Scanning type | 2 | 0b000:Scanning without association. 0b00l: Scanning with association level 0: association without coordination 0b010: Scanning with association level 1: association with coordination. 0b011: Scanning with association level 2: network assisted association. |
| } |  |  |
| }else { |  |  |
| for(j = 0; j < N_Recommended_BS_Index; j++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| Scanning type | 2 |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 | Number of neighboring BS to be scanned or associated, which are using full 48 bits BS ID. |
| for(i = 0; i < N_Recommended_BS_Full; i++){ |  |  |
| Recommended BS ID | 48 |  |
| Scanning type | 2 |  |
| } |  |  |
| Padding | variable |  |
| TLV encoded information | variable |  |
| } |  |  |

In this AAI-SCN-REQ message, scanning type for the carriers in neighbor ABSs is the same. Of course, the AMS could ask for different scanning type for each carrier in neighbor ABS. All the carriers in the serving ABS will be scanned without association. For single-radio AMS, scanning and data transfer carry on alternately. There should be scan iteration and interleaving interval in the message. For multi-radio AMS, which is using two or more RFs, scan interval parameter is enough. One RF is used for scanning, while another RF maintains data exchange with the ABS.

The ABS can also send an AAI-SCN-RSP message to start scan. AAI-SCN-RSP message is transmitted through the primary carrier of the AMS. An example of the AAI-SCN-RSP message is as follows:

Table XXX- AAI-SCN-RSP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_SCN-RSP_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Scan interval | TBD | When AMS is using multi RFs, scan interval is the total scanning length, in which case scan iteration and interleaving interval is not needed. |
| Interleaving interval indicator | 1 | 0：no scan iteration and interleaving interval 1：scan iteration and interleaving interval exist |
| If (Interleaving interval indicator) { |  |  |
| Scan Iteration | 8 |  |
| Interleaving interval } | 8 |  |
| Scan RF change indicator | 1 | To indicate if the RF used to scan is the current RF 1：Yes 0：No |
| If(Scan RF change indicator==0) { |  |  |
| scan RF ID} | TBD |  |
| Report mode | 2 | 0b00: No report 0b01: Periodic report 0b10: Event-triggered report 0b11: One-time scan report |
| Report period | 8 |  |
| Report metric | 8 | Bitmap indicating presence of certain metrics on which the corresponding triggers are based: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4–7: Reserved; shall be set to zero |
| Use_Nbr_Bitmap_Index | 1 |  |
| Use_Req_Bitmap_Index | 1 |  |
| if (Scan interval != 0) { |  |  |
| Start superframe | 8 |  |
| If( Use_Nbr_Bitmap_Index == 1){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index){ |  |  |
| Scanning type | 2 | 0b000:Scanning without association. 0b00l: Scanning with association level 0: association without coordination 0b010:Scanning with association level 1: association with coordination. 0b011: Scanning with association level 2: network assisted association. |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011) { |  |  |
| CDMA_code} | TBD |  |
| N_carrier_Index | 8 | Number of carriers to be scanned in the neighbor ABS |
| for(i==0,i<N_ carrier_Index; i++){ |  |  |
| carrier_Index | TBD | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_resource | 8 |  |
| ｝ |  |  |
| ｝ |  |  |
| ｝ |  |  |
| ｝else{ |  |  |
| N_Recommended_BS_Index | 8 |  |
| If(N_Recommended_BS_Index == 0){ |  |  |
| N_Recommended_BS_carrier_Index | 8 |  |
| for(i==0,i<N_Recommended_BS_carrier_Index; i++){ |  |  |
| Recommended_BS_carrier_Index | TBD | carrier physical index |
| } |  |  |
| } else{ |  |  |
| for(i = 0; i< N_Recommended_BS_Index; i++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} | TBD |  |
| N_Recommended_BS_carrier_Index |  |  |
| for(j==0,j<N_Recommended_BS_carrier_Index; j++){ |  |  |
| Recommended_BS_carrier_Index |  | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_opportunity | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| If( Use_Req_Bitmap_Index == 1 ){ |  |  |
| Req_Bitmap_Size | TBD |  |
| Req_Bitmap_Index | TBD |  |
| for( each ‘1’ in Req_Bitmap_Index ){ |  |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} | TBD |  |
| N_carrier_Index | 8 |  |
| for(i==0,i<N_ carrier_Index; i++){ |  |  |
| carrier_Index | TBD | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_opportunity | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 |  |
| for(j = 0; j < N_Recommended_BS_Full; j++){ |  |  |
| Recommended BS ID | 48 |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} |  |  |
| N_carrier_Index |  |  |
| for(j==0,j<N_ carrier_Index; j++){ |  |  |
| carrier_Index |  | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time |  |  |
| Transmission_opportunity |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable |  |
| } |  |  |
| TLV encoded information | variable |  |
| } |  |  |

In this AAI-SCN-RSP message, scanning type for the carriers in neighbor ABSs is the same. If the scanning type is association level 1 or association level 2, the target ABS will allocate a dedicated ranging code to the AMS.AMS use this dedicated ranging code to range with the selected carriers in the target ABS in different ranging resource and superframe.

If association level 2 is applied, the AMS does not have to wait for ranging response from the target ABS after sending ranging request or ranging code to the target ABS. Ranging adjust info may be sent by each target ABS to the serving ABS. The serving ABS will generate an AAI-ASC-REP message and send it to the AMS.

TableXXX- AAI-ASC-REP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_ASC_REPORT_Message_format() { |  |  |
| Management Message Type | 8 |  |
| N_Recommended_BS_Index | 8 |  |
| for(j = 0; j < N_Recommended_BS_Index; j++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| N_carrier_Index | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  | carrier physical index |
| Timing adjust | TBD |  |
| Power level adjust | TBD |  |
| Offset frequency adjust | TBD |  |
| Ranging status | 8 |  |
| Service level prediction | 8 |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 | Number of neighboring BS that are using full 48 bits BS ID. |
| for(j = 0; j < N_Recommended_BS_Full; j++){ |  |  |
| Neighbor_BS_ID | 48 |  |
| N_carrier_Index | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  | carrier physical index |
| Timing adjust | TBD |  |
| Power level adjust | TBD |  |
| Offset frequency adjust | TBD |  |
| Ranging status | 8 |  |
| Service level prediction | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |

After scanning, a scanning report should be sent to the ABS according to the report metric for the purpose of carrier reselection. AAI-SCN-REP message is transmitted through the primary carrier of the AMS. An example of the AAI-SCN-REP message is shown as follows:

Table XXX- AAI-SCN-REP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI-SCN-REP_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Report Mode | 1 | 0: Event-triggered report 1: Periodic report |
| Use_Nbr_Bitmap_Index | 1 |  |
| Use_Rsp_Bitmap_Index | 1 |  |
| Report metric | 8 |  |
| if( Use_Nbr_Bitmap_Index == 1 ){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index ){ |  |  |
| N_scan_carrier | 8 | Number of carriers to be scanned. |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| N_Neighbor_BS_Index | 8 |  |
| for(j = 0; j < N_Neighbor_BS_Index; j++) { |  |  |
| Neighbor_BS_Index | 8 |  |
| N_scan_carrier | 8 | Number of carriers to be scanned. |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| N_ Neighbor_BS_Full | 8 | Number of neighboring BS that are using full 48 bits BS ID. |
| for(j = 0; j < N_Neighbor_BS_Full; j++) { |  |  |
| Neighbor BSID | 48 |  |
| N_scan_carrier | 8 | Number of carriers to be scanned |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| if( Use_Rsp_Bitmap_Index == 1 ){ |  |  |
| Rsp_Bitmap_size |  |  |
| Rsp_Bitmap_Index |  |  |
| for( each ‘1’ in Rsp_Bitmap_Index ){ |  |  |
| N_scan_carrier | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| padding | variable |  |
| TLV encoded information | variable |  |
| } |  |  |

# Text proposal in the 802.16m amendment

---------------------------------------------------------- Text Start ----------------------------------------------------------------

# 15.2.4 AAI MAC Management Messages

# 15.2.4.x AAI_ SCN-REQ

An AAI_SCN-REQ message may be transmitted by an AMS to request a scanning over carriers within the serving ABS and the neighbor ABSs. An AMS shall generate AAI_SCN-REQ messages in the format shown in Table XXX.

TableXXX- AAI-SCN-REQ message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_ SCN-REQ_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Scan interval | TBD | When AMS is using multi RFs, scan interval is the total scanning length, in which case scan iteration and interleaving interval is not needed. |
| Interleaving interval indicator | 1 | 0：no scan iteration and interleaving interval 1：scan iteration and interleaving interval exist |
| If (Interleaving interval indicator) |  |  |
| { Scan Iteration | 8 |  |
| Interleaving interval } | 8 |  |
| Scan RF change indicator | 1 | To indicate if the RF used to scan is the current RF 1：Yes 0：No |
| If(Scan RF change indicator==0) |  |  |
| {scan RF ID} | TBD | Indicate the RF will be used to scan |
| N_Recommended_BS_Index | 8 | Number of neighboring BS to be scanned or associated, value 0 means the carriers to be scanned is within the serving ABS. |
| If(N_Recommended_BS_Index == 0){ |  |  |
| N_Recommended_BS_carrier_Index | 8 | Number of carriers to be scanned in the serving ABS |
| for(i==0,i<N_Recommended_BS_carrier_Index; i++){ |  |  |
| Recommended_BS_carrier_Index | TBD | carrier physical index |
| } |  |  |
| } |  |  |
| else if( N_Recommended_BS_Index == 0xFF){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index){ |  |  |
| Scanning type | 2 | 0b000:Scanning without association. 0b00l: Scanning with association level 0: association without coordination 0b010: Scanning with association level 1: association with coordination. 0b011: Scanning with association level 2: network assisted association. |
| } |  |  |
| }else { |  |  |
| for(j = 0; j < N_Recommended_BS_Index; j++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| Scanning type | 2 |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 | Number of neighboring BS to be scanned or associated, which are using full 48 bits BS ID. |
| for(i = 0; i < N_Recommended_BS_Full; i++){ |  |  |
| Recommended BS ID | 48 |  |
| Scanning type | 2 |  |
| } |  |  |
| Padding | variable |  |
| TLV encoded information | variable |  |
| } |  |  |

# 15.2.4.x AAI_ SCN-REQ

An AAI_SCN-RSP message shall be transmitted by the serving ABS either unsolicited or in response to a AAI_SCN-REQ message sent by an AMS. An ABS may transmit AAI_SCN-RSP to start AMS scanning procedure. The format of the AAI_SCN-RSP message is shown in Table XXX.

TableXXX- AAI-SCN-RSP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_SCN-RSP_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Scan interval | TBD | When AMS is using multi RFs, scan interval is the total scanning length, in which case scan iteration and interleaving interval is not needed. |
| Interleaving interval indicator | 1 | 0：no scan iteration and interleaving interval 1：scan iteration and interleaving interval exist |
| If (Interleaving interval indicator) { |  |  |
| Scan Iteration | 8 |  |
| Interleaving interval } | 8 |  |
| Scan RF change indicator | 1 | To indicate if the RF used to scan is the current RF 1：Yes 0：No |
| If(Scan RF change indicator==0) { |  |  |
| scan RF ID} | TBD |  |
| Report mode | 2 | 0b00: No report 0b01: Periodic report 0b10: Event-triggered report 0b11: One-time scan report |
| Report period | 8 |  |
| Report metric | 8 | Bitmap indicating presence of certain metrics on which the corresponding triggers are based: Bit 0: BS CINR mean Bit 1: BS RSSI mean Bit 2: Relative delay Bit 3: BS RTD Bits 4–7: Reserved; shall be set to zero |
| Use_Nbr_Bitmap_Index | 1 |  |
| Use_Req_Bitmap_Index | 1 |  |
| if (Scan interval != 0) { |  |  |
| Start superframe | 8 |  |
| If( Use_Nbr_Bitmap_Index == 1){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index){ |  |  |
| Scanning type | 2 | 0b000:Scanning without association. 0b00l: Scanning with association level 0: association without coordination 0b010:Scanning with association level 1: association with coordination. 0b011: Scanning with association level 2: network assisted association. |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011) { |  |  |
| CDMA_code} | TBD |  |
| N_carrier_Index | 8 | Number of carriers to be scanned in the neighbor ABS |
| for(i==0,i<N_ carrier_Index; i++){ |  |  |
| carrier_Index | TBD | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_resource | 8 |  |
| ｝ |  |  |
| ｝ |  |  |
| ｝ |  |  |
| ｝else{ |  |  |
| N_Recommended_BS_Index | 8 |  |
| If(N_Recommended_BS_Index == 0){ |  |  |
| N_Recommended_BS_carrier_Index | 8 |  |
| for(i==0,i<N_Recommended_BS_carrier_Index; i++){ |  |  |
| Recommended_BS_carrier_Index | TBD | carrier physical index |
| } |  |  |
| } else{ |  |  |
| for(i = 0; i< N_Recommended_BS_Index; i++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} | TBD |  |
| N_Recommended_BS_carrier_Index |  |  |
| for(j==0,j<N_Recommended_BS_carrier_Index; j++){ |  |  |
| Recommended_BS_carrier_Index |  | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_opportunity | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| If( Use_Req_Bitmap_Index == 1 ){ |  |  |
| Req_Bitmap_Size | TBD |  |
| Req_Bitmap_Index | TBD |  |
| for( each ‘1’ in Req_Bitmap_Index ){ |  |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} | TBD |  |
| N_carrier_Index | 8 |  |
| for(i==0,i<N_ carrier_Index; i++){ |  |  |
| carrier_Index | TBD | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time | 8 |  |
| Transmission_opportunity | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 |  |
| for(j = 0; j < N_Recommended_BS_Full; j++){ |  |  |
| Recommended BS ID | 48 |  |
| Scanning type | 2 |  |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| CDMA_code} |  |  |
| N_carrier_Index |  |  |
| for(j==0,j<N_ carrier_Index; j++){ |  |  |
| carrier_Index |  | carrier physical index |
| if( (Scanning_type == 0b010) OR (Scanning_type == 0b011){ |  |  |
| Rendezvous_time |  |  |
| Transmission_opportunity |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | variable |  |
| } |  |  |
| TLV encoded information | variable |  |
| } |  |  |

# 15.2.4.x AAI_ SCN-REP

An AMS shall transmit an AAI_SCN-REP message to report the scanning results to its serving ABS if the trigger condition is met.

Table XXX- AAI-SCN-REP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI-SCN-REP_Message_format() { |  |  |
| Management Message Type | 8 |  |
| Report Mode | 1 | 0: Event-triggered report 1: Periodic report |
| Use_Nbr_Bitmap_Index | 1 |  |
| Use_Rsp_Bitmap_Index | 1 |  |
| Report metric | 8 |  |
| if( Use_Nbr_Bitmap_Index == 1 ){ |  |  |
| Nbr_Bitmap_Size | TBD |  |
| Nbr_Bitmap_Index | TBD |  |
| for( each ‘1’ in Nbr_Bitmap_Index ){ |  |  |
| N_scan_carrier | 8 | Number of carriers to be scanned. |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } else { |  |  |
| N_Neighbor_BS_Index | 8 |  |
| for(j = 0; j < N_Neighbor_BS_Index; j++) { |  |  |
| Neighbor_BS_Index | 8 |  |
| N_scan_carrier | 8 | Number of carriers to be scanned. |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| N_ Neighbor_BS_Full | 8 | Number of neighboring BS that are using full 48 bits BS ID. |
| for(j = 0; j < N_Neighbor_BS_Full; j++) { |  |  |
| Neighbor BSID | 48 |  |
| N_scan_carrier | 8 | Number of carriers to be scanned |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| if( Use_Rsp_Bitmap_Index == 1 ){ |  |  |
| Rsp_Bitmap_size |  |  |
| Rsp_Bitmap_Index |  |  |
| for( each ‘1’ in Rsp_Bitmap_Index ){ |  |  |
| N_scan_carrier | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  |  |
| if( Report_metric[Bit 0] == 1) |  |  |
| BS_CINR_mean | 8 |  |
| if( Report_metric[Bit 1] == 1) |  |  |
| BS_RSSI_mean | 8 |  |
| if( Report_metric[Bit 2] == 1) |  |  |
| Relative_delay | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| padding | variable |  |
| TLV encoded information | variable |  |
| } |  |  |

# 15.2.4.x AAI_ ASC-REP

If association level 2 is applied, the AMS does not have to wait for ranging response from the target ABS after sending ranging request or ranging code to the target ABS. Ranging adjust info may be sent by each target ABS to the serving ABS. The serving ABS will generate an AAI-ASC-REP message and send it to the AMS.

Table XXX- AAI-ASC-REP message format

| Syntax | Size （bits） | Notes |
| --- | --- | --- |
| AAI_ASC_REPORT_Message_format() { |  |  |
| Management Message Type | 8 |  |
| N_Recommended_BS_Index | 8 |  |
| for(j = 0; j < N_Recommended_BS_Index; j++){ |  |  |
| Neighbor_BS_Index | 8 |  |
| N_carrier_Index | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  | carrier physical index |
| Timing adjust | TBD |  |
| Power level adjust | TBD |  |
| Offset frequency adjust | TBD |  |
| Ranging status | 8 |  |
| Service level prediction | 8 |  |
| } |  |  |
| } |  |  |
| N_Recommended_BS_Full | 8 | Number of neighboring BS that are using full 48 bits BS ID. |
| for(j = 0; j < N_Recommended_BS_Full; j++){ |  |  |
| Neighbor_BS_ID | 48 |  |
| N_carrier_Index | 8 |  |
| for(j = 0; j < N_scan_carrier; j++) { |  |  |
| carrier_Index |  | carrier physical index |
| Timing adjust | TBD |  |
| Power level adjust | TBD |  |
| Offset frequency adjust | TBD |  |
| Ranging status | 8 |  |
| Service level prediction | 8 |  |
| } |  |  |
| } |  |  |
| } |  |  |

------------------------------- ----------------------------Text End ------------------------------------------------------------

# References

[1] IEEE 802.16-09/0010r1a, “IEEE 802.16m Amendment Working Document”, March 31, 2009.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”