| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on Carrier Management in Multi-Carrier Operation |  |
| Date Submitted | 2009-04-28 |  |
| Source(s) | Yuqin Chen, Nan Li, Hongyun Qu ZTE Corporation | Voice: [Telephone Number (optional)]] E-mail: chen.yuqin@zte.com.cn li.nan25@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD New Contribution / Multi-Carrier |  |
| Abstract | This contribution proposes the text for carrier management in multi-carrier operation to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on Carrier Management in Multi-Carrier Operation

Yuqin Chen，Nan Li, Hongyun Qu

ZTE Corporation

# Introduction

In current SDD document 80216m-008_003r8, it’s defined that the primary carrier can be switched to anther fully configured carrier, and the secondary carrier can be assigned, updated and released based on loading and other factors. To realize the functions mentioned above, we propose the carrier management scheme and messages in this contribution.

# Proposed Carrier Management Scheme

Since loading balance is the most important factor for carrier management, it is suggested that ABS is the one who is in charge of the carrier management. However, the carrier management message could be sent out by ABS unsolicitedly, or as a response to AMS’ request. After receiving the carrier management message from ABS, an ACK is required at AMS. If no ACK is received when the timer expires, ABS retransmits the carrier management message again.

The carrier management scheme includes primary carrier and secondary carrier switching, secondary carrier assignment and release. These procedures are realized by carrier management message. Three messages are proposed in this contribution: Carrier_Management-Request message, Carrier_Management message, and ACK message.

The Carrier_Management-Request message is used by AMS to ask for carrier switching or carrier release. The carrier switching procedure could happen when current carrier at AMS has a bad channel condition, and the carrier release happens under many conditions, like QoS changing or power consumption consideration. The message format proposed is shown in Table 1.

Table 1. Carrier_Management-Request message

| Syntax | Size | Notes |
| --- | --- | --- |
| Carrier_Management-Request message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| MS Station ID | TBD |  |
| Action Code | TBD | 00: Primary Carrier Switching 01: Secondary Carrier Assignment 10: Secondary Carrier Update 11: Secondary Carrier Release |
| If (Action Code = = 00){ |  |  |
| Target Carrier index | TBD | The target primary carrier suggested by AMS to switch to. |
| } |  |  |
| If (Action Code = = 01){ |  |  |
| Carrier index | TBD | The target secondary carrier suggested by AMS to assign. |
| } |  |  |
| If( Action Code = = 10){ |  |  |
| Carrier index | TBD | The carrier suggested by AMS to switch. |
| Target carrier index | TBD | The carrier suggested by AMS to switch to. |
| } |  |  |
| If( Action Code = = 11){ |  |  |
| Carrier index | TBD | The carrier suggested by AMS to release. |
| } |  |  |
| } |  |  |

The Carrier _Management message is used by ABS to indicate AMS to implement the carrier management operation. This message could be sent out in unsolicited manner, or as a response to the Carrier_Management-Request message received from AMS. The message format proposed is shown in Table 2.

Table 2 Carrier_Management message

| Syntax | Size | Notes |
| --- | --- | --- |
| Carrier_Management message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| MS Station ID | TBD |  |
| Action Code | TBD | 00: Primary Carrier Switching 01: Secondary Carrier Assignment 10: Secondary Carrier Update 11: Secondary Carrier Release |
| Action Time | TBD |  |
| If (Action Code = = 00){ |  |  |
| Target Carrier index | TBD | The target primary carrier to switch to. |
| } |  |  |
| If (Action Code = = 01){ |  |  |
| Carrier index | TBD | The target secondary carrier to assign. |
| } |  |  |
| If( Action Code = = 10){ |  |  |
| Carrier index | TBD | The secondary carrier to switch. |
| Target carrier index | TBD | The secondary carrier to switch to. |
| } |  |  |
| If( Action Code = = 11){ |  |  |
| Carrier index | TBD | The carrier to release. |
| } |  |  |
| } |  |  |

After receiving Carrier_Management message, AMS feedbacks an ACK message to acknowledge the reception. If no ACK is received when timer expires, ABS should retransmit the Carrier_Management message. The ACK message format is shown in Table 3.

Table 3 ACK message

| Syntax | Size | Notes |
| --- | --- | --- |
| ACK message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| TLV encoded information |  | TLV specific |
| } |  |  |

# Text proposal in the 802.16m amendment

---------------------------------------------------------- Text Start ----------------------------------------------------------------

# x Support for Multi-Carrier Operation

# x.x Multi-Carrier Management Scheme

The carrier management scheme includes primary carrier and secondary carrier switching, secondary carrier assignment and release. The carrier management is performed by ABS, sent out unsolicitedly or as a response to AMS’ carrier management request. After receiving the Carrier_Management message, AMS is required to feedback an acknowledgement. If no ACK is received when the timer expires, ABS retransmits the carrier management message again.

# 15.2.4.x Carrier_Management-request message

| Syntax | Size | Notes |
| --- | --- | --- |
| Carrier_Management-Request message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| MS Station ID | TBD |  |
| Action Code | TBD | 00: Primary Carrier Switching 01: Secondary Carrier Assignment 10: Secondary Carrier Update 11: Secondary Carrier Release |
| If (Action Code = = 00){ |  |  |
| Target Carrier index | TBD | The target primary carrier suggested by AMS to switch to. |
| } |  |  |
| If (Action Code = = 01){ |  |  |
| Carrier index | TBD | The target secondary carrier suggested by AMS to assign. |
| } |  |  |
| If( Action Code = = 10){ |  |  |
| Carrier index | TBD | The carrier suggested by AMS to switch. |
| Target carrier index | TBD | The carrier suggested by AMS to switch to. |
| } |  |  |
| If( Action Code = = 11){ |  |  |
| Carrier index | TBD | The carrier suggested by AMS to release. |
| } |  |  |
| } |  |  |

# 15.2.4.x Carrier_Management message

| Syntax | Size | Notes |
| --- | --- | --- |
| Carrier_Management message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| MS Station ID | TBD |  |
| Action Code | TBD | 00: Primary Carrier Switching 01: Secondary Carrier Assignment 10: Secondary Carrier Update 11: Secondary Carrier Release |
| Action Time | TBD |  |
| If (Action Code = = 00){ |  |  |
| Target Carrier index | TBD | The target primary carrier to switch to. |
| } |  |  |
| If (Action Code = = 01){ |  |  |
| Carrier index | TBD | The target secondary carrier to assign. |
| } |  |  |
| If( Action Code = = 10){ |  |  |
| Carrier index | TBD | The secondary carrier to switch. |
| Target carrier index | TBD | The secondary carrier to switch to. |
| } |  |  |
| If( Action Code = = 11){ |  |  |
| Carrier index | TBD | The carrier to release. |
| } |  |  |
| } |  |  |

# 15.2.4.x ACK message

| Syntax | Size | Notes |
| --- | --- | --- |
| ACK message { |  |  |
| Message Type | 8 |  |
| Transaction ID | TBD |  |
| TLV encoded information |  | TLV specific |
| } |  |  |

------------------------------- ----------------------------Text End ------------------------------------------------------------

# References

[1] IEEE 802.16-09/0010r1a, “IEEE 802.16m Amendment Working Document”, March 31, 2009.

[2] IEEE 802.16m-07/003r8, “802.16m System Description Document”