| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text on the Ranging Structures for Non-synchronized AMSs |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon, Sungho Moon LG Electronics Xinrong Wang, Yang-Seok Choi Intel Corporation Hwasun Yoo, Si-Hyun Park Samsung Electronics | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com xinrong.wang@intel.com, yang-seok.choi@intel.com hwasun.yoo@samsung.com |
| Re: | IEEE 80216m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” “Comments on AWD 15.3.9 UL-CTRL” |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text on the Ranging Structures for Non-synchronized AMSs

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon and Sungho Moon

LG Electronics

Xinrong Wang and Yang-Seok Choi

Intel Corporation

Hwasun Yoo and Si-Hyun Park

Samsung Electronics

- Introduction

This contribution proposes the text of ranging channel section to be included in IEEE 802.16m AWD [1].

- References

[1] IEEE 802.16m-08/0010r1a, “IEEE 802.16m Amendment Working Document,” March 2009.

- Text proposal for inclusion in the 802.16m amendment

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in [1]

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

[  ------------------------------------ Text Proposal #1 ----------------------------------  ]

------------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

# Ranging Channel Structure for Non-synchronized AMSs

The ranging channel for non-synchronized AMSs is used for initial network entry and association and for ranging against a target BS during handover.

A physical ranging channel for non-synchronized AMSs consists of the ranging preamble (RP) with length of TRP depending on the ranging subcarrier spacing ∆fRP, and the ranging cyclic prefix (RCP) with length of TRCP in the time domain.

A ranging channel occupies a localized bandwidth corresponding to the [1 or 2] subbands.

Power control operation described in subclause [TBD] applies to ranging signal transmission.

illustrates the ranging channel structures in the time domain.

(a) Structure 1

(b) Structure 2

- Structure 3

- Structure 4

Figure 450 - The ranging channel structures in the time domain

Error: Reference source not found contains ranging channel formats and parameters.

Table 691 - Ranging Channel Formats and Parameters

| Format No. | Ranging Channel FormatStructure | TRCP | TRP | ∆fRP |
| --- | --- | --- | --- | --- |
| 0 | Structure 1 | Tg + k×Tb (a) | 2×Tb | ∆f / 2 |
| 1 | Structure 23 |  |  |  |
| 2 | Structure 32 | m×Tg + n×Tb (b) | 2×2×Tb (c) |  |
| …3 | …Structure 3 | 7×Tg + Tb | 8×Tb | ∆f / 8 |
| 4 | Structure 4 | Tg | Tb | ∆f |

where Ts, Tb, Tg and ∆f are defined in Section 15.3.2.4 Derived parameters.

(a): The TRCP for Formats 0 and 1 depends on OFDMA parameters, subframe types as follows:

Nsym is the number of OFDMA symbols in a subframe as defined in Section 15.3.8.1 Physical and logical resource unit. Fs and NFFT are defined in Section 15.3.2.4 Derived parameters.

(b): The TRCP for Formats 2 depends on OFDMA parameters, subframe types as follows:

Nsym is the number of OFDMA symbols in a subframe as defined in Section 15.3.8.1 Physical and logical resource unit.

(c): TRP for Format 2 denotes the total length of repeated ranging preamble.

In the ranging channel Format 0, the repeated RCPs and RPs are used as a single time ranging opportunity within a subframe in  (a). Format 12 consists of a single RCP and repeated RPs within a subframe. Format 21 consists of a single RCP and RP which is a part of the Format 0. When Format 1 is used, there are two time opportunities within a subframe. Format 3 has the same structure with Forma 1 but its length is different. RP1, RP2, RP3, and RP4 in Format 4 are constructed by the first portion, second portion, third portion, and the last portion of the ranging sequences [the detail is TBD depending on the ranging sequences], respectively.

For the ranging opportunity of the non-synchronized AMS, each AMS randomly chooses one of ranging preamble sequences from the available ranging sequence set in a cell defined in TBD Ranging preamble codes.

When the ranging channel format is configured as Format 0, 2, 3, 4, or Format 1 using the first time-opportunity in the time domain, the transmission start time of the ranging channel is aligned with the UL subframe start time at the AMS. For the Format 1 using the second time-opportunity, the transmission of the ranging channel starts at TRCP+TRP in Format 1 after the start time of first time-opportunity.

------------------------------------------------------------------------- Text End ---------------------------------------------------------------------------

[  ------------------------------------ Text Proposal #2 ----------------------------------  ]

------------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

[Insert the following text into the section 15.3.9.2.4.1 Ranging channel for non-synchronized AMSs.]

# Ranging signal transmission

# Ranging signal transmission

<<Error: Reference source not found>>Equation (238) specifies the transmitted signal voltage to the antenna, as a function of time, during ranging channel format 0, 1, 2 or 3.

s(t)=franging()

(238)

where

franging()is a TBD function

t		is the elapsed time since the beginning of the subject ranging channel.

NRP			is the length of ranging preamble code in frequency domain.

xp(n)		is the p-th ranging preamble code with length NRP.

Koffset		is the parameter related to the frequency position and is defined by

NPRU		is the total number of PRUs as defined 15.3.8.2.1 Subband partitioning.

k0		is a logical ranging channel parameter in the frequency domain as units of N1, where N1 is the number of the adjacent PRUs within a subband as defined 15.3.8.2.1 Subband partitioning.

Psc			is the number of the consecutive subcarriers within a PRU in frequency domain as defined 15.3.8.1 Physical and logical resource unit.

△fRP		is the ranging subcarrier spacing.

Toffset		is the parameter related to the length of ranging cyclic prefix and is defined by

The transmitted signal voltage to the antenna in Format 4 is generated by the equation (173).

------------------------------------------------------------------------- Text End ---------------------------------------------------------------------------