| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text on the Ranging Configurations for Non-synchronized AMSs |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon, Sungho Moon LG Electronics Hwasun Yoo, Si-Hyun Park Samsung Electronics | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com hwasun.yoo@samsung.com |
| Re: | IEEE 80216m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” “Comments on AWD 15.3.9 UL-CTRL” |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text on the Ranging Configurations for Non-synchronized AMSs

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon and Sungho Moon

LG Electronics

Hwasun Yoo, Si-Hyun Park

Samsung Electronics

- Introduction

This contribution proposes the text of ranging channel section to be included in IEEE 802.16m amendment [1]. The proposed text is developed in order to be readily combined with IEEE P802.16 Rev2/D9 [2]. It is also compliant to the 802.16m SRD [3] and the 802.16m SDD [4].

- Ranging Channel Configurations

Only one ranging channel in frequency domain is allocated at a time. Then, different subframe period can be used for the ranging channel allocation depending on the ranging loads (e.g. higher system bandwidths). This provides the simple signaling with smaller ranging waiting time.

In the TDD mode, since it is desirable to allocate the ranging channels contiguously in the time domain due to lack of UL subframe, the number of ranging channel can be indicated for the ranging channel allocation depending on the ranging loads.

- References

[1] IEEE 802.16m-08/0010r1a, “IEEE 802.16m Amendment Working Document,” March 2009.

[2] IEEE P802.16Rev2/D9a, “DRAFT Standard for Local and metropolitan area networks / Part 16: Air Interface for Broadband Wireless Access Systems,” March 2009.

[3] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements,” January 2009.

[4] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document,” April 2009.

[5] IEEE 802.16m-09/1093, “Proposed AWD Text on the Ranging Codes for Non-synchronized AMSs,” April 2009.

- Text proposal for inclusion in the 802.16m amendment

------------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in the subclause 15.3.9.2.4.1 Ranging channel for non-synchronized AMSs of [1]

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start ---------------------------------------------------------------------------

[Insert the following text in the section 15.3.9.2.4.1 Ranging channel for non-synchronized AMSs.]

# Ranging channel configurations

# Ranging channel configurations

The information for ranging time/frequency resource allocation is indicated by the S-SFH. The ranging configuration information in the S-SFH consists of the frame-period (PF) and subframe-period (PSF) with subframe-offset (OSF) for ranging resource allocation in the time domain, and the starting subband position (k0) as units of subband for ranging allocation in the frequency domain. Within a superframe, the ranging channels are allocated from the PFth frame in every frame (PF=0) or every odd frame (PF=1). Within the allocated frame, the ranging channels are allocated from the OSF th UL subframe with period of PSF in the FDD mode. In the TDD mode, the ranging channels are allocated in the consecutive PSF subframes from the OSF th UL subframe. Table xxx shows the ranging channel configurations.

Table xxx. Ranging channel configurations

| No. | PF | PSF | OSF |
| --- | --- | --- | --- |
| 0 |  |  |  |
| 1 |  |  |  |
| 2 |  |  |  |
| .... |  |  |  |

The occupied resource for a ranging channel is defined as Nr1 subframes by Nr2 subbands. For Format 0, 1, 2, and 4, Nr1 = 1. For Format 3, N1 = 3. The unit of Nr2 is 1 subband for all Formats. Figure xxx shows an example of ranging channel allocation.

Figure xxx. The example of ranging channel allocation in time domain for FDD and TDD duplex mode, respectively.

There are different types of ranging opportunities in ranging channel formats. First, each AMS randomly selects a ranging subframe assigned in the superframe, which is indicated by S-SFH. When ranging channel Format 1 is used, each AMS randomly selects one of time opportunities within a selected ranging subframe. Finally, each AMS randomly selects a code-opportunity from the available ranging code set in a sector defined in Subclause 15.3.9.2.4.1.1 Ranging preamble codes.

------------------------------------------------------------------------- Text End ---------------------------------------------------------------------------