| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ DG AWD Text Proposal with DG comment resolution |  |
| Date Submitted | 2009-05-05 |  |
| Source(s) | Youngbin Chang 16m ARQ DG | E-mail: yb.chang@samsung.com Phone: +82-31-279-5519 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD-DG Comment / Area: ARQ DG IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. |  |
| Abstract | This contribution proposes ARQ DG text proposal. |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ DG AWD Text Proposal with DG comment resolution

Youngbin Chang

16m ARQ DG

# Contribution and comment lists (ARQ DG activity)

This table shows the March meeting proposals.

| Contribution | REV | TITLE | AUTHORS | AFFILIATION | E-mail |
| --- | --- | --- | --- | --- | --- |
| C80216m-09/0512 |  | ARQ for IEEE 802.16m | Muthaiah Venkatachalam, Xiangying Yang, Aran Bergman | Intel Corporation | muthaiah.venkatachalam@intel.com |
| C80216m-09/0521 |  | MAC PDU Format and ARQ Design | Hua Xu, Xiao Xu, John Harris | Motorola | Hua.xu@motorola.com |
| C802.16m-09/0595 | r1 | Proposed Text of ARQ Operation Section for the IEEE 802.16m Amendment | Ming-Hung Tao Mamadou Kone Ying-Chuan Hsiao | ITRI | MHTao@itri.org.tw |
| C802.16m-09/0624 |  | Proposed Text Related to ARQ for the IEEE 802.16m Amendment | Eunjong Lee, Doo-hyun Sung and Taegon Kong | LGE | mehappy3@lge.com, dh.sung@lge.com, tgkong@lge.com |
| C802.16m-09/0644 | r2 | ARQ mechanism for 16m AWD | Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son, Roberto Albanese, Shashikant Maheshwari, Yousuf Saifullah, Ming-Hung Tao, Mamadou Kone, Ying-Chuan Hsiao, Eunjong Lee, Doo-hyun Sung and Taegon Kong | Samsung,Nokia, NSN, ITRI, LGE | yb.chang@samsung.com, roberto.albanese@nokia.com, shashi.maheshwari@nsn.com, MHTao@itri.org.tw, dh.sung@leg.com |
| C802.16m-09/0646 |  | ARQ in 802.16m | Roberto Albanese, Andrea Bacioccola, Shashikant Meheshwari, Yousuf Saifullah | Nokia, NSN | roberto.albanese@nokia.com, shashi.maheshwari@nsn.com |

This table shows the comment about 1st ARQ DG Draft circulation.

| Contribution | REV | TITLE | AUTHORS | AFFILIATION | E-mail |
| --- | --- | --- | --- | --- | --- |
| C802.16m-ARQ_DG-09/001_Intel |  | 1st ARQ DG AWD Draft | Muthaiah Venkatachalam, Xiangying Yang, Aran Bergman | Intel Corporation | muthaiah.venkatachalam@intel.com |
| C80216m-ARQ_DG-09_001 - Stavros_Tzavidas_Motorola |  | 1st ARQ DG AWD Draft | Starvros Tzavidas, Hua Xu, Xiao Xu, John Harris | Motorola | Hua.xu@motorola.com |
| C80216m-ARQ_DG-09_001-YeongikSeo_POSDATA |  | Comment on the 1st ARQ DG AWD Draft | Yeong Ik Seo Kyounghwan Lee Sang-heon Shin Hyung Joon Jeon Yerang Hur | Posdata | future0000@posdata.co.kr, khlee@posdata-usa.com shshin@posdata.co.kr hjjun@posdata.co.kr yehur@posdata-usa.com |
| C802.16m-ARQ_DG-09/001-Youngbin_samsung |  | ARQ mechanism for 16m AWD | Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son | Samsung | yb.chang@samsung.com, |

This table shows the comment about 2nd ARQ DG Draft circulation.

| Contribution | REV | TITLE | AUTHORS | AFFILIATION | E-mail |
| --- | --- | --- | --- | --- | --- |
| C80216m-ARQ r2 |  | 1st ARQ DG AWD Draft | Muthaiah Venkatachalam, Aran Bergman, Xiangying Yang, Shantidev Mohanty, Phil Barber, Yih-Shen Chen | Intel Corporation Huawei MediaTek | muthaiah.venkatachalam@intel.com |
| IEEE C802.16m-ARQ_DG-09/002- POSDATA |  | On the Size of Selective ACK MAP | Yeong Ik Seo Kyounghwan Lee | Posdata | future0000@posdata.co.kr, khlee@posdata-usa.com |
| C80216m-ARQ_DG-09_002_POSDATA-Samsung |  | Comments on 2nd ARQ DG AWD Draft | Yeong Ik Seo Kyounghwan Lee Sang-heon Shin Hyung Joon Jeon Yerang Hur, Youngbin Chang | POSDATA, Samsung | future0000@posdata.co.kr, khlee@posdata-usa.com, yb.chang@samsung.com |
| IEEE C802.16m-ARQ_DG-09/002-Samsung |  | Comment on 2nd ARQ DG AWD Draft | Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son | Samsung | yb.chang@samsung.com |

This table shows the comment about 3rd ARQ DG Draft circulation.

| Contribution | REV | TITLE | AUTHORS | AFFILIATION | E-mail |
| --- | --- | --- | --- | --- | --- |
| C80216m-09/003r1 |  | 3rd ARQ DG AWD Draft | Muthu Venkatachalam, Xiangying Yang Yihshen Chen Phillip Barber | Intel Corporation, MediaTek, Huawei | muthaiah.venkatachalam@intel.com |
| C80216m-ARQ_DG-09_003-JaesunCha_ETRI |  | Comments on 3rd ARQ DG AWD Draft | Jaesun Cha, Kwangjae Lim, Sungcheol Chang, Chulsik Yoon | ETRI | jscha@etri.re.kr |
| C80216m-ARQ_DG-09_003-samsung |  | Comments on 3rd ARQ DG AWD Draft | Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son | Samsung | yb.chang@samsung.com |

# Text proposal

============================= Start of Proposed Text ========================

15.2.x ARQ mechanism

ARQ may be enabled on a per-connection basis. ARQ parameters shall be specified and negotiated during connection setup.

A connection shall not have a mixture of ARQ and non-ARQ traffic. The scope of a specific instance of ARQ is limited to one unidirectional flow.

15.2.x.2 ARQ feedback

15.2.x.2.1 ARQ feedback IE

Table xxx defines the ARQ Feedback IE for ARQ block. It is used by the receiver to signal positive or negative acknowledgments. A set of IEs of this format may be transported either as a packed payload (“piggybacked”) within a packed MAC PDU or as a payload of a standalone MAC PDU.

Table xxx – ARQ feedback IE format for ARQ block

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | variable |  |
| Flow ID | 4 | The ID of the flow being referenced |
| LAST | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG = 0, ARQ block up to and including SN has been received successfully. FLAG = 1, ARQ block less than SN has been received successfully. |
| While(FLAG == 1) { |  |  |
| Selective ACK MAP | 7 | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous SN. |
| FLAG | 1 | 0 = No more selective ACK MAP 1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| } |  |  |

15.2.x.2.2 ARQ feedback polling request

Transmitter may send ARQ feedback polling request to the receiver, to update the reception status of the transmitted ARQ blocks. The ARQ polling request is sent using a FPEH or MEH. When the transmitter is at the AMS side, the ARQ feedback poll may only be sent to the ABS by piggybacking an FPEH or MEH into an uplink data packet belonging to an existing uplink flow. The AMS shall not request uplink bandwidth solely for the purpose of sending an ARQ feedback poll to the ABS.

In downlink, an ABS may assign unsolicited bandwidth using extended header or MAC management message for the AMS to send the ARQ feedback in the same MPDU. The ABS indicates that the purpose of the unsolicited bandwidth is for sending ARQ feedback.

15.2.x.2.3 ARQ feedback triggering conditions

Receiver sends an ARQ feedback when an ARQ feedback polling request is received from the transmitter or when an ARQ block has been missing for a predetermined period.

15.2.x.3 ARQ parameters

15.2.x.3.1 ARQ_SN_MODULUS

ARQ_SN_MODULUS is equal to the number of unique SN values, i.e., 2^(10).

15.2.x.3.2 ARQ_WINDOW_SIZE

ARQ_WINDOW_SIZE is defined in 6.3.4.3.2.

15.2.x.3.3 ARQ_SUB_BLOCK_SIZE

ARQ_SUB_BLOCK_SIZE is the ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement.

15.2.x.3.4 ARQ_BLOCK_LIFETIME

ARQ_BLOCK_LIFETIME is defined in 6.3.4.3.3.

15.2.x.4.5 ARQ_RX_PURGE_TIMEOUT

ARQ_RX_PURGE_TIMEOUT is defined in 6.3.4.3.6

15.2.x.4.6 MAXIMUM_ ARQ_BUFFER_SIZE

This is a negotiation parameter between ABS and AMS. AMS shall inform maximum ARQ buffer size to ABS. ABS should send ARQ blocks not exceeding MAXIMUM_ ARQ_BUFFER_SIZE.

============================== End of Proposed Text ========================