| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Multi-BS MIMO PHY Amendments – Closed-Loop Macro Diversity and Collaborative Precoding |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Keying Wu, Dong Li, Hongwei Yang, Xiaolong Zhu Alcatel-Lucent Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-09/0020 Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content Category: AWD – New contribution: Interference Mitigation “Text proposal on Interference Mitigation using Advanced Antenna Technologies” |  |
| Abstract | This contribution proposes the text for the downlink multi-BS MIMO in the physical layer. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink Multi-BS MIMO PHY Amendments – Closed-Loop Macro Diversity and Collaborative Precoding

Keying Wu, Dong Li, Hongwei Yang, Xiaolong Zhu

Alcatel-Lucent Shanghai Bell Co., Ltd.

# 1.	Introduction

In the current 16m SDD [1], multi-BS MIMO is accepted as an advanced MIMO technique to mitigate inter-cell interference (ICI) in Section 20.2. The concept of multi-BS MIMO has also been accepted in 3GPP LTE-A [2], known as coordinated multi-point (CoMP) transmission, as an advanced ICI mitigation solution. The discussions about CoMP in LTE-A include various implementation techniques from complicated but high-performance joint processing to simple but less-optimal coordinated scheduling/beamforming, to cope with different requirements of diversified application scenarios. In order to keep competence against LTE-A, different multi-BS MIMO techniques need to be defined in 16m to provide different tradeoffs between complexity and performance.

In 16m SDD [1], several different multi-BS MIMO techniques have been defined for this purpose, including precoding matrix index (PMI) coordination, inter-cell interference nulling, closed-loop macro diversity (CL-MD) and multi-BS collaborative precoding (Co-precoding). Among these schemes, PMI coordination and inter-cell interference nulling [3] have the lowest implementation complexity due to their single-BS transmission property, but also less throughput gain (as shown in [3]). In this contribution, we will show that by allowing user data sharing among multiple coordinating BSs, the throughput gain can be significantly increased (see system-level simulation results below) through CL-MD and Co-precoding due to additional macro diversity gain. CL-MD uses single-user transmission and Co-precoding uses multi-user transmission, so it provides multiuser diversity gain as well.

In this contribution, we will give a more detailed discussion about CL-MD and Co-precoding, including their basic principles, benefit and cost analysis, impacts on the specification and system-level simulation results to demonstrate their advantages. Finally, corresponding amendment text is proposed for CL-MD and Co-precoding.

# 2.	Multi-BS MIMO

# Closed-Loop Macro Diversity

In closed-loop macro diversity (CL-MD), multiple BSs perform MIMO transmission to one MS. Each BS performs single-BS single user (SU) adaptive precoding independently. Multiple BSs transmit the same information to the MS using RF combining. The data traffic intended to the MS shall be replicated in all BSs involved in coordination. Fig. 1 shows one example of CL-MD. Compared with 16e open-loop MDHO, CL-MD is often more effective to achieve RF combining gain at MS because CL-MD is much more constructive, but not disruptive, via precoding.

Figure 1. Illustration of the CL-MD technique.

Both sounding based precoding and codebook based precoding can be used. When sounding based precoding is used in TDD, multiple BSs involved in coordination estimate the CSI of the intended MS via its sounding signals, and calculate their precoding matrices independently. When codebook based precoding is used in TDD or FDD, the MS selects one PMI for each coordinating BS, and reports these to the serving BS, which in turn forwards the PMI to the relevant BS via the network interface.

In the following, we try to analyze the cost and gain of CL-MD using Fig. 1 as an example. The signals received at MS-1 can be represented as

y1=H1,1w1s+H1,2w2s+n1								(1)

The SINR of MS-1 under CL-MD can be expressed as

SINR1(CL−MD)=||H1,1w1||2p+||H1,2w2||2pσ2						(2)

If single-BS MIMO is used, each BS servers its own MS using SU adaptive precoding. In this case, the transmission from BS-2 becomes interference to MS-1, and its SINR is changed to

SINR1(single)=||H1,1w1||2p||H1,2w2||2p+σ2									(3)

Comparing (2) and (3), it is clear that CL-MD will always bring a SINR gain to MS-1, which comes from the elimination of one interference source and the increase of signal power due to signal transmission from the neighboring BS. However, this gain comes at the price of twice resource usage. Taking this cost into consideration, we can say that the CL-MD technique brings benefit to the aggregate throughput only when the normalized throughput of MS-1 (normalized by the used resource) is larger in CL-MD than in single-BS situation, i.e.,

log2(1+SINR1(CL−MD))>2×log2(1+SINR1(single)).						(4)

This suggests that CL-MD is most suitable for cell-edge MSs, since cell-edge MSs can benefit the most from the signal transmission from a neighboring BS as well as the elimination of one major interference source. For a cell-center MS, as its distance from the neighboring BS is larger, the signal strength from the neighboring BS will be significantly lower than that from the serving BS, and brings neglectable contribution. In addition, since the SINR of a cell-center MS is not interference-limited, eliminating one interference source also brings a much smaller gain. As a result, the gain from CL-MD transmission may not worth the doubled resource usage for cell-center MSs.

# Collaborative precoding

In collaborative precoding (Co-precoding), multiple BSs perform MU-MIMO transmission to multiple MSs located in different cells. Each BS performs single-BS MU adaptive precoding towards multiple MSs (i.e, SDMA) independently, with one stream per MS. Multiple BSs transmit the same information to each MS using RF combining. The data traffic intended to one MS shall be replicated in all BSs involved in coordination. Fig. 2 shows an example of Co-precoding.

Figure 2. Illustration of the Co-precoding technique.

Both sounding based and codebook based MU precoding can be used for Co-precoding. When sounding based precoding is used in TDD, multiple BSs involved in coordination estimate the CSI of multiple MSs via their sounding signals, and calculate multiuser precoding matrices independently. When codebook based precoding is used in TDD or FDD, a MS selects one PMI for each BS involved in coordination, and reports these to the serving BS, which then forwards the PMIs to relevant BSs via network interface. The BS then derives the MU precoding matrices according to the feedback.

Comparing Fig. 2 and Fig. 1, Co-precoding can be regarded as a generalization of CL-MD to support multiple MSs over one resource unit. Therefore, it is reasonable to expect that Co-precoding is also most suitable for cell-edge MSs that can benefit most from the signal transmission from neighboring BSs, and provides additional multiuser gain over CL-MD.

# Impacts on the specification

Impact on resource permutation

CL-MD and Co-precoding require the BSs involved in coordination to synchronize their resource permutation and allocation to the intended MS(s).

Impact on demodulation pilot

With CL-MD or Co-precoding, multiple BSs transmit the same information to each MS using RF combining. The MS needs to estimate the equivalent RF combined channel. For this purpose, different BSs should use the same pilot pattern instead of interlaced patterns, so that their pilot signals will be RF combined in space.

Impact on UL sounding

With Co-precoding, a BS needs to receive the sounding signals from multiple MSs involved in coordination and estimate their CSI. To avoid the interference among the sounding transmissions from these MSs, orthogonal sounding channels should be assigned to these MSs, which needs to be negotiated between the serving BSs of these MSs, and signaled to them.

Impact on feedback information

When codebook based feedback is used for multi-BS MIMO, a MS needs to feedback one PMI for each coordinating BS.

# Simulation Results

From the above discussion, we can see that the multi-BS MIMO techniques are most suitable for cell-edge MSs. In our simulation, cell-edge MSs are classified by their long-term SINR. If one MS has a long-term SINR lower than a pre-defined threshold T, it will be classified as a cell-edge MS; otherwise it will be classified as a cell-center MS. The Multi-BS MIMO operation will only be applied to cell-edge MSs.

Based on simulation assumptions in Table 1, we provide some system-level simulation results for CL-MD and Co-precoding in Fig. 3, and summarize their gains over single-BS MIMO in Table 2. In Fig. 4, we show that both systems can fulfill the proportionally fairness requirement. From Fig. 3 and Table 2, we can see that CL-MD can achieve significant throughput gain at cell edge over single-BS MIMO, and Co-precoding can achieve throughput gain in terms of both average and cell edge throughput. Compare the results in Table 2 with those in [3], the advantage of joint antenna processing over single cell Processing with multi-BS coordination is obvious.

Table 1. Simulation condition

| User traffic model | Full buffer | Site layout | 7 cells, 3 sector per cell |
| --- | --- | --- | --- |
| BS-to-BS distance | 500m | User dropping | Uniformly dropped, 10 users per sector |
| Frequency reuse | 1 | Carrier frequency | 2 GHz |
| Total bandwidth | 10 MHz | BS Tx power | 46 dBm |
| Antenna gain | 17 dBi for BS, 0 dBi for MS | MS cable loss | 0.5 dB |
| PSD of noise | -174.3 dBm/Hz | MS Rx noise figure | 7 dB |
| Antenna configuration | Linear uniform array, BS spacing 4l | Antenna pattern | 3 sector antenna defined in 3GPP |
| Antenna number | 2/4 Tx for BS, 2 Rx for MS | Receiver type | MMSE for SM |
| Lognormal shadowing | Mean = 0, standard deviation = 8 dB | Distance-dependent pathloss | L=128.1+37.6log10(R), R in kilometers |
| Penetration loss | 20 dB | Shadowing correlation | 0.5 between cells, 1.0 between sectors |
| Channel model for signal | SCM, urban micro, 3 km/h | Channel model for interference | Time-variable multipath SCM for 8 strongest interfering sectors, time- variable single-path spatial-white channel from other 12 weakest interfering sectors |
| Channel acquisition | Perfect channel estimation | Modulation and coding scheme (MCS) | QPSK 1/2 with repetition 1/2/4/6, QPSK 3/4, 16QAM 1/2, 16QAM 3/4, 64QAM 2/3, 64QAM 3/4, 64QAM 5/6, all with CTC |
| Band selection for AMC | 1 CQI feedback every 2 continuous logical band (LB) | HARQ mechanism | Synchronous HARQ with chase combining Max num of retransmission = 3 HARQ retransmission delay = 3 frames |
| PHY abstraction | ESM | Feedback type | UL sounding |
| Multi-user scheduling | Proportional fair (PF) scheduler with windows width 0.75s, PF exponent index 0.75, 1-frame scheduling delay | SINR threshold T | 0dB |

(a)													(b)

Figure 3. User spectrum efficiency comparison between single-BS MIMO and multi-BS MIMO of (a) CL-DM and (b) Co-precoding.

Table 2. Gain of multi-BS MIMO over single-BS MIMO

| Multi-BS MIMO mode | CL-MD |  | Co-precoding |  |
| --- | --- | --- | --- | --- |
| Gain over single-BS MIMO | Sector throughput | 0% | Sector throughput | 27% |
|  | 5% throughput | 31% | 5% throughput | 60% |

(a)												  (b)

Figure 4. Distributions of normalized user throughput for (a) CL-MD and (b) Co-precoding.

# 3.	Proposed text for amendment working document

------------------------------------------- Start of Proposed Amendment Text --------------------------------------------

15.3.5.4.1 Pilot patterns

…

The interlaced pilot patterns are generated by cyclic shifting the base pilot patterns. The interlaced pilot patterns are used by different BSs for one and two streams. Interlaced pilot patterns for one stream is shown in Figure 414 and interlaced pilot patterns on stream 0 and stream 1 for two streams are shown in Figure 415 and Figure 416, respectively. Each BS chooses one of the three pilot pattern sets (pilot pattern set 0, 1, and 2) as shown in Figure 414 and Figure 415. The index of the pilot pattern set used by a particular BS with Cell_ID = k is denoted by pk. The index of the pilot pattern set is determined by the Cell_ID according to the following equation:

pk=mod(k,3)                                                                  (189)

In the multi-BS MIMO modes of CL-MD and Co-precoding, in order to enable the MS(s) to detect the RF-combined signal, the involved multiple BSs shall allocate the same contiguous LRUs to the MS(s). In this case, the pilot patterns aren’t interlaced among the involved multiple BSs and all involved multiple BSs shall apply the pilot patterns associated with the serving cell of the target MS.

…

          - MIMO feedback information

Table 670 – MIMO feedback information

|  | Feedback information type | Description |
| --- | --- | --- |
| Long period feedback | Rank information | For MIMO modes 1 and 2 |
|  | Subband selection |  |
|  | Stream index (TBD) | For MIMO mode 3, indicating which streams are preferred. |
|  | Quantized Correlation matrix | For adaptive codebook feedback mode or long term beamforming [TBD] |
|  | Time Correlation coefficient information [TBD] | For differential codebook feedback mode |
|  | PMI report for serving cell [TBD] | For long-term wideband beamforming |
|  | PMI report for neighboring cell | For PMI coordination among multiple BSs |
|  | CQI |  |
| Short period feedback | CQI |  |
|  | PMI report for serving cell | For short-term beamforming with MIMO modes 2 and 4 |
|  | PMI report for neighboring cell | For codebook based CL-MD or Co-precoding |
| Event-driven feedback | Preferred MFM (MIMO feedback mode), etc. |  |

- 15.3.7.2.6.5. Downlink signaling support of DL-MIMO modes

Table 672 – DL MIMO control parameters

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - Broadcast Information |  |  |  |  |
| - Nt | - Number of transmit antennas at the BS | - 0b00: 2 - 0b01: 4 - 0b10: 8 | - SFH (system information) | - Nt must be known before decoding the DL A-MAP IE |
| - OL_Region[TBD] | - OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | - TBD | - Broadcast information |  |
| - SU_CT | - SU base codebook type |  | - Broadcast information | - SU base codebook subset indication |
| - MU_CT (TBD) | - MU base codebook type |  | - Broadcast Information | - MU base codebook subset indication |
| - BC_SI | - Rank-1 base codebook subset indication | - BitMAP - (Same size as rate-1 codebook for each number of transmit antenna) | - Broadcast information | - Rate-1 codebook element restriction/recommendation information - It shall be ignored if CCE = 0b0 |
| - MaxMt (TBD) | - Maximum number of streams | - 0b00: 2 - 0b01: 3 - 0b10: 4 - 0b11: reserved | - Broadcast information | - If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| - Unicast Information |  |  |  |  |
| - MEF | - MIMO encoder format | - 0b00: SFBC - 0b01: Vertical encoding - 0b10: Horizontal encoding - 0b11: n/a | - A-MAP IE (unicast) | - MIMO encoder format. - [MEF bitfield may not be explicitly indicated in DL A-MAP IE] |
| - Mt | - Number of streams in transmission | - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 - (Mt <= Nt ) | - A-MAP IE (unicast) | - Number of streams in the transmission. - When MEF=0b00: Mt =2 MEF=0b10, Mt <= 4 - [Bit-field length is variable, depending on the number of Tx at BS] |
| - RU allocation (TBD) | - RU [and stream] indicator for the burst of data | - TBD | - A-MAP IE (unicast) | - Refer to DL control group. |
| - SI (TBD) | - Index of pilot stream allocation | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 | - A-MAP IE (unicast) | - SI shall be indicated if MEF = 0b010 - [Bit-field length is variable, depending on the number of Tx at BS] - RU allocation and SI can be merged together depending on other DG’s decision |
| - Feedback Allocation IE |  |  |  |  |
| - MFM | - MIMO feedback mode | - Refer to Table 671 | - Feedback allocation IE (unicast) | - To decide the feedback content and related MS processing |
| - DLRU (TBD) | - Downlink RU, indicating - which RUs or which type of RU (DRU or miniband-based CRU) to work on for feedback | - TBD (Tree structure, bit map etc) | - Feedback allocation IE (unicast) | - To process CQI (PMI) estimation for the indicated RUs - Refer to other DG |
| - FT | - MIMO feedback type | - 0b00: codebook - 0b01: sounding | - Feedback allocation IE (unicast) |  |
| - CM | - Codebook feedback mode | - 0b00: standard - 0b01: adaptive - 0b10: differential | - Feedback allocation IE (unicast) | - Enabled when FT = 0b00 |
| - CCE | - Codebook Coordination Enable | - 0b0: disable - 0b1: enable |  | - CCE = 0b0: MS finds PMI within whole broadcasted codebook type entry - CCE = 0b1: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |
| - MBE | - Multi-BS MIMO enable | - 0b0: disable - 0b1: enable | - Feedback allocation IE (unicast) | - If MBE=0b1 and MFM indicates a SU feedback mode, CL-MD is used. - If MBM=0b1 and MFM indicates a MU feedback mode, Co-precoding is used. |

          - Quantized MIMO feedback for closed-loop transmit precoding

            - Quantized feedback modes

An MS may feedback a Preferred Matrix Index (PMI) to support DL precoding. During multi-BS MIMO operation, a MS can be instructed to feedback PMIs for one or more neighboring BSs as well as its serving BS.

There are three types of codebook feedback modes.

          - Unquantized MIMO feedback for closed-loop transmit precoding

            - UL Sounding

To assist the BS in determining the precoding matrix to use for SU-MIMO or MU-MIMO, the BS may request the MS transmit a sounding signal in an UL sounding channel. The BS may translate the measured UL channel response to an estimated DL channel response. The transmitter and receiver hardware of BS and MS shall be calibrated.

The UL sounding channel defined in subclause [TBD] is used in MIMO transmission. To support sounding based multi-BS MIMO, two or more neighboring BSs can coordinate their sounding channel allocation to ensure the orthogonality between sounding signals transmitted in neighboring cells.

-------------------------------------------- End of Proposed Amendment Text -------------------------------------------

# References

[1]	IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”.

[2]	TR 36.814v1.0.0, “Further Advancement of E-UTRA, Physical layer aspects”, 3GPP Technical Report

[3]		IEEE 802.16m-09/1102, “Downlink Multi-BS MIMO PHY amendments – interference nulling”