| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Downlink Multi-BS MIMO PHY Amendments – Interference Nulling |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Keying Wu, Dong Li, Hongwei Yang, Xiaolong Zhu Alcatel-Lucent Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | IEEE 802.16m-09/0020 Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content Category: AWD – New contribution: Interference Mitigation “Text proposal on Interference Mitigation using Advanced Antenna Technologies” |  |
| Abstract | This contribution proposes the text for the downlink multi-BS MIMO. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Downlink Multi-BS MIMO PHY Amendments – Interference Nulling

Keying Wu, Dong Li, Hongwei Yang, Xiaolong Zhu

Alcatel-Lucent Shanghai Bell Co., Ltd.

# 1.	Introduction

In the current 16m SDD [1], multi-BS MIMO is accepted as an advanced MIMO technique to mitigate inter-cell interference (ICI) in Section 20.2. In 3GPP LTE-A, the concept of multi-BS MIMO (referred to as coordinated multi-point transmission) has already be accepted, and two different approaches, namely, coordinated scheduling/beamforming and joint processing/transmission, have been identified to suit different application scenario and performance/complexity requirements. To keep the competitiveness, 16m also needs to support different multi-BS MIMO techniques to provide different tradeoffs between complexity and performance and cope with different system requirements. In 16m SDD, several multi-BS MIMO techniques have been defined, including precoding matrix index (PMI) coordination, inter-cell interference nulling, closed-loop macro diversity (CL-MD) and multi-BS collaborative precoding (Co-precoding). Among these techniques, inter-cell interference nulling has the lowest implementation complexity and least impact on the current network. It can achieve a considerable throughput gain for cell edge users with negligible additional complexity.

PMI coordination and inter-cell interference nulling have the similar concept of mitigating interference to a neighboring cell during precoding. The difference is that the former is based on PMI feedback and the latter based on uplink sounding. Compared with PMI coordination, interference nulling doesn’t need to exchange channel state information between BSs, and thus is potentially more robust to delay of backhaul network transmission or MAC messaging.

In this contribution, we will give a more detailed discussion about inter-cell interference nulling, including its basic principle, benefit and cost analysis, impact on the specification, and system-level simulation results to demonstrate its advantage. Finally, corresponding amendment text is proposed for inter-cell interference nulling.

# 2.	Inter-Cell Interference Nulling

# Basic principle

Inter-cell interference nulling through multi-BS coordination aided precoding exploits any additional spatial degrees of freedom at a single BS to reduce the interference to a neighboring cell. Fig. 1 shows an example of inter-cell interference nulling, in which each MS receives data only from its serving BS, while the neighboring BS adjusts its signal waveform to reduce the interference to this MS.

In order to null/restrict the interference to a neighboring MS, a BS needs to know the channel state information (CSI) between itself and the neighboring MS. In TDD, this can be done through UL sounding based feedback: A BS estimates the CSI of the neighboring MS via its sounding signals, and avoids the interference to this MS via proper precoding matrix selection for its own MS.

Figure 1. Illustration of the interference nulling technique

From the above discussion, we can see that inter-cell interference nulling shares many similarities with PMI coordination. They both perform single-cell adaptive precoding while avoiding interference to a MS served by a neighboring BS via proper precoding matrix selection. The major difference between inter-cell interference nulling and PMI coordination is that the former uses sounding based precoding while the latter uses the codebook based precoding.

# Analysis

In the following, we try to analyze the cost and gain of the inter-cell interference nulling technique using Fig. 1 as an example. Denote by Hi,j the fading matrix between BS-j and MS-i. The signals received at MS-i can be represented as

yi=Hi,iwisi+Hi,jwjsj+ni,          i≠j								(1)

where si is the message transmitted at BS-i, wi the precoding matrix used at BS-i, and ni the additive white Gaussian noise (AWGN) at MS-i. The signal-to-interference-plus-noise-ratio (SINR) of MS-i can be expressed as

SINRi=||Hi,iwi||2p||Hi,jwj||2p+σ2,              i≠j								(2)

where p is the transmission power, and 2 the noise power.

When inter-cell interference nulling is used, each precoding matrix wj is selected such that the interference power to the neighboring MS ||Hi,jwj||2 is nulled/controlled, leading to an interference reduction gain for the neighboring MS. At a cost, the freedom that can be used to select wj is reduced, which consequently leads to a loss in the signal power ||Hj,jwj||2 to the serving MS. Consequently, interference nulling is beneficial to the aggregate throughput only when the throughput gain due to interference reduction gain is larger than the throughput loss due to signal power loss. This suggests that the inter-cell interference nulling technique is most suitable for cell-edge MSs. Since the SINR of a cell-edge MS is mainly limited by the inter-cell interference, eliminating/decreasing one major interference source will bring a considerable gain. For a cell-center MS, however, the gain from eliminating/decreasing one major interference source is much lower, which may not worth the resultant cost of signal power loss at the interfering BS.

# Impacts on the specification

The inter-cell interference nulling technique will have the following impacts on the 16m specification.

The impact on resource permutation

To achieve better performance, the inter-cell interference nulling technique requires the BSs involved in coordination to synchronize their resource permutation and allocation. For example, the two BSs in Fig. 1 is better to schedule the exactly same physical resource to the two MSs, so that their interference to each other can be nulled/controlled over the whole resource allocation.

The impact on UL sounding

When performing interference nulling, a BS needs to estimate the CSI between itself and its serving MS, as well as the CSI between itself and the neighboring MS. For this purpose, the BS must receive the sounding signals from both MSs. To avoid the inter-MS interference in the sounding transmission, orthogonal sounding channels should be used for the two MSs, which need to be negotiated between the two serving BSs of the two MSs, and signaled to the two MSs.

# Simulation results

From the above discussion, we can see that the interference nulling technique is most suitable for cell-edge MSs. In our simulation, cell-edge MSs are classified by their long-term SINR. If one MS has a lower long-term SINR, it will be classified as a cell-edge MS; otherwise it will be classified as a cell-center MS. The interference nulling operation will only be applied to cell-edge MSs.

Based on simulation assumptions in Table 1, we compare system-level simulation results between single-BS MIMO and inter-cell interference nulling in Table 2. As a conclusion, we can see 29% cell-edge performance improvement with no sector average performance loss.

Table 1. Simulation condition

| User traffic model | Full buffer | Site layout | 7 cells, 3 sector per cell |
| --- | --- | --- | --- |
| BS-to-BS distance | 500m | User dropping | Uniformly dropped, 10 users per sector |
| Frequency reuse | 1 | Carrier frequency | 2 GHz |
| Total bandwidth | 10 MHz | BS Tx power | 46 dBm |
| Antenna gain | 17 dBi for BS, 0 dBi for MS | MS cable loss | 0.5 dB |
| PSD of noise | -174.3 dBm/Hz | MS Rx noise figure | 7 dB |
| Antenna configuration | Linear uniform array, BS spacing 4l | Antenna pattern | 3 sector antenna defined in 3GPP |
| Antenna number | 2/4 Tx for BS, 2 Rx for MS | Receiver type | MMSE for SM |
| Lognormal shadowing | Mean = 0, standard deviation = 8 dB | Distance-dependent pathloss | L=128.1+37.6log10(R), R in kilometers |
| Penetration loss | 20 dB | Shadowing correlation | 0.5 between cells, 1.0 between sectors |
| Channel model for signal | SCM, urban micro, 3 km/h | Channel model for interference | Time-variable multipath SCM for 8 strongest interfering sectors, time- variable single-path spatial-white channel from other 12 weakest interfering sectors |
| Channel acquisition | Perfect channel estimation | Modulation and coding scheme (MCS) | QPSK 1/2 with repetition 1/2/4/6, QPSK 3/4, 16QAM 1/2, 16QAM 3/4, 64QAM 2/3, 64QAM 3/4, 64QAM 5/6, all with CTC |
| Band selection for AMC | 1 CQI feedback every 2 continuous logical band (LB) | HARQ mechanism | Synchronous HARQ with chase combining Max num of retransmission = 3 HARQ retransmission delay = 3 frames |
| PHY abstraction | ESM | Feedback type | UL sounding |
| Multi-user scheduling | Proportional fair (PF) scheduler, 1-frame scheduling delay |  |  |

Table 2. Throughput comparison between single-BS MIMO and multi-BS MIMO

| MIMO mode | Sector spectral Efficiency (bps/Hz/sector) | Cell-edge user spectral efficiency (bps/Hz/sector) |
| --- | --- | --- |
| Single-BS SU OL SM with 2 Tx per BS | 1.38 | 0.032 |
| Single-BS SU CL SM with 4 Tx per BS | 1.98 | 0.054 |
| Interference nulling with 4 Tx per BS | 1.98 | 0.069 |

Table 3. Gain of multi-BS MIMO over single-BS MIMO

| Multi-BS MIMO mode | Interference nulling |  |
| --- | --- | --- |
| Gain over single-BS OL SU SM with 2Tx | Sector throughput | 43% |
|  | 5% throughput | 114% |
| Gain over single-BS CL SU SM with 4Tx | Sector throughput | 0 |
|  | 5% throughput | 29% |

# 3.	Proposed text for amendment working document

------------------------------------------- Start of Proposed Amendment Text --------------------------------------------

          - Unquantized MIMO feedback for closed-loop transmit precoding

            - UL Sounding

To assist the BS in determining the precoding matrix to use for SU-MIMO or MU-MIMO, the BS may request the MS transmit a sounding signal in an UL sounding channel in the UL sounding zone. The BS may translate the measured UL channel response to an estimated DL channel response. The transmitter and receiver hardware of BS and MS shall be calibrated.

The UL sounding channel defined in [refer to UL-Control part] is used in MIMO transmission. To support sounding based multi-BS MIMO, two or more neighboring BSs can coordinate their sounding channel allocation to ensure the orthogonality between sounding signals transmitted in neighboring cells.

-------------------------------------------- End of Proposed Amendment Text -------------------------------------------

# References

[1]	IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”.