| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text about 16m MAC PDU Formats |  |
| Date Submitted | 2009-04-28 |  |
| Source(s) | Hongyun Qu Mary Chion ZTE Corporation | E-mail: qu.hongyun@zte.com.cn mchion@zteusa.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD-new contribution/ Area: MAC PDU Formats IEEE 80216m-09_0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. |  |
| Abstract | This contribution proposes the amendment text for 16m MAC PDU formats based on SDD. |  |
| Purpose | To be discussed and accepted as amendment text by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text about 16m MAC PDU Formats

Hongyun Qu, Mary Chion

ZTE Corporation

# Introduction

This contribution proposes amendment text for 16m MAC PDU formats based on the description in the section 10.12 MAC PDU Formats of 16m SDD document IEEE 802.16m-08/003r8.

The proposed TOC for 16m MAC PDU formats is described as follows.

15 Advanced Air Interface

15.2 Medium Access Control

15.2.x MAC PDU formats

15.2.x.1 MAC Header formats

15.2.x.1.1 Generic MAC Header

15.2.x.2 MAC Extended Header formats

15.2.x.2.1 Fragmentation and Packing Extended Header for Transport Connection

15.2.x.2.2 Fragmentation Extended Header for Management Connection

15.2.x.2.3 Multiplexing Extended Header

# Proposed Amendment Text

15 Advanced Air Interface

15.2 Medium Access Control

15.2.x MAC PDU formats

Each MAC PDU shall begin with a MAC Header. The MAC PDU shall contain the payload. The payload shall contain at lease one extended headers, and/or at least one or more MAC SDUs, and/or fragments thereof. MAC PDU shall be of the form illustrated on Figure xxx-1.

Figure xxx-1 MAC PDU format

A MAC PDU may contain CRC. The inclusion of CRC for one connection shall be negotiated during connection setup. All the reserved fields shall be set to zero on transmission and ignored on reception.

15.2.x.1 MAC Header formats

MAC Header formats are defined for both DL and UL.

15.2.x.1.1 Generic MAC Header

Figure xxx-2 Generic MAC Header format

Generic MAC Header begins each DL and UL MAC PDU containing either MAC management messages or CS data.

- Flow ID (Flow Identifier): This field indicates the service flow that is addressed. This field is 4 bits long.

- EH (Extended Header Presence Indication): When set to ‘1’, this field indicates that an Extended Header is present following this GMH.

- Length: This field indicates the length in bytes of the payload in bytes. This field is 11 bits long.

15.2.x.2 MAC Extended Header formats

The inclusion of Extended Header is indicated by EH indicator bit in MAC Header. The Extended Header format is shown in the following figure and will be used unless specified otherwise.

Figure xxx-3 Extended Header format

- Last: When set to “1”, this extended header is the last one. If this bit is not set, another extended header will follow the current extended header.

- Type: This field indicates the type of extended header. The length is 5bits.

- Body Contents: Type-dependent contents.

15.2.x.2.1 Fragmentation and Packing Extended Header for Transport Connection

This Fragmentation and Packing Extended Header shall be used when MAC PDU contains transport connection payload. The location of this extended header exists after the last extended header (i.e. the extended header with ‘Last’ = ‘1’) if ‘EH’ in GMH set to ‘1’ or after the GMH if ‘EH’ in GMH set to ‘0’. This Fragmentation and Packing Extended Header is shown in the following figure.

Figure xxx-4 Fragmentation and Packing Extended Header Format for Transport Connection

- SN (Sequence Number): This SN field indicates payload sequence number. The length is 10 bits.

- FC (Fragmentation Control): The definition of fragmentation control field is given in Table yyy. The length is 2 bits.

- End: If this bit set to ‘0’, another ‘Length’ and ‘End’ fields are followed. If this bit set to ‘1’, reserved bits may follow for byte alignment.

- Length: This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in the extend header. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. The length of this field is 11 bits.

- Rvd: Reserved bits for byte alignment.

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only First fragment of an SDU b) MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | a) MPDU with only Last fragment of an SDU b) MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 1 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | a) MPDU with only middle fragment of an SDU b) MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

Table yyy – Fragmentation Control information

15.2.x.2.2 Fragmentation Extended Header for Management Connection

This Fragmentation Extended Header for Management Connection shall be used when MAC PDU contains management message payload.

Figure xxx-5 Fragmentation Extended Header for Management Connection

- Last: This bit is always set to ‘1’.

- Type: This field indicates the type of extended header. The length is 4 bits.

- SN (Sequence Number): This field indicates the sequence number of the payload. The length is 8 bits.

- FC (Fragmentation Control): The definition of fragmentation contol bits is given in Table yyy. The length is 2 bits.

- Rvd: Reserved bits for byte alignment.

15.2.x.2.3 Multiplexing Extended Header